3GPP TSG-SA WG2 Meeting #127 	S2-183270

Sanya, China, April 16th – April 20nd			       					      revision of S2-18xxxx

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0268 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Propose to add the network Requested PDU Session Release procedure via Untrusted non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-04-07 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification TS23.502 doesn’t provide the network requested PDU session Release procedure via untrusted non-3GPP access. This signalling procedure needs to be added because the Network is allowed to initiate the release of a PDU session over non-3GPP access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the signalling procedure of the network requested PDU session Release via untrusted non-3GPP access. The core network handling refers the network requested PDU session release for Non-Roaming and Roaming with Local Breakout in clause 4.3.4.2. The N3IWF handling refers the UE requested PDU SessionRelease via untrusted non-3GPP access in clause 4.12.7. The new clause describes the difference in respect the existing UE or Network PDU session release procedure for 3GPP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** Start of Change *****

4.12.x Network Requested PDU Session Release via Untrusted non-3GPP access

The PDU session Release Procedure via untrusted non-3GPP access allows the SMF or PCF to initiate the release of a PDU Session.

The Procedure is the same as the network requested PDU Session Release Procedure specified in clause 4.3.4.2 (for Non-Roaming and Roaming with Local Breakout) with the following differences:

-	The (R)AN corresponds to an N3IWF.

- 	In step 5 the N3IWF upon receiving N2 SM request to release the AN resources associated with the PDU Session from the AMF, the N3IWF triggers the release of the corresponding Child SA to the UE as specified in step 5 and 6, in Figure 4.12.7 clause 4.12.7.

-	User Location Information is not included in the step 6, 7a, 9, 10a and 12 of the procedure.

***** End of Changes *****