SA WG2 Meeting #127	S2-183579

Sanya, China, 16 - 20 April	(revision of S2-18xxxx)

Source:	LG Electronics, Motorola Mobility, Lenovo

Title:	ATSSS Solution – Update of Solution 2

Document for:	Approval

Agenda Item:	6.8 Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes to update solution 2 so that single PDU Session ID is used for a multi-access PDU Session.

Proposal

This contribution updates solution 2 so that a single PDU Session ID (instead of two) is used for a multi-access PDU Session.

* * * * Start of 1st Change * * * *

6.2	Solution 2: Support of Multi-Access PDU Sessions

6.2.1	General

A Multi-Access PDU (MA-PDU) session is created by bundling together two separate PDU sessions, which are established over different accesses.

An MA-PDU session is schematically illustrated in the figure below. It is composed of two PDU sessions, referred to as "child PDU sessions"; one established over 3GPP access and the other established over untrusted non-3GPP access (e.g. a WLAN AN).

Figure 6.2.1-1: Illustration of a Multi-Access PDU session with two child PDU sessions

The child PDU sessions of a MA-PDU session share the following characteristics:

(i)	a common DNN;

(ii)	a common UPF anchor (UPF-A);

(iii) a common PDU type (e.g. IPv6);

(iv) common IP address(es);

(v)	a common SSC mode;

(vi) a common S-NSSAI; and

(vii) a common PDU Session ID.

An MA-PDU session realizes a multi-path data link between the UE and an UPF-A, as shown in the figure 6.2.1-1. It operates below the IP layer and it is transparent to the IP layer and all layers above the IP layer.

As discussed in the next clause, a MA-PDU session can be established with one of the following procedures:

(i)	Established with two separate PDU session establishment procedures; one of each child PDU session. This is called "separate establishment".

(ii)	Established with a single MA-PDU session establishment procedure, where the two child PDU sessions are established in parallel. This is called "combined establishment".

The child PDU sessions established with any of the above procedures have the same IP address.

After a MA-PDU session is established, SM signalling related to this MA-PDU session (e.g. for modification or removal of a child PDU session, etc.) can be conducted over any of the available accesses of the MA-PDU session.

The UE discovers the support of ATSSS for a PDU session upon attempting to establish the MA-PDU session as described in clause 6.2.2.

6.2.2	Establishment of Multi-Access PDU session

6.2.2.1	Separate Establishment

In this case, the two child PDU sessions are established with two separate PDU session establishment procedures. For instance, the UE may establish first the PDU session over 3GPP access and, later, after it connects to a WLAN and establishes a signalling IPsec SA with the N3IWF, the UE may establish also the PDU session over non-3GPP access. The two PDU sessions are linked together and they become child PDU sessions of a MA-PDU session.

To support the separate establishment, the existing "UE Requested PDU session establishment" procedure, as specified in TS 23.502, clause 4.3.2.2, can be used for each child PDU session. However, when the second child PDU session is established, then:

-	A PDU Session ID should be provided to 5GC in the PDU Session ID IE that refers to an existing PDU session. The 5GC shall link the second PDU session with the referred PDU session identified by the PDU Session ID and designate them as child PDU sessions of the same MA-PDU session.

-	The PDU Session Establishment Request message should include an MA-PDU Request flag and Request Type="Existing PDU Session". When the 5GC receives this PDU Session Establishment Request, the 5GC interprets that as a request to create a MA-PDU session by linking the requested PDU session with the existing PDU session.

An overview of the separate establishment procedure for a MA-PDU session is shown in the figure below. In this figure, the child PDU session over 3GPP access is established first and the child PDU session over non-3GPP is added subsequently (e.g. after the UE connects to an untrusted non-3GPP access network).

Figure 6.2.2.1-1: Overview of the UE Requested MA-PDU Session Establishment (Separate Establishment)

0.	If the UE is not registered via 3GPP access, the UE initiates Registration procedure via 3GPP access as described in TS 23.502 [6].

1.	The UE sends a PDU Session Establishment Request message with a new PDU Session ID and Request Type = "initial request". The UE also includes an MA-PDU Capability flag to indicate to the network that it can support MA-PDU sessions.

2.	The AMF selects an SMF as described in clause 4.3.2.2.3. In this step, the AMF stores Access Type of the PDU Session = "3GPP access". When the AMF received the MA-PDU Capability flag and it supports MA-PDU sessions, it includes an MA-PDU Capability flag in the Nsmf_PDUSession_CreateSMContext Request message sent to SMF.

3. The SMF registers with the UDM.

4. If needed, the secondary authorization/authentication procedure takes place, as specified in TS 23.502, clause 4.3.2.3.

5.	If the UE has sent an MA-PDU Capability flag and both the AMF and the SMF support MA-PDU sessions, the SMF sends an MA-PDU Capability flag to PCF when it performs Session Management Policy Establishment procedure. The PCF decides whether the MA-PDU session is allowed or not based on operator policy and subscription.

6-9. If the PCF permits the use of MA-PDU procedures for the requested PDU session (e.g. to later establish another child PDU over non-3GPP access), the SMF sends a PDU Session Establishment Accept message with a MA-PDU Capability flag to inform the UE that the network can support MA-PDU procedures for this PDU session. Otherwise, the SMF may accept the PDU Session establishment but does not include the MA-PDU Capability flag in the PDU Session Establishment Accept message.

If the MA-PDU Capability flag is received by the UE, the UE may later add another child PDU session to the existing PDU session with the steps below.

10.	If the UE is not registered via non-3GPP access, the UE initiates Registration via non-3GPP access as described in TS 23.502 [6]. The UE sends PDU Session Establishment Request message with PDU Session ID allocated in step 1, Request Type = "Existing PDU Session", MA-PDU Request flag and includes the same PDU Session parameters (e.g. S-NSSAI, DNN, PDU Session Type, SSC mode, PDU Session ID) used with the PDU Session established via 3GPP access. In the MM NAS message, the UE includes MA-PDU Request flag.

11.	The AMF selects the same SMF selected in step 2 based on Request Type = "Existing PDU Session". In this step, the AMF updates the Access Type of the PDU Session to "Multi-Access". When the AMF received MA-PDU Capability flag and it support MA-PDU sessions, it includes MA-PDU Capability flag in the Nsmf_PDUSession_CreateSMContext Request message sent to SMF.

NOTE 2:	The AMF may reject this request if the AMF cannot select the same SMF, e.g. 3GPP PDU Session is established as a roaming with LBO but the UE is registered in HPLMN via non-3GPP access.

12.	If the UE has sent an MA-PDU Request flag and the AMF and the SMF supports MA-PDU sessions, the SMF sends an MA-PDU Request flag to PCF when it performs Session Management Policy Modification procedure. The PCF decides whether the requested PDU session can be added to the existing PDU session based on operator policy and subscription.

13-15.	If the PCF did not permit the establishment of a MA-PDU Session, the SMF rejects the PDU session establishment request sent by the UE over non-3GPP access.

6.2.2.2	Combined Establishment

In this case, the two child PDU sessions are established in parallel with a single procedure called "UE Requested MA-PDU session establishment" procedure. This is useful when the UE decides to establish an MA-PDU session while it is already registered to 5GC via both accesses in the same PLMN. If a UE is registered to different PLMNs over 3GPP and non-3GPP accesses, the UE shall not use this procedure. Such UE can use the Separate Establishment procedure described in clause 6.2.2.1.

Instead of the UE initiating two separate PDU session establishment procedures, the UE initiates one MA-PDU session establishment procedure which results in the establishment of two child PDU sessions. The key advantage of the MA-PDU session establishment (versus the separate establishment of two PDU sessions) is less signalling over the air interface, less signalling over network interfaces, as well as fewer procedures in the UE and, thus, less battery consumption.

Editor's note: The advantages of the combined establishment versus the separate establishment should be further analyzed, considering also failure scenarios.

The figure below shows a high-level procedure of the combined establishment, i.e. it shows a "UE Requested MA-PDU session establishment" procedure.

Figure 6.2.2.2-1: Overview of the UE Requested MA-PDU Session Establishment (Combined Establishment)

0.	The UE is Registered via 3GPP access and non-3GPP access in the same PLMN.

1.	The UE initiates the "UE Requested PDU Session Establishment" procedure as specified in TS 23.502 [6], clause 4.3.2.2, and includes a single PDU Session ID, Request Type = "initial request", and a MA-PDU Request flag. In the MM NAS message, the UE includes the MA-PDU Request flag. The MA-PDU Request flag and the Request Type = "initial request", indicates that this PDU Session Establishment Request is to establish a Multi-Access PDU Session using a Combined Establishment procedure..

2.	The AMF selects an SMF and sends to SMF an Nsmf_PDUSession_CreateSMContext Request. When the AMF received the MA-PDU Request flag and it supports MA-PDU sessions, it includes an MA-PDU Capability flag in the Nsmf_PDUSession_CreateSMContext Request message sent to SMF.

If the AMF receives an initial request message with MA-PDU Request flag but the UE is registered via only one access, the AMF rejects the UE request.

3.	The SMF registers with the UDM .

4.	If needed, the secondary authorization/authentication procedure takes place, as specified in TS 23.502, clause 4.3.2.3.

5.	If dynamic PCC is deployed, the UE sent the MA-PDU Request flag, and both the AMF and the SMF support MA-PDU sessions, then the SMF includes sends an MA-PDU Request flag to PCF when it performs Session Management Policy Establishment procedure. The PCF decides whether the MA-PDU session is allowed or not based on operator policy and subscription.

The SMF retrieves the default Multi-Access PCC rules from PCF. The Multi-Access PCC rules include QoS rules for each child PDU session and include ATSSS rules that indicate how traffic within the MA-PDU session should be routed across the two child PDU sessions. Different QoS flows may be assigned to the child PDU sessions established over different accesses.

6-9.	The user-plane of the child PDU session over untrusted non-3GPP access is established.

-	In step 7 the SMF indicates to AMF that the transfer of N1/N2 information should take place over non-3GPP access.

-	In step 8 the normal steps of an N2 PDU session establishment over untrusted non-3GPP access are carried out.

10-13.	The user-plane of the child PDU session over 3GPP access is established.

-	The SMF also provides ATSSS forwarding rules to UPF that indicate how downlink traffic should be routed across the two child PDU sessions. The ATSSS forwarding rules are derived based on the Multi-Access PCC rules provided by PCF.

-	In step 11 the SMF indicates to AMF that the transfer of N1/N2 information should take place over 3GPP access.

-	In step 12 the normal steps of an N2 PDU session establishment over 3GPP access are carried out. The UE receives a PDU Session Establishment Accept message containing the PDU Session ID, and the MA-PDU Capability flag.  The UE determines that the MA-PDU session is established successfully when it receives the PDU Session Establishment Accept message including the MA-PDU Capability flag and user-plane resources on both accesses have been established.

-	In addition, in step 12 the UE may receive ATSSS forwarding rules that indicate how uplink traffic should be routed across the two child PDU sessions.

Editor's note: Further details of the "UE Requested MA-PDU session establishment" procedure (including PCC, QoS, handover and user-plane aspects) are FFS.

* * * End of Changes * * * *