SA WG2 Meeting #127	S2-183715

April 16 –April 20, 2018, Sanya, China	(was S2-18xxxx)

Source:	CATT

Title:	Update solution 6 of User Privacy Setting send via AMF

Document for:	Approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution: This pCR proposes to update solution 6 to solve FFS and correct some editorial errors.

1.	Discussion

This P-CR is proposed to solve the following FFS in solution 6:

Editors’ Note: How this solution works in roaming scenario is FFS.

To solve this FFS, this pCR adds the description of roaming scenario in the procedure. In the scenario, when the V-GMLC receives the UE privacy setting preference from serving AMF, it sends the preference to H-GMLC. Then, H-GMLC sends the preference to PPR.

Note: Services provided by PPR and GMLC is FFS.

To solve this FFS, this pCR adds the description of the services provided by PPR and GMLC.

Furthermore, this solution also corrects some editorial errors.

2.	Proposal

It is proposed to update solution 6 as following:

*** Start of changes ***

6.6	Solution 6: User Privacy Setting send via AMF

6.6.1	Introduction

This solution addresses Key Issue 4: Reduce overhead for repetitive non-successful privacy verification.

6.6.2	Functional Description

GMLC is responsible for privacy verification based on information from PPR and UE. The function of PPR and procedure between GMLC and PPR could be found in TS 23.271.

This solution mainly focuses on how UE provides privacy setting to GMLC. That is, once user has changed the Privacy Setting on UE, the UE updates the network with such changes during Registration procedure. Such update procedure involves AMF, PPR and GMLC. The PPR decides the UE privacy profile based on operator policy and UE provided Privacy Setting preference. The PPR sends the changed UE privacy profile to GMLC. When LCS request arrives, the GMLC performs the privacy verification. PPR may be a standalone network entity or it may be integrated in H-GMLC.

6.6.3	Procedures

6.6.3.1	Privacy Setting Update

The UE may provide Privacy Setting to GMLC during Registration procedure, once the Privacy Setting has changed by user. For this procedure, the V-GMLC is involved in roaming case.

# Figure 6.6.3-1: Privacy Setting Update procedure

1.	UE sends Registration Request to AMF. The message includes the UE privacy setting preference, e.g. allowed service type.

2.	AMF sends Ngmlc_UserPrivacy_Set Request to V-GMLC including the UE privacy setting preference and UE ID.

3.	V-GMLC sends Ngmlc_UserPrivacy_Set Request to H-GMLC.

4.	H-GMLC sends Nppr_PVPreference_Set Request to PPR. The message includes the UE ID and UE privacy setting preference.

5.	PPR sends Nppr_PVPreference_Set Response to H-GMLC, which includes the PPR decided UE Privacy Setting. PPR may change the privacy setting preference based on operator policy..

6.	H-GMLC stores the UE Privacy Setting, and sends Ngmlc_UserPrivacy_Set Response to V-GMLC. The message includes the UE Privacy Setting received from PPR.

7.	V-GMLC stores the UE privacy setting, and sends Ngmlc_UserPrivacy_Set Response to AMF. The message includes the UE Privacy Setting.

8.	AMF sends Registration Accept to UE. The message includes the UE Privacy Setting accepted by network.

0

6.6.3.2	Privacy Verification

When GMLC receives LCS Service Request from External Client, it performs privacy verification based on the subscriber LCS Privacy Profile (SLPP) information from PPR first and then the User privacy setting profile received from the PPR. If any one of the privacy verification fails, GMLC shall terminate the request, and response to the External client with appropriate error code. Otherwise, the GMLC continue the location procedure.

6.6.3.x	GMLC service

6.6.3.x.1	General

The following table shows the GMLC service and GMLC Service Operation.

# Table 6.6.3.x.1-1: GMLC Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_UserPrivacy | Set | Request/ Response | Peer GMLC, AMF |

6.6.3.x.2	Ngmlc_UserPrivacy_Set service operation

Service operation name: Ngmlc_UserPrivacy_Set

Description: Receives the UE Privacy Setting preference from the consumer NF and provides the Privacy Setting accepted by network to the consumer NF.

Input, Required: UE Privacy Setting preference, UE ID.

Input, Optional: None.

Output, Required: Privacy Setting accepted by network.

Output, Optional: None.

6.6.3.y	PPR service

6.6.3.y.1	General

The following table shows the PPR service and PPR Service Operation.

# Table 6.6.3.y.1-1: PPR Services

| Service Name | Service Operations | Operation Semantic | Known Consumer(s) |
| --- | --- | --- | --- |
| Nppr_PVPreference | Set | Request/ Response | GMLC |

6.6.3.y.2	Nppr_PVPreference_Set service operation

Service operation name: Nppr_PVPreference_Set

Description: Receives the UE Privacy Setting preference from the consumer NF and provides the Privacy Setting accepted by network to the consumer NF.

Input, Required: UE privacy setting preference, UE ID.

Input, Optional: None.

Output, Required: Privacy Setting accepted by network.

Output, Optional: None.

6.6.4	Impacts on existing entities and interfaces

Impacted entities:

-	UE provides privacy setting preference to AMF.

-	AMF:

-	Receive privacy setting preference from UE

-	Send the UE provided privacy setting preference to PPR, receive updated setting from PPR and provide it to UE.

-	PPR decides the privacy setting based on UE provided setting preference and operator policy, and provides it to GMLC.

-	GMLC receives UE privacy setting profile from PPR and performs privacy check based on the setting.

Impacted interfaces:

-	N1 and interface between PPR and GMLC are impacted.

6.6.5	Evaluation

Editor's Note: This clause provides an evaluation of the solution.

*** End of changes ***