3GPP TSG-SA2 Meeting #127	S2-183752

# Sanya, China, 16th Apr 2018 - 20th Apr 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0361 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting Registration Request parameters for EPS to 5GS Idle mode mobility with N26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the previous meeting it has been concluded to not include the list of PDU Session IDs and related S-NSSAIs in the Registration request message during EPS to 5GS mobility. However, TS 23.502 clause 4.11.1.3.3 (EPS to 5GS Mobility Registration Procedure (Idle and Connected State) using N26 interface) step 2 still includes the following text "The UE also includes the list of PDU Session IDs and related S-NSSAIs in the Registration request." In addition, the aspect that the UE "includes at least the S-NSSAIs associated with the established PDN connections in the Requested NSSAI in RRC and NAS" is mentioned twice in step 2. The same applies to " The additional GUTI enables the AMF to retrieve the UE's MM context from the old AMF (if available). " |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove from clause 4.11.1.3.3 step 2: - "The UE also includes the list of PDU Session IDs and related S-NSSAIs in the Registration request." - " The additional GUTI enables the AMF to retrieve the UE's MM context from the old AMF (if available). " - " The UE also includes at least the S-NSSAIs associated with the established PDN connections in the Requested NSSAI in RRC and NAS." |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary information sent in Registration request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

##### 4.11.1.3.3	EPS to 5GS Mobility Registration Procedure (Idle and Connected State) using N26 interface

Figure 4.11.1.3.3-1 describes the mobility procedure from EPS to 5GS when N26 is supported for idle and connected states.

Figure 4.11.1.3.3-1: EPS to 5GS mobility for single-registration mode with N26 interface

1.	The Registration procedure is triggered, e.g. the UE moves into NG-RAN coverage. Step 2 to 9 except step 5, 6 and 8 follow the Registration procedure in clause 4.2.2 with following enhancement.

2.	The UE sends Registration Request with registration type set to "Mobility Registration Update", including 5G-GUTI mapped from EPS GUTI as the old GUTI, the native 5G-GUTI (if available) as additional GUTI and indicating that the UE is moving from EPC. The additional GUTI enables the AMF to retrieve the UE's MM context from the old AMF (if available). The UE also includes at least the S-NSSAIs associated with the established PDN connections in the Requested NSSAI in RRC and NAS (as described in TS 23.501 [2] clause 5.15.7.2 or 5.15.7.3).

In the case of idle mode mobility the UE additionally includes a TAU request message integrity protected using the EPS security context (for further security verification by the MME) in the Registration Request. If the UE holds a native 5G-GUTI for this PLMN then the UE also includes the GUAMI part of the native 5G-GUTI in RRC to enable the NG-RAN to route the Registration Request to the same AMF (if available).

The UE integrity protects the Registration Request message using a 5G security context (if available).

3-4.	Steps 2-3 of clause 4.2.2.2.2 are performed.

Step 5, 6 and 8 are not performed when this procedure is part of EPS to 5GS handover.

5.	This step is only performed for IDLE mode mobility. The target AMF derives the MME address and 4G GUTI from the old 5G-GUTI and sends Context Request to MME including EPS GUTI mapped from 5G-GUTI and the TAU request message according to TS 23.401 [13].

6.	This step is only performed for IDLE mode mobility. Step 5 from clause 5.3.3.1 (Tracking Area Update procedure with Serving GW change) in TS 23.401 [13] with the modification captured in clause 4.11.1.5.3.

The AMF converts the received EPS MM Context into the 5GS MM Context. The UE context includes IMSI, ME Identity, UE security context, UE Network Capability, and EPS Bearer context(s). An EPS Bearer context includes the SMF + PGW-C address and APN.

7.	Steps 8-9 of clause 4.2.2.2.2 are optionally performed.

8.	This step is only performed for IDLE mode mobility. The target AMF sends Context Acknowledge (Serving GW change indication) to MME according to TS 23.401 [13].

9.	Steps 11-12 of clause 4.2.2.2.2 are optionally performed.

Step 10, 12, 13 are performed only if the target AMF is different from the old AMF and the old AMF is in the same PLMN as the target AMF.

10.	If the UE' additional 5G-GUTI was included in the Registration Request, the target AMF shall invoke the Namf_Communication_UEContextTransfer to the old AMF identified by the additional 5G-GUTI, to request the UE's SUPI and MM Context. The old AMF associated to the additional 5G-GUTI also transfers the event subscriptions information by each consumer NF and the list of SM PDU Session ID / associated SMF ID, for the UE, to the target AMF.

11.	Steps 13-14c of clause 4.2.2.2.2 are performed, with the following difference: if MM context retrieved from the old AMF indicates non-3GPP access, then the target AMF indicates to the UDM that the target AMF identity to be registered in the UDM applies to both 3GPP and non-3GPP accesses.

12.	The old AMF unsubscribes with the UDM for subscription data using Nudm_SDM_Unsubscribe. The UDM may unsubscribe to changes for the same data from UDR by using Nudr_DM_Unsubscribe.

13.	The UDM invokes Nudm_UECM_DeregistrationNotification to the old AMF. The UDM may update UE context in AMF information from UDR by Nudr_DM_Update.

14.	Steps 16-20 of clause 4.2.2.2.2 are optionally performed (initiated by target AMF) with the following addition:

In the home-routed roaming case, the AMF selects default V-SMFs and invokes Nsmf_PDU_Session_CreateSMContext service operation of the V-SMF to create an association with the AMF. It includes EPS Bearer ID, H-SMF ID, indicates whether User plane connection needs to be established based on active flag setting along with List Of PDU Sessions To Be Activated (i.e. PDN connections). V-SMF creates the association and based on the received SMF ID, V-SMF invokes Nsmf_PDU_Session_Update request service operation of H-SMF and provides the information received.

-	The (H-)SMF finds the corresponding PDU Session based on the PDN Connection Context in the request. The (H-)SMF responds with the PDU Session ID, other PDU session parameters, such as PDU Session Type, Session AMBR in the Nsmf_PDU_Session_Update response. The V-SMF and AMF stores an association of the PDU Session ID and the SMF ID.

In non-roaming and LBO cases, AMF invokes Nsmf_PDU_Session_UpdateSMContext Request (UE EPS PDN Connection) service operation of  the PGW-C+SMF and indicates all the PDU Session(s) to be re-activated as received in the Registration request message along with List Of PDU Sessions To Be Activated (i.e. EPS bearer contexts). This step is performed for each PDN Connection and the corresponding PGW-C+SMF address/ID in the UE context the AMF received in Step 6. The PGW-C+SMF sends the PDU Session ID and the S-NSSAI of the PDU Session in the Nsmf_PDU_Session_UpdateSMContext Response. The PGW-C+SMF finds the corresponding PDU Session based on the EPS Bearer Context(s). in the request. The PGW-C+SMF activates User Plane connections if the corresponding PDU Session is in the received PDU Session(s) to be re-activated. The PGW-C+SMF allocates its local resources and sends session modification request message to UPF(s). UPF(s) responds to the PGW-C+SMF(s) with session modification response. The PGW-C+SMF(s) updates its SM contexts and returns a Nsmf_PDU_Session_UpdateSMContextResponse message including N2 SM Context to the AMF depending on the need to establish User plane connection. The SMF responds with the PDU Session ID corresponding to the SMF ID, default EPS bearer ID in the request, and with the allocated EBI(s) information. The AMF stores an association of the PDU Session ID and the SMF ID, AMF stores the allocated EBI(s) associated to the PDU Session ID.

In case the PDN Type of a PDN Connection in EPS is non-IP, and it was originally established as Ethernet PDU Session when UE was camping in 5GS (known based on local context information that was set to PDU Session Type Ethernet in UE and SMF), the PDU Session Type in 5GS shall be set to Ethernet by the SMF and UE. In case the PDN type of a PDN Connection in EPS is non-IP, and is locally associated in UE and SMF to PDU Session Type Unstructured, the PDU Session Type in 5GS shall be set to Unstructured by the SMF and UE.

15 - 16.	Step 13 - 14 from clause 5.3.3.1 (Tracking Area Update procedure with Serving GW change) in TS 23.401 [13].

17-18.	These steps follow the steps 21 and 22 of Registration procedure in clause 4.2.2.2.2.

The Registration Accept message shall include the updated 5G-GUTI to be used by the UE in that PLMN over any access. If the active flag was included in the Registration request, Tthe AMF may provide NG-RAN with a Handover Restriction List taking into account the last used EPS PLMN ID and the Return preferred indication. The Handover Restriction List contains a list of PLMN IDs as specified by TS 23.501 [2].

If a native 5G security for 3GPP access is available in the AMF (or has been retrieved from the old AMF), the AMF may continue to use this security context. Otherwise, the AMF shall either create the mapped security context based on the EPS security context obtained from the MME or initiate an authentication of the UE.

*** End of changes ***