SA WG2 Meeting #S2-127	S2-183818

23 - 27 April, 2018, Sanya, China

Source:	Vodafone

Title:	New solution: Addition of SGW-U as uplink classifier instead of relocating S/PGW-U

Document for:	Approval

Agenda Item:	6.17

Work Item / Release:	FS_LLC_Mob / Rel-16

## 1	Introduction

The operator may not wish to provision all of their PGW-U capacity close to the edge of the network and the operator may wish to only add ‘low latency UPFs’ “on demand”.

The operator may also wish to provide access to these low latency user plane functions to existing, pre-release 15 UEs.

Release 14 CUPS (TSs 23.214/29.244) seems to provide tools that enable a SGW-U to also support Uplink Classifier type functionality such as the breakout of certain flows, usage reporting for those flows, LI, etc.

This paper proposes that the use of the application detection described in solution #3 in TR 23.739 can be used to trigger the insertion of an SGW-U configured as an “uplink classifier” to provide break out of the required low latency application’s traffic.

## 2	Proposal

It is proposed to add the following text to TR 23.739 v0.2.1

****************  start of ALL NEW text ***********************

## 6.Y	Solution #Y: Detection of application requiring low latency which leads to the addition of a user plane node as an Uplink Classifier

### 6.Y.1	Introduction

The solution addresses Key Issue #1 "How to detect the activation of a service requiring low latency user plane support".

This solution targets the use case where an operator has much of their (legacy) SGi-LAN infrastructure centralized and only wishes to use the limited capacity of "Edge" SGi-LAN infrastructure when the UE is actually using an application that will benefit from the low user plane latency.

This solution also addresses the case where an operator wishes to provide this lower latency support using pre-Release 15 UEs.

### 6.Y.2	Functional Description

The MME uses the APN to select a combined S/PGW-C.

The operator has prior knowledge of which applications (and which UEs) they want to provide with very low latency. The operator has knowledge of which user plane functions can provide access to the servers for such low latency applications, and has provisioned this information in the S/PGW-Cs.

The PCRF requests application detection to detect when those UEs are using those applications.

Upon the detection of the application (or detection of UE mobility), the S/PGW-C may decide to connect in an additional user plane node in between the RAN and the SGW-U. This user plane node is in a low latency location (closer to the UE location). This user plane node has SGW-U and PGW-U functionality, but appears as a SGW-U to the RAN and the PGW-U. When such a node is added, it is reported to the PCRF.

The PCRF will provide the relevant rules to configure this user plane node to breakout the flows related to this application (along with related usage reporting). This requires this user plane node to have SGW-U capabilities and some PGW-U capabilities: but combined S/PGW-Us are supported by the Release 14 CUPS specifications.

### 6.Y.3	Procedure

This solution describes a triggering mechanism that, after triggering, can then add an appropriately located user plane node and provide it with traffic breakout rules.

The solution comprises the following steps:

1)	At PDN connection establishment, the PGW-C contacts the PCRF (IMSI, APN).

2)	The PCRF contacts the SPR/UDR and the SPR/UDR downloads subscription policy data relevant for that APN and that UE. For this solution, the APN/UE combination is tagged in the SPR/UDR for "application dependent low latency".

3)	The PCRF requests the PDN-GW to activate User Location Reporting or Presence Reporting Area reporting. The PCRF requests the TDF/PCEF to detect when traffic for the low latency application is detected.

4) At detection of the low latency application (or UE mobility), the S/PGW-C uses its DNS (or local configuration) to determine the appropriate user plane function using APN and the UE's reported eNB ID/TAI as inputs. This type of DNS enquiry is specified in clause 5.10 of TS 29.303 [4].

5)	If the DNS response (or use of the local configuration) indicates that the insertion of an additional user plane node is needed for low latency, then the CUPS procedures for insertion of a user plane node are initiated. The S/PGW-C sends a [new] message to the MME so that the MME sends an E-RAB modify message to the RAN to update the RAN with the new uplink transport address for that PDN connection. The S/PGW-C informs the original PGW-U of the downlink transport address for that PDN connection.

6) The S/PGW-C reports and requests session modification to the PCRF and new policies will be installed by the S/PGW-C in the new user plane node for diverting the low latency traffic; detecting the start/stop of the application traffic; and performing GTP-U forwarding for the other flows.

If no dynamic rules are used, the S/PGW-C will use locally configured rules.

After step 6, the S/PGW-C can return the PDN connection to the central S/PGW-U, at detection of application traffic stop and report it to PCRF, which will instruct new policies for the modified session.

### 6.3.4	Impacts on existing entities and interfaces

Editor's note:	This clause is not necessarily complete.

a)	The SPR/UDR/PCRF needs to be configured with data as to which UEs and which APNs are tagged for "application dependent low latency" service. This is in line with the specification in TS 29.335 [7] for the operation of the Ud interface between UDR and PCRF.

b)	Potentially, a new event trigger codepoint needs to be defined (e.g. UE mobility with low latency application) on the PCRF-PCEF interface.

c)	The PGW-C needs to be configured with information related to which user plane functions have access to Application Servers deployed locally, and needs to react to the application usage and/or mobility events of low latency users potentially allocating an additional user plane node as an uplink classifier.

d)	In order to return the UE's S/PGW-U to a central location, application detection functionality is needed at the 'low latency' user plane node location. This could be a standalone TDF or implemented within the user plane node acting as PCEF.

e)	at step 5, a new GTP-C message may be needed in order for the SGW-C to inform the MME that the uplink transport address for that PDN connection has changed. MME functionality may be needed to generate the subsequent S1-AP E-RAB modification signalling.

f)	If the MME requests SGW relocation at some time after the insertion of the additional user plane node, the S/PGW-C treats the additional user plane node as a PGW-U for that procedure.

g)	the UE is not informed of the addition (or removal) of the traffic breakout function at the additional user plane node. This may impact the UE’s application.

### 6.3.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

************* end of new added text **************