SA WG2 Meeting #127	S2-184581

Apr 16 – 20, 2018, Sanya, China	(was 4237 WAS 3823 was3717)

Source:	CATT, China Mobile

Title:	New Key Issue: Extend Service Concept into User Plane

Document for:	Approval

Agenda Item:	6.X

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a new key issue for Extend Service Concept into User Plane and related aspects.

1. Discussion

Following aspect is within the scope of the study; hence corresponding key issue should be included in the TR.

-----------------------

2) Study the feasibility of extending the service concept from 5GC control plane to the user plane function(s)

NOTE: Impact to User plane traffic processing is not expected in this study.

-----------------------

2 Proposal

It is propose to include following Key Issue in the TR.

*** Start of 1st changes ***all-new text***

5.x Key Issue X: Extend Service Concept into User Plane

5.x.1 Description

In Rel-15, the Service Concept has been introduced into control plane of 5G core. This key issue will study extending the service concept from 5GC control plane to the user plane function(s), however, this key issue will focus on how to extend service concept only to the N4 interface, but not to the N3/N6/N9 interface.

This key issue study following aspects:

-	How to integrate the specific aspects of the UPF (e.g. its resources, states of PDU Sessions and user plane tunnels,etc) into the service-based architecture model and make sure that those aspects of UPF are encompassed by existing principles of SBA and those going to be newly defined during this study;

-

What does UPF will expose/consume on the service interface to/from SMF? e.g. service related to PDU Sessions and user plane tunnel establishment, data usage report, and etc.

-	Any impact to the session management procedure defined in TS23.501[x] and TS23.502[y] with UPF having a service interface?

-	The bootstrap procedures of UPF with service interface, e.g. Whether the Procedure defined in section 4.17 of TS23.502[y] can be re-used or need to define new procedures?

-	Within the mix deployment of both the UPF using PtP interface and UPF using service interface, how to handle session management procedures e.g. UPF selection, UPF relocation, etc? Is it possible to use both UPF using P2P interface and UPF using service interface for a UE/PDU session?

NOTE: For solutions of this key issue, the impact to User plane traffic processing is not expected.

*** Start of 2nd changes

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[y]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

*** End of changes ***