SA WG2 Meeting #127	S2-184637

April 16 – 20, Sanya, China	(e-mail revision 2 of S2-184182)

Source:	Huawei, HiSilicon

Title:	Solutions for common IMS network through multiple 5GC network slices

Document for:	Approval

Agenda Item:	6.14

Work Item / Release:	FS_eIMS5G / Release 16

Abstract of the contribution: Proposes one solutions for common IMS network through multiple 5GC network slices

# Introduction

This paper proposes a solution to address the network slicing with common IMS network scenario of Key Issue 3: Network Slicing and IMS Key Issue.

# 2. Discussion

In Network Slicing and IMS Key Issue, there is a scenario “Each 5GC slice is associated with a common IMS network” as shown in the following diagram:

For example, one UE has two 5GC slices. The UE has subscribed one IMPI and one IMPU, and accesses to an IMS network.

In the TS 23.501, the UE can choose correct S-NSSAI applicable to a specific application according to URSP and decide to use the existing PDU Session or establish new PDU Session(s):

The NSSP rules associate an application with one or more Subscribed S-NSSAI corresponding to the Subscribed S-NSSAIs of the UE.

The UE uses UE Configuration (e.g. NSSP in the URSP rules) to determine whether ongoing traffic can be routed over existing PDU Sessions belonging to other Network Slices or establish new PDU Session(s) associated with same/other Network Slice

There are two types of IMS application Identification defined in 3GPP TS 23.228, which are ICSI (IMS Communication Service Identifiers) and IARI (Identification of IMS applications). The following diagram which is excerpted from 3GPP TS 23.228 describes the relationship between ICSI and IARI:

Figure 4.13-1: IMS applications on top of an IMS communication service

The UE can use IMS application information to choose correct S-NSSAI, and IMS application information may include ICSIs or IARIs.

# 2. Proposal

It is proposed to agree the following solution for network slicing with common IMS network.

* * * * Start of Change * * * *

## 6.x	Solution #X: IMS signalling and media of an application through one 5GC slice

### 6.x.1	Description

This solution addresses network slicing with common IMS network scenario of Key Issue 3: Network Slicing and IMS Key Issue.

As an example for this scenario, one 5GC slice is used for common voice communication application and another 5GC slice is used for public safety communication application.

IMS signalling and media of an application go through the same 5GC slice, so each application can map to a single 5GC slice.

In this solution, the UE initiates independent IMS registration through each 5GC slice with the common IMS network.

The procedure is shown as the following:

Figure 6.x.1-1: IMS signalling and media of an application through one 5GC slice

1.	When the UE registers to 5GC, the UE gets Allowed NSSAI, containing the S-NSSAIs it can use, and URSP.

2.	According to URSP and IMS application information the UE requests, the UE establishes the PDU Sessions with each corresponding 5GC Slice for specific IMS application. The UE uses the IMS application information it requests to select the corresponding 5GC Slice. The IMS application information may contain ICSIs or IARIs, as defined in the TS 23.228 [2].

3.	For each PDU session established for IMS, the UE initiates registration request to IMS network with the corresponding IMS application information through the corresponding 5GC slice.

Editor's note:	It is FFS what is the 5G CN entity that communicates with the IMS network.

4.	After successful IMS registration, the S-CSCF records the binding relationship between the IMS application information and the UE contact address.

Editor's note:	It is FFS what is the UE contact address.

5a.	For MO, the UE sends the SIP request with the requested IMS application information to the IMS network. The Originating S-CSCF routes the SIP request together with the IMS application information to the IMS network of the called party, after routing the SIP request to the appropriate application servers for originating services according to the requested IMS application information.

5b.	For MT, the Terminating S-CSCF routes the SIP request to the appropriate application servers for terminating services according to the requested IMS application information.

The Terminating S-CSCF decides the UE contact address registered for the requested IMS application, then routes the SIP request to the UE with the matched UE contact address.

When the SIP request does not contain specific IMS application information, the Terminating S-CSCF selects the appropriate application server based on the contents of the SIP request (such as SDP media capabilities, Content-Type header field, media feature tag), and route the SIP request to the UE with the corresponding UE contact address.

Editor's note:	In MT calls, it is FFS how a non-supporting terminating S-CSCF selects the appropriate application server.

Editor's note:	In MT calls, it is FFS whether the selection of the application server based on IMS application information and the selection of the application server based on SDP contents would give same results or not.

### 6.x.2	Impacts on existing nodes and functions

On the S-CSCF:

After successful registration, S-CSCF records the binding relationship between the registered IMS application information and the UE contact address.

The Terminating S-CSCF decides the UE contact address according to the requested IMS application information, then routes the request to the UE with the matched UE contact address.

On the UE:

According to URSP and IMS application information the UE requests, the UE establishes the PDU Sessions with each corresponding 5GC Slice for specific IMS application.

For each PDU session for IMS, the UE initiates registration request to IMS network with the corresponding IMS application information. The UE uses the IMS application information it requests to select the corresponding 5GC Slice. The IMS application information may contain ICSIs or IARIs.

### 6.x.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.

* * * * End of Changes * * * *