SA WG2 Meeting #127	S2-184644

Apr 16 – 20, 2018, Sanya, China	(e-mail revision 6 of S2-184581)

Source:	CATT, China Mobile

Title:	New Key Issue: Extend Service Concept into User Plane

Document for:	Approval

Agenda Item:	6.X

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a new key issue for Extend Service Concept into User Plane and related aspects.

# 1. Discussion

Following aspect is within the scope of the study; hence corresponding key issue should be included in the TR.

-----------------------

2) Study the feasibility of extending the service concept from 5GC control plane to the user plane function(s)

NOTE: Impact to User plane traffic processing is not expected in this study.

-----------------------

# 2 Proposal

It is propose to include following Key Issue in the TR.

## *** Start of 1st changes ***all-new text***

## 5.x Key Issue X: Extend Service Concept into User Plane

### 5.x.1 Description

In Rel-15, the Service Concept has been introduced into control plane of 5G core. This key issue will study extending the service concept from 5GC control plane to the user plane function(s), however, this key issue will focus on how to extend service concept only to the N4 interface, but not to the N3/N6/N9 interface.

This key issue study following aspects:

-	How to integrate the specific aspects of the UPF (e.g. its resources, states of PDU Sessions and user plane tunnels,etc.) into the service-based architecture model and make sure that those aspects of UPF are encompassed by existing principles of SBA and those going to be newly defined during this study;

-	What will UPF expose/consume on the service interfaces to/from SMF? e.g. service related to PDU Sessions and user plane tunnel establishment etc.

-	Possibility of structuring and separating services into control, reporting and exposure services.

-	Any impact to the session management procedure defined in TS 23.501 [x] and TS 23.502 [y] due to UPF having a service based interface?

-	The bootstrap procedures of UPF with service interface, e.g. Whether the Procedure defined in clause 4.17 of TS 23.502 [y] can be re-used or need to define new procedures?

-	Within the mix deployment of both the UPF using PtP interface and UPF using service based interface e.g. within a single PLMN, how to handle session management procedures e.g. UPF selection, UPF relocation, etc?.

NOTE:	As stated in the Objectives of the eSBA SID, impacts to User Plane traffic processing are not expected in the eSBA study. Therefore, solutions proposed to address this key issue are not expected to impact functionality handling User plane traffic processing. In addition, solutions addressing this key issue shall not impactN3/N9 tunnelling protocols defined in Rel-15.

## *** Start of 2nd changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[y]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

## *** End of changes ***