SA WG2 Meeting #128bis	S2-188264

20 - 24 August 2018, Sophia Antipolis, France

Source:	Huawei, HiSilicon

Title:	        Solution for Data Collection from AFs using Existing SA2 Services

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a solution for data collection from AFs using existing SA2 services as defined in TS 23.502.

1 Discussion

The existing Network Exposure mechanism defined in TS 23.502 seems also applied to the data collection from AFs, e.g.

- “4.15.6 External Parameter Provisioning” applies to data collection from AFs to NWDAF.

Hence it is proposed that NWDAF reuse the current Network Exposure mechanism defined in TS 23.502 for Data collection from AFs.

2 Proposal

It is proposed to make the following changes to the TR 23.791.

* * * * Start of Change * * * *

6.X	Solution X: Data Collection from AFs using the existing SA2 services

6.X.1	Description

This solution addresses the Key Issue #3: "Interactions with 5GS NFs/AFs for Data Collection".

This solution reuses the current Network Exposure mechanism as defined in TS 23.502[3] for data collection from AFs.

6.X.1.1 Procedure for Data Collection from AFs

This procedure is used by NWDAF to subscribe/unsubscribe at AF to be notified on service data, using existing external parameter provisioning services as defined in clause 4.15.6 TS 23.502[3].

Figure 6.X.1.1-1: Procedure for Data Collection from AFs

1-7. Same procedures as specified in step 1-7, clause 4.15.6.2 TS 23.502[3].

8.	(This step occurs only after successful 4) UDM may notify the subscribed NWDAF of the updated subscriber data via Nudm_SDM_Notification Notify message.

NOTE: The NWDAF could use the immediate reporting flag as defined in Table 4.15.1-1 TS 23.502[3] to meet the request-response model for data collection from AFs.

6.1.2	Impacts on Existing Nodes and Functionality

Editor's note:	Capture impacts on existing 3GPP nodes and functional elements.

6.1.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * End of Change * * *