3GPP TSG-SA2 Meeting #128Bis	S2-188356

# Sophia Antipolis, France, August 20th - 24th  2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0617 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on NFprofile parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS-Ph1 |  |  |  |  |  | Date: |  |  | 2018-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to clause 6.2.6 of TS 23.501 [2], it is indicated that the NF profile includes the following parameters: “NF profile of NF instance maintained in an NRF includes the following information: - NF instance ID - NF type - PLMN ID - Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID - FQDN or IP address of NF - NF capacity information - NF Specific Service authorization information - Names of supported services - Endpoint Address(es) of instance(s) of each supported service - Identification of stored data/information” However, the “Names of supported services” is specified as optional parameter by stage 3 in the NF profile in TS 29510 section 6.2.6.2.3. Especially, this parameter is not applicable if the registering NF is not a service producer (e.g. AF, UPF) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - indicate “If applicable” before “Names of supported services” parameter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | incoherence with stage 3 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 6.2.6	NRF

The Network Repository Function (NRF) supports the following functionality:

-	Supports service discovery function. Receive NF Discovery Request from NF instance, and provides the information of the discovered NF instances (be discovered) to the NF instance.

-	Maintains the NF profile of available NF instances and their supported services.

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID

-	NF type

-	PLMN ID

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID

-	FQDN or IP address of NF

-	NF capacity information

-	NF Specific Service authorization information

-	if applicable, Names of supported services

-	Endpoint Address(es) of instance(s) of each supported service

-	Identification of stored data/information

NOTE 1:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in TS 23.502 [3] clause 5.2.7.2.2. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g., DNN, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Routing ID part of SUCI.

-	One or more GUAMI(s), in case of AMF.

-	TAI(s), in case of AMF.

-	UDM Group ID, applicable for UDM only.

-	UDR Group ID, applicable for UDR only.

-	AUSF Group ID, applicable for AUSF only.

NOTE 2:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 3:	It is also expected that the NRF stores a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) to enable discovery of UDM, UDR, AUSF using SUPI, SUPI ranges as specified in clause 6.3.

In the context of Network Slicing, based on network implementation, multiple NRFs can be deployed at different levels (see clause 5.15.5):

-	PLMN level (the NRF is configured with information for the whole PLMN),

-	shared-slice level (the NRF is configured with information belonging to a set of Network Slices),

-	slice-specific level (the NRF is configured with information belonging to an S-NSSAI).

In the context of roaming, multiple NRFs may be deployed in the different networks (see clause 4.2.4):

-	the NRF(s) in the Visited PLMN (known as the vNRF) configured with information for the visited PLMN.

-	the NRF(s) in the Home PLMN (known as the hNRF) configured with information for the home PLMN, referenced by the vNRF via the N27 interface,

**** End of Changes ****