SA WG2 Meeting #128	S2-186414

2 – 6 July, Vilnius, Lithuania 	(revision of S2-18xxxx)

Source:	Ericsson

Title:	Solution proposal “hierarchical service framework” to address key issue “Improvements to service framework related aspects”

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a solution that addresses key issue "Improvements to service framework related aspects”

# 1 Proposal

This document proposes to add following solution addressing key issue “Improvements to service framework related aspects” to TR 23.742.

* * * Start of Change (ALL TEXT IS NEW) * * *

## 6.x Solution #X: Hierarchical service framework

### 6.x.1 Introduction

This solution addresses key issues 3 “Improvements to service framework related aspects”.

### 6.x.2 High level description

New technologies are coming for inter container communication, such as message bus communication solutions and service mesh solutions. This solution allows for integrating these solutions into the existing service framework. See figure 6.x.2-1

figure 6.x.2-1 hierarchical service framework

In this architecture the new innovations can be introduced within each service cluster. A service cluster may span just a POD, data centre or even several sites, it is really up to how an operator wants to deploy this architecture. The service cluster hides the specifics of the cluster specific service framework. All services that shall be reachable from the outside world are registered in the NRF. The different clusters may use rel 15 mechanisms to register these services. Exactly how that is done within the service cluster is service cluster specific, it could be from the service itself or from some entity within the cluster specific service framework.

Editor's note: If a cluster should be part of the NF profile and be part of a discovery is FFS

If a service consumer within the service cluster need to connect to a service producer outside the service cluster, some entity in the service cluster (the service itself or the cluster specific service framework) will discover and select the service producer according to rel 15 principles. Inter cluster addressing of services are done according to rel. 15. The external entry and exit points of the service cluster for a service producer and service consumer may be the service itself, the NF, or some entity within the service cluster specific framework.

The figure 6.x.2-1 shows an example of 3 different service clusters. The NF/NF services that are addressable outside the clusters have non-dotted lines. For example, NF A and its services are present in service cluster #1 and service cluster #2, service cluster #3 does not provide NF A and its services (at least not outside the cluster). The figure also shows some internal cluster services by dotted lines, for example services x1 and x2. These services are not reachable from outside the service cluster #1, only consumers within service cluster #1 may consume these services, and thus they are not registered within the NRF. It is even so that the services that are reachable from the outside world, may be built up of several services within the service cluster, however this would be completely hidden from outside of the service cluster. This is illustrated in service cluster #3, where NF B and service b1 and b2 are build-up of services z1 through z6. The addressable resources from the outside world in service cluster #3 are services b1 and b2, but internally, service z1 through z6 execute them (c1 and c2). Service z1 through z6 are hidden and not addressable from outside.

This architecture will allow for adopting new technologies, but at the same time keep existing Rel 15 NFs and NF services if needed.

The inter cluster service framework may be a distributed or centralized service framework according to 6.x “Flexible service framework”

### 6.x.4 Impacts on existing NF/NF Services and Interfaces

No impact on existing NF/NF services and interfaces.

### 6.x.5 Evaluation of the Solution

Editor's note: This clause provides an evaluation of the solution.

End of changes (all new text)