3GPP TSG-SA2 Meeting #128	S2-186448

# Vilnius, Lithuania, 2nd Jul 2018 - 6th Jul 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0467 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MO service correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A UE set to "voice centric" for 5GS shall always try to ensure that Voice service is possible, even if E-UTRAN connected to EPC is not available. Section 5.15.3.5 defines how a UE selects a domain for voice calls. If 5GS is not available there is a bullet that describes what the UE should do next, specifically this text is of interest: “By disabling capabilities to access 5GS, the UE re-selects to E-UTRAN connected to EPC first (if available)” One will notice 2 issues with this text - that in brackets it says “if available”. There is no functionality specified if the opposite is true, E-UTRAN is not available. This needs to be added. - First indicates there is some else that could occur, nothing has been defined. The UE only performs Voice Domain Selection procedures as defined in TS 23.221 when connected via E-UTRAN to EPC. Thus a requirement is needed in this TS to select GERAN/UTRAN for voice servies while still disabling the capability to access 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Select GERAN/UTRAN when E-UTRAN connected to EPC is not available, if the UE is "voice centric". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A "voice centric" UE fails to ensure that voice service is possible even if UTRAN or GERAN is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.16.3.5	Domain selection for UE originating sessions / calls

For UE originating calls, the 5GC capable UE performs access domain selection. The UE shall be able to take following factors into account for access domain selection decision:

-	The state of the UE in the IMS. The state information shall include: Registered, Unregistered.

-	The "IMS voice over PS session supported indication" as defined in clause 5.16.3.2.

-	Whether the UE is expected to behave in a "voice centric" or "data centric" way for 5GS.

-	UE capability of supporting IMS PS voice.

-	UE capability for operating in dual-registration mode with selective PDU Session transfer as defined in clause 5.17.2.3.3.

-	Whether 3GPP PS Data Off is active or not and whether IMS voice is included in 3GPP PS Data Off Exempt Services or not as defined in clause 5.24.

NOTE 1:	In this release of the specification, the exact logic of which PDU sessions are kept in which system for Dual Registration UE with selective transfer of certain PDU Sessions as defined in clause 5.17.2.3.3, is left up to UE implementation. The voice centric UE will keep the PDU Session used for IMS services to a system that supports voice over IMS. The voice centric UE can re-register with the IMS (if needed) when the IMS PDU session is transferred between 5GS and EPS.

To allow for appropriate domain selection for originating voice calls, the UE shall attempt Initial Registration in 5GC. If the UE fails to use IMS for voice, e.g. due to "IMS voice over PS session supported indication" indicates voice is not supported in 5G System, the UE behaves as described below for "voice centric" for 5GS or "data centric" for 5GS:

-	A UE set to "voice centric" for 5GS shall always try to ensure that Voice service is possible. A voice centric 5GC capable and EPC capable UE unable to obtain voice service in 5GS shall not select a cell connected only to 5GC. By disabling capabilities to access 5GS, the UE re-selects to E-UTRAN connected to EPC first (if available). When the UE selects E-UTRAN connected to EPC, the UE performs Voice Domain Selection procedures as defined in TS 23.221 [23]. When the UE is unable to select E-UTRAN connected to EPC, the UE shall attempt to select GERAN/UTRAN (see clause 5.17.2.4). When the UE selects GERAN/UTRAN, the UE performs Voice Domain Selection procedures as defined in TS 23.221 [23].

-	A UE set to "data centric" for 5GS does not need to perform any reselection if voice services cannot be obtained.

NOTE 2:	The related radio capabilities in order for the voice centric UE to not reselect to NR or E-UTRA cell connected to 5GC (i.e. avoid ping pong) will be defined by RAN WGs.