3GPP TSG-SA2 Meeting #128	S2-186465

# Vilnius, Lithuania, 2nd July – 6th July 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0097 | rev |  | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Session selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2018-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When there are multiple PDU sessions that have the same selector value (S-NSSAI, DNN, SSC Mode, etc.) derived from the URSP, the current spec leaves it to UE implementation to select one PDU session for the application traffic. However, during the SSC Mode 3 PDU Session anchor change procedure as described in clause 4.3.5.2 of TS 23.502 [3], two PDU Sessions of the same values may exist for a period of time, and in this case it should be explicitly specified that the UE should not select the old PDU session which is to be released soon. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A note is added to clarify that for the case of SSC mode 3 anchor change, the UE should select the new PDU session (the one without a Lifetime value assigned) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE might select a PDU session that is soon to be released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** Start of Changes ****

#### 6.6.2.3	UE procedure for associating applications to PDU Sessions based on URSP

For every newly detected application the UE evaluates the URSP rules in the order of Rule Precedence and determines if the application is matching the Traffic descriptor of any URSP rule.

When the Application ID, DNN or traffic of the application is found matching a Traffic descriptor in an URSP rule, the UE shall select a Route Selection Descriptor within this URSP rule in the order of the Route Selection Descriptor Precedence. The selected Route Selection Descriptor shall be only valid if all of the following criteria are fulfilled:

-	If any S-NSSAI(s) is present, the S-NSSAI(s) is in the Allowed S-NSSAI(s).

-	If any DNN is present and the DNN is an LADN DNN, the UE is the area of availability of this LADN.

When a valid Route Selection Descriptor is found, the UE determines if there is an existing PDU Session that matches all components in the selected Route Selection Descriptor. The UE compares the components of the selected Route Selection Descriptor with the existing PDU Session(s) as follows:

-	For a component which only contains one value (e.g. SSC mode), the value of the PDU Session has to be identical to the value specified in the Route Selection Descriptor.

-	For a component which contains a list of values (e.g. Network Slice Selection), the value of the PDU Session has to be identical to one of the values specified in the Route Selection Descriptor.

When a matching PDU Session exists the UE associates the application to the existing PDU Session, i.e. route the traffic of the detected application on this PDU Session.

If the UE determines that there is more than one existing PDU Session which matches (e.g. the selected Route Selection Descriptor only specifies the Network Slice Selection, while there are multiple existing PDU Sessions matching the Network Slice Selection with different DNNs), it is up to UE implementation to select one of them to use.

NOTE:	During the SSC Mode 3  PDU Session anchor change procedure as described in clause 4.3.5.2 of TS 23.502 [3], two PDU Sessions of the same values may exist for a period of time. In this case the UE should select the PDU Session that is not associated with a PDU Session Address Lifetime value.

If none of the existing PDU Sessions matches, the UE tries to establish a new PDU Session using the values specified by the selected Route Selection Descriptor. If the PDU Session Establishment Request is accepted, the UE associates the application to this new PDU Session. If the PDU Session Establishment Request is rejected, based on the rejection cause, the UE selects another combination of values in the currently selected Route Selection Descriptor if any other value for the rejected component in the same Route Selection Description can be used. Otherwise, the UE selects the next Route Selection Descriptor in the order of the Route Selection Descriptor Precedence, if any.

The UE re-evaluates the URSP rules and may change the association of applications to PDU Sessions if the UE experiences certain conditions, for example:

-	periodic URSP rules re-evaluation based on UE implementation;

-	an existing PDU Session that is used for routing traffic of an application based on a URSP rule is released;

-	the URSP is updated by the PCF;

-	the UE moves from EPC to 5GC;

-	UE registers over 3GPP or non-3GPP access;

-	UE establishes connection to a WLAN access.

Details of the conditions are defined by TS 24.501 [14].

If the selected Route Selection Descriptor contains a Non-Seamless Offload indication and the UE has established a connection to a WLAN access, the UE routes the traffic matching the traffic descriptors of the URSP rule via the WLAN access outside of a PDU session.

**** End of Changes ****