3GPP TSG-SA WG2 Meeting #128	S2-186476

2-6 July 2018, Vilnius, Lithuania	(was S2-185388)

Source:	Huawei, HiSilicon

Title:	Architectural assumptions for M.E.A.N.S

Document for:	Approval

Agenda Item:	6.21

Work Item / Release:	FS_ENS/Rel-16

Abstract of the contribution: This contribution is a resubmission of S2-174595/5388, proposing architectural assumptions for Mutually Exclusive Access to Network Slices, a.k.a MEANS.

# 1	Introduction

For an extensive discussion of the reasons of the definition and the architectural assumptions, please see S2-184595 and the report of the discussions during SA2#127.

# 2.	Proposal

Based on the observations above, it is proposed to capture in the TR the following architectural assumptions for our work on mutually exclusive access to network slices.

First change

# 4	Architectural Assumptions and Requirements

For Mutually Exclusive Access to Network Slices the following assumptions apply:

-	It is assumed that the support of mutually exclusive access to network slices uses Rel-15 network slicing feature as the baseline.

-	Support of the Mutually Exclusive Access to Network Slices in a PLMN shall not impact Rel-15 5G UEs' behaviour and UEs that do not need to access mutually exclusive network slices (i.e. mutually exclusive access to network slices shall be an optional feature for the UE).

-	The network operator shall be able to ensure that the UE is prevented to access Network Slices which are mutually exclusive for that UE.

-	It is assumed that the support of mutually exclusive access to network slices in a UE does not prevent operation in a Rel-15 5GC, i.e. a UE, supporting mutually exclusive access to network slices, shall be able to roam in a Rel-15 5GC network without impacting that network.

-	It is assumed that PLMNs are going to deploy their network slices in their networks so that all the services that are not intended to be used in exclusion of other services are available to the users simultaneously (i.e. there will be at least a set of network slice instances in the home network that can offer all the S-NSSAIs that the UE can request simultaneously in its home network).

End of changes