SA WG2 Meeting #128	S2-186525

Vilnius, Lithuania , July 2 - 6, 2018	(was S2-18xxxx)

Source:	Sony

Title:	Solution for Key Issue 5

Document for:	Discussion/Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC

Abstract of the contribution: Solution for Key Issue 5 to utilize Xn for initial ultra low latency user data

# 1	Discussion

Solution to Key Issue 5 on how to achieve low latency when the UE transitions from RRC_Inactive to RRC_Connected State in a NG RAN Node that does not have the UE Context.

This solution allows the user data to be delivered directly to the current DNN, serving a local ultra low latency application server nearby the anchor RAN node before:

- The UE context is transferred from the anchor RAN node to the new RAN node in case of EDT

- The N3 path switch has been performed

- Possible a new N6 path has been setup

- In case of edge computing a new application server nearby the new RAN node may also need to retrieve the user application context data from the previous application server nearby the anchor RAN node.

This solution is similar to:

- Solution 28 “SMF assisted NG-RAN information” in FS_CIoT_5G (23.724 clause 6.28)

- Solution 36 “CM-CONNECTED with RRC Inactive state for frequent small data transmission” in FS_CIoT_5G (23.724 clause 6.36)

# 2. Proposal

The following addition is proposed to TS 23.725 (all new)

************** Start of Changes *****************

## 6.X	Solution for Key Issue 5, supporting low latency for initial data delivery without requiring that the UE to always be in RRC_Connected State

### 6.X.1	Description

This solution applies to key Issue 5 when the UE change state from RRC_Inactive to RRC_Connected in another NG-RAN node than the one that has the UE context, where the nodes are within the same RAN Notification Area, and there are MO or MT data that need to be delivered with ultra low latency or with low jitter.

This solution allows the UE to deliver initial ultra low latency data to the existing DNN via the Xn interface without having the network to reconfigure the N3 and possible also the N6 interface.

CM-CONNECTED with RRC Inactive is used for URLLC User Data with following enhancement:

- The SMF sends URLLC Assistant Information to the RAN to assist the RAN's decision on how to handle the UE when the UE sends RRC_Resume_Request. The URLLC Information may include communication pattern, etc. The SMF may retrieve URLLC Information based on network configuration, user subscription, etc.

- When the UE send RRC_Resume request to a NG-RAN Node that does not have the UE Context then RAN may decide that the anchor NG-RAN Node will not do a path switch of N3 to the new NG-RAN Node immediately.

NOTE: RAN may decide to only deliver the first EDT from the UE and then perform N3 path switch or, temporary setup a data forwarding tunnel of the PDU session over the Xn interface [38.420 & 38.415] or not to switching N3 path to serving NG-RAN to avoid unnecessary N2 signalling if it is limited size of URLLC user data.

### 6.X.2	Procedures

The procedure below shows the ultra low latency solution where the user data is delivered over Xn.

Figure 6.X.2.1: Ultra low latency user data delivery

- Precondition is that the UE successfully done a PDU Session Establishment Procedure according to clause 4.3.2.2.1 in TS.23.502 with the following additions:

  - The SMF includes URLLC Assistant Information in the N2 SM information in step 11 and

  - AMF forwards the N2 SM information to the anchor RAN Node including the URLLC Assistant Information in step 12

Based on the URLLC Assistant Information NG-RAN Node have configured the UE to enter RRC_Inactive state.

- UE to NG-RAN: RRC Resume Request (Resume ID) according to existing procedure.

- The new NG-RAN node uses the Resume ID to identify the anchor NG-RAN node and request to retrieve the UE Context from the anchor NG-RAN Node. In case of EDT the UE context retrieval may also include the early data. Based on URLLC Assistant Information in the N2 SM information, the anchor NG-RAN Node may decide to configure the new NG-RAN Node as a relay for that UE. In that case the new NG-RAN Node will relay future MO & MT user data between the UE and the Anchor RAN Node.

- The new RAN Node sends the RRC Resume message to the UE.

- The UE Enter RRC_Connected state.

- All MO user data will be transferred from the UE to the new NG-RAN Node and then over Xn to the anchor NG-RAN Node and further transported via existing N3 & N6 to the DNN and the low latency application service. MT user data will be sent in the opposite direction.

### 6.X.3	Impacts on Existing Nodes and Functionality

SMF:

-	The SMF includes "URLLC Assistant Information" in the N2 SM information if the SMF determines the PDU session is used for URLLC User Data.

RAN:

-	The RAN decides whether the UE can be sent to RRC Inactive state based on the URLLC Assistant Information received from the SMF.

-	The RAN, based on the "URLLC Assistant Information" received from the SMF, may decide to establish a forwarding tunnel between target NG-RAN and old NG-RAN during the Resume procedure or, RAN may decide to only forward the EDT user data included in RRC_Resume_Request in the UE Context Retrieval request and allow the Anchor NG-RAN Node to deliver the EDT user data over existing N3 without any N3 path switch.

### 6.X.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

************** End of Changes *****************