3GPP TSG-SA WG2 Meeting #128	S2-186718

Vilnius, Lithuania, 02 July - 06 July, 2018	(revision of S2-18XXXX)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0505 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SSC Mode Selection Policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-05-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#127 meeting, it is clarified that the SSC Mode Selection Policy takes priority over applicable user preference in S2-184185. However, in TS 23.501, the SSC Mode Selection Policy is only applied when there is no application request. In order to resolve this confliction, we propose to update TS 23.501 so that SSC Mode Selection Policy shall be used to select SSC mode unless there is no matching SSC Mode Selection Policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that application requested SSC mode is only used when there is no matching SSC Mode Selection Policy for the application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistence specification may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.6.9.3	SSC mode selection

The SSC mode selection policy shall be used to determine the type of session and service continuity mode associated with an application or group of applications for the UE.

It shall be possible for the operator to provision the UE with SSC mode selection policy. This policy includes one or more SSC mode selection policy rules which can be used by the UE to determine the type of SSC mode associated with an application or group of applications. The policy may include a default SSC mode selection policy rule that matches all applications of the UE.

When an application requests data transmission (e.g. opens a network socket), the UE shall determine the SSC mode associated with this application by using the SSC mode selection policy. If there is no SSC mode selection policy, the UE may use application requested SSC mode. In addition, the following behaviour applies for the UE and network:

a)	If the UE has already an established PDU Session that matches the SSC mode associated with the application, then the UE routes the data of the application within this PDU Session unless other conditions in the UE do not permit the use of this PDU Session. Otherwise, the UE requests PDU Session Establishment for a new PDU Session with an SSC mode that matches the SSC mode associated with the application.

b)	The SSC mode associated with the application is either the SSC mode included in a non-default SSCMSP rule that matches the application or the SSC mode included in the default SSC mode selection policy rule, if present. If the SSCMSP does not include a default SSCMP rule, there is no SSCMSP rule matches the application and the application itself does not indicate the required SSC mode, then the UE requests the PDU Session without providing the SSC mode. In this case, the network determines the SSC mode of the PDU Session.

The SSC mode selection policy rules provided to the UE can be updated by the operator.

The SMF receives from the UDM the list of supported SSC modes and the default SSC mode per DNN per S-NSSAI as part of the subscription information.

If a UE provides an SSC mode when requesting a new PDU Session, the SMF selects the SSC mode by either accepting the requested SSC mode or rejecting the PDU Session Establishment Request message with the cause value and the SSC mode(s) allowed to be used back to UE based on subscription and/or local configuration. Based on that cause value and the SSC mode(s) allowed to be used, the UE may re-attempt to request the establishment of that PDU Session with the SSC mode allowed to be used or using another URSP rule.

If a UE does not provide an SSC mode when requesting a new PDU Session, then the SMF selects the default SSC mode for the data network listed in the subscription or applies local configuration to select the SSC mode.

SSC mode 1 shall be assigned to the PDU Session when static IP address/prefix is allocated to the PDU Session based on the static IP address/prefix subscription for the DNN and S-NSSAI. The SMF shall inform the UE of the selected SSC mode for a PDU Session.

SSC mode 3 shall not be assigned to the PDU Session if the Unstructured PDU Session type or Ethernet PDU Session type is selected.

* * * * End of Changes * * * *