SA WG2 Meeting #128	S2-186724

02 July - 06 July, 2018, Vilnius, Lithuania

Source:	Samsung

Title:	A solution to provide the available analytic information metadata

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA / Rel-16

Abstract of the contribution: This contribution proposes a solution to provide the currently available analytic information to 5GS NFs and AFs.

- Discussion

This contribution propose a solution to provide related to the contribute S2-186723.

The main idea of this solution is to provide the metadata of the available analytic information from NWDAF to 5GS by using service based interfaces. By introducing this solution, it expected to bring several benefits, 1) enhance the programmability of 5GS NFs/AFs, 2) improve the reusability of the analytic information by sharing multiple NFs/AFs, 3) encourage the diversity of 5G NFs/AFs by allowing discovery of the available analytic information, 4) may improve vendor-neutrality by allowing the exchange of the analytic information after the deployment.

2.	Proposal

It is proposed to include the added text into the TS 23.791. The detailed issue to solve in this document is described in S2-186723. This contribution proposes solutions for Key Issue 1: Analytic information exposure to 5GS NF and Key Issue 2: Analytic information exposure to AF.

* * * * First Change * * * *

6.x	Solution X: Metadata exposure to 5GS NF and AF

6.x.1	Description

Editor's note:	Describe the solutions. Sub-clause(s) may be added to capture details, procedural flow, etc.

This is a solution to Key Issue #1: Analytic Information Exposure to 5GS NF, and Key Issue #2: Analytic information exposure to AF.

6.x.1.1	Metadata request by 5G NF

This procedure is used by any NF service consumer to obtain the metadata of the available analytic information from the NWDAF such as the list of the available analytics.

Figure 6.x.1.1-1: Metadata request/response by a 5G NF

- The 5G NF service consumer request to obtain the metadata of the available analytic information to NWDAF by invoking the request operation via service-based interface with several parameters to efficiently filter out non-related with the request. For example, the parameters of the request operation may be NF type, analytic information category, NF ID, and etc.

- When the NWDAF successfully received the request from the NF, the metadata containing the analytic information  list should be returned via available analytic information response operation.

- (Optional) If the NF require to get notified when the metadata of the available information is changed, the NWDAF may notify the NF by invoking an analytic information update operation.

6.x.1.2	Available analytic information request by AF

This procedure is used by an AF service consumer to obtain the metadata of the available analytic information from the NWDAF. Not like a case of 5G NF, the request from an AF may be bypassed via NEF to filter out not authorized requests from 3rd party AFs.

Figure 6.x.1.2-1: Metadata request/response by an AF via NEF

- An AF service consumer request to obtain the metadata of the available analytic information to NWDAF by invoking the request operation via service-based interface with several parameters to efficiently filter out non-related with the request. For example, parameters of the request operation may be NF type (AF), analytic information category, AF ID, and etc. The request message may be bypassed by NEF.

- When the NWDAF successfully received the request from the AF, the metadata of the analytic information list should be returned via available analytic information response operation. The response message may be bypassed by NEF.

- (Optional) If the NF require to get notified when the metadata of the available information is changed, the NWDAF may notify the NF by invoke analytic information update operation. The update message may be bypassed by NEF.

6.x.1.3	How 5GS NF/AF request the metadata of the available analytic information

5G NF or AF request the metadata of the available analytics using the Nnwdaf service defined in 6.x.1.1 and 6.x.1.2 that may input several parameters to efficiently filter out highly related analytics only.

| Parameter | Description (examples of patterns) |
| --- | --- |
| NF Type | The requesting NF type to find out a set of the information related with the role of the NF. For example, AMF may request the information related with the mobility. |
| NF ID | NF’s ID to distinguish the NF from others. This parameter can be utilized when an AF requests the set of the analytic information. Using further information, NF ID may help NWDAF to figure out the characteristics of the AF. |
| Information category | Several categories of the analytic information can be defined to categorize type of analytic information, such as load, mobility, session, and etc. |

Editor’s Note: 	The parameters of the request operation should be further studied to improve the usability of the NWDAF by other NFs

6.x.1.4	How NWDAF provides a set of the available analytics

As a response of the metadata for the available analytic information request, the NWDAF should provide the information to allow NF and/or AF can subscribe or request analytics successfully. NWDAF should provide the available analytic information metadata with event ID and available event filters at least. The candidate information contained in the response (or update) can be detailed in 6.1.1.3.

6.x.2	Impacts on Existing Nodes and Functionality

NWDAF: A new Nwdaf service should be provided to allow requests from 5G NF/AF. No impacts on the existing Nnwdaf interface.

Consumer 5G NF/AF: For a NF such as PCF or NSSF that are consumers of new Nnwdaf service, the impact is that need to allow the processing of the response/update message from NWDAF.

NEF: To connect NWDAF and AFs, NEF should allow the newly defined NWDAF service.

6.x.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * * End of change * * * *