SA WG2 Meeting #128	S2-186731

02 July - 06 July, 2018, Vilnius, Lithuania

Source:	Samsung

Title:	A solution to register a new definition of analytic information provided by NWDAF

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA / Rel-16

Abstract of the contribution: This contribution proposes a solution to allow the registration of a new definition of analytic information of the NWDAF by 5GS NF and AF.

- Discussion

This contribution propose a solution to provide related to contribute S2-186726.

The main idea of this solution is to allow the registration of a new definition of analytic information to NWDAF by 5GS NFs and AFs. The registration procedure can occur through 5GS service based interfaces through HTTP/JSON. Using JSON document format, a sophisticated expression or algorithms to derive an analytic information can be presented. For example, JSON document can contains a plain text that express an algorithm to calculate the newly defined analytic information, or it can contain a functions (or object) that can be easily converted into a running object on the NWDAF. In this contribution, the representation of the definition is not limited by a certain method. To resolve the bootstrapping problem to define a new analytic information, the NWADF should provide a default set of the basic analytic information and operations. The new definition should be expressed from the default information and operations. After the successful registration of the new analytic information, 5GS NF or AF can consume the information through the solution described in S2-186724. After 5GS NF recognize the newly available analytic information, the remaining procedure to consume the information described on Section 6.1.

By introducing this solution, it can be expected to bring several benefits, 1) improve the programmability of 5G NFs and AFs, 2) eliminate the dependency on a specific NWDAF implementation, 3) improve vendor-neutrality, and 4) allow to extend the functionality of NWDAF after deployment.

2.	Proposal

It is proposed to include the added text into the TS 23.791. The detailed issue to solve in this document is described in S2-186726. This contribution proposes solutions for Key Issue 1: Analytic information exposure to 5GS NF and Key Issue 2: Analytic information exposure to AF.

* * * * First Change * * * *

6.x	Solution X: available analytic information exposure to 5GS NF and AF

6.x.1	Description

Editor's note:	Describe the solutions. Sub-clause(s) may be added to capture details, procedural flow, etc.

This is a solution to Key Issue #1: Analytic Information Exposure to 5GS NF, and Key Issue #2: Analytic information exposure to AF.

6.x.1.1	A registration of new analytic information by 5G NF

This procedure is used by any NF service consumer to register a new definition of analytic information provided by NWDAF when a necessary information is not currently available to proceed own decision mechanism. The basic assumption of this procedure is that the NWDAF provides a default set of basic analytic information and operations. A 5G NF requests to register a new analytic information can derived from the combination of the default of the information and operations.

Figure 6.x.1.1-1: new analytic information registration by a 5G NF

- The 5G NF service consumer send a registration of a new analytic information via 5G service based interface. The request contains the definition of the new information to describe how to derive the information.

- NWDAF may response to the 5G NF with the result of the registration operation.

6.x.1.2	A registration of new analytic information by

This procedure is used by any NF service consumer to register a new definition of analytic information provided by NWDAF when a necessary information is not currently available to proceed own decision mechanism. The basic assumption of this procedure is that the NWDAF provides a default set of basic analytic information and operations. A 5G NF requests to register a new analytic information can derived from the combination of the default of the information and operations. Not like a case of 5G NF, the request from an AF may be bypassed via NEF to filter out not authorized requests from 3rd party AFs.

Figure 6.x.1.2-1: new analytic information registration by an AF via NEF

- An AF service consumer send a registration of a new analytic information via 5G service based interface. The request contains the definition of the new information to describe how to derive the information. The request message may be bypassed by NEF.

- NWDAF may response to the 5G NF with the result of the registration operation. The response message may be bypassed by NEF.

6.x.1.3	How 5GS NF/AF request the available analytic information

5G NF or AF may request to register a new analytic information with several parameters. At least, the parameters have to be enough to express how to derive the newly defined information computationally by the NWDAF. The following table contain the example parameters.

| Parameter | Description (examples of patterns) |
| --- | --- |
| Name | The name of the newly defined analytic information. |
| Description | A descriptive text of the newly defined analytic information. This information may help human operator or developer to understand about the information. |
| Algorithm | A representation of the derivation method of the newly defined |
| Filters | A set of filter to help 5G NF or AF to efficiently consume the information. |
| Information category | A meta data used to categorize the information for convenience. Multiple category can be declared. |

NOTE: 	The parameters of the registration operation should be further studied to improve the usability of the NWDAF by other NF and AF.

6.x.1.4	How NWDAF provides a set of the available analytics

As a response of the registration operation, the NWDAF should provide the confirmation response whether the operation was successful or not. If successfully registered, then NWDAF should response with the event ID. If not, NWDAF may provide the reason of the failure.

6.x.2	Impacts on Existing Nodes and Functionality

NWDAF: A new Nwdaf service should be provided to allow registration operations from 5G NF/AF. No impacts on the existing Nnwdaf interface.

Consumer 5G NF/AF: For a NF such as PCF or NSSF that are consumers of new Nnwdaf service, the impact is that need to allow the processing of the response message from NWDAF. If the consumer need to register new analytic information, then it need to present the definition of the information.

NEF: To connect NWDAF and AFs, NEF should allow the newly defined NWDAF service.

6.x.3	Solution Evaluation

Editor's note:	Use this section for evaluation at solution level.

* * * * End of change * * * *