SA WG2 Meeting #128	S2-187237

2-6 July 2018, Vilnius                                                 (was S2-187202 was S2-187154)

Source:	Thales

Title:	Reference Satellite Integration Architectures for FS_5GSAT_ARCH TR

Document for:	Approval

Agenda Item:	6.24

Work Item / Release:	FS_5GSAT_ARCH/Rel-16

Abstract of the contribution: This contribution proposes an Annex introducing some architecture models for  satellite integration in 5GS to be included in TR FS_5GSAT_ARCH.

# Proposal

This contribution proposes an Annex introducing some  architecture models for satellite integration in 5GS  for TR FS_5GSAT_ARCH.

* * * Start of changes * * * *

# Annex B (Informative): Architecture Models for Satellite Integration in 5GS

## B.1	Elementary satellite network architecture

The physical implementation of a satellite network architecture can always be sketched as displayed in the following figure:

Figure B-1: Reference Satellite Access Network Architecture

A satellite network is always composed of:

- One, or more, satellite(s), embarking a telecommunication payload

- Satellite enable UEs, such as IoT devices, broadband vehicular or fixed terminals,

- A Hub, including a RF Earth station, and a data centre including (co-located or not) RAN and CN related functions. The hub is also interconnected to (a) data network(s).

- UEs interconnected to the hub through the satellite.

## B.2	Reference architecture with satellite enabled NR-RAN

The following figure depicts a reference architecture for a direct access with a satellite NR-RAN (see below):

Figure B-2: 5GS with Satellite enabled NR-RAN

Knowing the possible nature of satellite payloads (transparent or re-generative), the instantiation of this architecture can have several forms:

In a first implementation, the satellite is transparent, the NR signals are generated from gNB’s that are located on ground. The satellite is equivalent to a Radio Frequency (RF) Remote Unit, and is full transparent to the New Radio protocols, including the physical layer (see below).

Figure B-3: 5GS with transparent satellite enabled NR-RAN

In a second implementation, the satellite is regenerative. The satellite payload implements a gNB-DU from the point of view of the NR-RAN functionalities. Some of the protocols of the NR are processed by the satellite. A Satellite Radio Interface (SRI) transports the F1 protocol between the on-ground CU and the on-board DU (it can be the NR (see below).

Figure B-4: 5GS with regenerative satellite enabled NR-RAN and distributed gNB

In a third implementation, the satellite is regenerative. The satellite payload implements a full gNB. A Satellite Radio Interface (SRI) transports the N1/N2/N3 interfaces between the on-ground 5G CN and the on-board gNB-CU (see below).

Figure B-5: 5GS with regenerative satellite enabled NR-RAN and on-board gNB

One can note that with a regenerative implementation (either with distributed gNB or not) a 5GS can have a global coverage, providing a single 5G CN with global or regional (continental or sub-continental) coverage as well. This is valid for GEO or LEO (see below):

Figure B-6: 5GS with regenerative satellite enabled NR-RAN, with ISL for regional or global coverage

Another configuration (see below) may consist in satellites each embarking a gNB, and 5G CN’s accessing these gNB’s, as described in the following figure, where specific (or not) satellite radio interfaces are used to handle control and user planes:

Figure B-7: 5GS with regenerative satellite enabled NR-RAN, with ISL & multiple 5G CN connectivity

* * * End of changes * * * *