SA WG2 Meeting #128	S2-187519

2 – 6 July, Vilnius, Lithuania 	(revision of S2-187445)

Source:	Ericsson

Title:	Solution proposal “Flexible Service Framework” to address key issue “Improvements to service framework related aspects”

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes a solution that addresses key issue "Improvements to service framework related aspects”

# 1 Proposal

This document proposes to add following solution addressing key issue “Improvements to service framework related aspects” to TR 23.742.

* * * Start of Change (ALL TEXT IS NEW) * * *

## 6.x Solution #X: Flexible Service Framework Deployment

### 6.x.1 Introduction

This solution addresses key issues 3 “Improvements to service framework related aspects” and key issue 5 “SBA backward and forward compatibility”.

### 6.x.2 High level description

In rel. 15 the service framework consists of the functional components illustrated by the figure 6.x.2-1 below.

Figure 6.x.2-1 Rel 15 service framework

The NRF handles the service registry and the consumer authorization. The NF may consist of one or several NF Service consumers and NF Service producers. An NF service consumer consists of the service business logic, service discovery and service instance selection. An NF producer consists of service business logic, service registration and optionally load balancing. If there is a Load balancer is dependent on implementation. NF service consumers communicates directly with NF service producers. The ovals in figure 6.x.2-1 can be seen as components of the service framework.

NOTE: An NF service may include both the consumer and producer roles in the same NF service. They are discussed and shown separately in figures in order to clarify the functionality needed for the 2 different roles.

In a new service framework, these types of components will still need to exist in one way or the other. The solution in 6.3.1 introduces a service framework where all of these components are moved to a logically centralized service framework. This does not mean that in deployment time that the service framework is centralized. For simplicity reasons in this clause 6.x centralized framework mean that it is logically centralized.

To enable co-existence of existing Re. 15 service and the possibility to choose at network design which type of service frame work is wanted by an operator, the following model is proposed as shown in figure 6.x.2-2 below.

Figure 6.x.2-2 Flexible service framework

The NF/NF services in can be kept as Rel-15. If a centralized service framework is selected in the network design (deployment), the NRF will only provide the address(es) of centralized service framework to the NF service consumers, and thus indirect communication will be used. This means the service instance selection in the consumer will in principle be a NULL function (there is only one service instance to select). If a centralized service framework is not selectedin the network design (in deployment), then rel. 15 procedures will be used, and service instance selection is done in the service consumer.

Standardized consumers always do discovery of a service producer. The reason for this is backward compatibility and the possibility to support both services frameworks. The discovery procedure is done seldom, and the basic part can be implemented in a few lines of code, and depending on implementation architecture used by a vendor, it may be a common SW entity that is re-used by all the vendor’s consumer implementations. However, in rel. 15 there are some special behaviour for discovery of for example UDM where MNC and MCC are important parameters to consider, and thus it may not be so that all consumers can have just a generic discovery procedure.

Standard consumer always has instance selection, however, if a centralize service framework is wanted, only address(es) the service framework’s message passing function is used. This means that the contact point (FQDN or IP addresses will be to the message passing system, and that service producer resources are addressed via this contact point.

One should observe here that instance selection is not always generic. For example, when AMF selects an SMF instance the following should be considered:

-	Selected Data Network Name (DNN).

-	S-NSSAI.

-	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed

-	per S-NSSAI: the subscribed DNN(s)

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed

-	Local operator policies.

-	Load conditions of the candidate SMFs.

-	Access technology being used by the UE

This is different from when AMF selects another AMF then the following shall be considered:

-	AMF Region ID and AMF Set ID derived from GUAMI.

-	Requested NSSAI.

-	Local operator policies.

-	Availability of candidate AMFs.

-	Load balancing across candidate AMFs (e.g. considering weight factors of candidate AMFs in the AMF Set).

As can be seen some of the factors to consider for AMF when selecting SMF or AMF could be generalized, but some are unique per selection. If a centralized service framework is deployed, then also these unique factors must be considered within the centralized service framework, if existing functionality shall be retained.

.

### 6.x.3 Services and illustrated Procedures

Editor's note: This clause describes services and related high-level procedures for the solution.

#### 6.x.3.1 Registration and de-registration of NF instances/NF service instances

This section describes the high-level procedures for the registration of NF instances/NF service instances at the service framework. In general, the procedures for the registration of NF/NF service (instances) are unchanged compared to R15.

Registration of NF instances/NF service instances

- the NF instances/NF service instance uses the Rel. 15 mechanism to register it self

De-Registration of NF instances/NF service instances

- the NF instances/NF service instance uses the Rel. 15 mechanism to deregister itself

#### 6.x.3.2 Discovery of NF services and communication between consumer and producer

A legacy consumer uses the Rel. 15 mechanism to discover producers. New standardized consumers services always do service discovery. See 6.x.2. For examples of consumer to producer communication see below figures 6.x.3.2-1 and 6.x.3.2-2 for non-roaming and 6.x.3.2-3 and 6.x.3.2-4 for roaming

Figure 6.x.3.2-1 Consumer to producer communication, no centralized service framework

Figure 6.x.3.2-1 is the rel. 15 behaviour, and will be used if no centralized service framework is wanted.

Figure 6.x.3.2-2 Consumer to producer communication, with centralized service framework

In figure 6.x.3.2-2 represent an example of a consumer x instance #1 that wants to communicate with the producer y. The consumer does discovery and gets pseudo network address of the NF/NF service which will be the network address (IP addresses or FQDN) of the message passing function within the centralised service framework, and if needed corresponding optional discovery result. This is illustrated in the figure by pseudo producer y#1. The consumer x instance #1 can only select pseudo producer y#1 and sends the request to that address (which is the address of the message passing function). The message passing function will select a producer instance to be used for this operation, producer y #2 in this example. The message passing system may have several IP addresses for redundancy reasons.

The following figures shows roaming examples of how a logically centralised service framework deployed the visited network interacts with a Rel-15 based Service framework in the home network and vice versa.

Figure 6.x.3.2-3 Consumer to producer communication over roaming interface, centralized service framework in VPLMN

Figure 6.x.3.2-4 Consumer to producer communication over roaming interface, centralized service framework in HPLMN

The SEPP in figures 6.x.3.2-3 and 6.x.3.2-4 may of course do topology hiding. These figures illustrated the principles.

#### 6.x.3.3 Non-standardized services

Non-standardized services could be implemented targeting a specific network design model. These non-standard services may utilize a centralized service framework by for example to not do service discovery or instance selection. They may interact with the centralized service framework in a different way compared to the standardized services.

### 6.x.4 Impacts on existing NF/NF Services and Interfaces

Editor's note: This clause describes impacts to existing services and interfaces.

No impact on existing NF/NF services and interfaces

### 6.x.5 Evaluation of the Solution

Editor's note: This clause provides an evaluation of the solution.

End of change (all new text)