SA WG2 Meeting #129bis	S2-1812064

November 26 – 30, 2018, West Palm Beach

Source:	CableLabs, Charter Communications

Title:	Slicing for FN-CRG

Document for:	Discussion and approval

Agenda Item:	6.7

Work Item / Release:	FS_5WWC / Rel-16

Abstract of the contribution: This contribution proposes slicing support for FN-CRG.

# Introduction

This contribution proposes slicing support for FN-CRG.

# 2	Proposal

It is proposed to adopt the following proposals and the related changes to TR 23.716.

BEGIN 1st CHANGES

## 6.25	Solution #25: Supporting FN-CRG in 5GC

### 6.25.1	Overview

This solution specifies the 5GC functionality required to support registration and session management of FN-CRG, when interworking functionality defined by CableLabs is provided in the W-5CGAN.

In essence, the solution is based on the W-5GCAN authenticating the FN-CRG, providing registration and session management for / on behalf of the FN-CRG. The 5GC impacts are thus limited; the adjustments that are still needed are described in this solution.

The solution is applicable to one scenario, the interworking functions are used in the W-5CGAN as the GW to the 5GC.

### 6.25.2	Registration and PDU management for FN-CRG

#### 6.25.2.1 Initial registration and session setup for FN-CRG

Initial registration and session setup procedures for the FN-CRG follow the same structure as for the FN-RG as described in section 6.23.2.1, except for the following DOCSIS aspects:

•	The W-5CGAN provides registration and session set up interworking rather than the FAGF

•	A DOCSIS CM identifier defined by CableLabs is used rather than a Line ID. SUPI is derived from this identifier. New SUPI format is expected.

•	The serving W-5GCAN identity is stored in the UDM as part of the Nudm_UECM registration procedure (step 6 in figure 6.23.2-2).

•	Existing DOCSIS authentication methods apply - between FN-CRG and W-5GCAN (i.e., outside 3GPP)

•	Resource reservations between FN-CRG and W-5GCAN, when applicable, are defined by DOCSIS (i.e., outside 3GPP)

#### 6.25.2.2 Registration management for FN-CRG

The W-5CGAN shall provide re-registration for the FN-CRG, as requested by the 5GC.

The 5GC may explicitly de-register the FN-CRG, e.g. in case of termination of the subscription.

#### 6.25.2.3 PDU session management for FN-CRG

The 5GC may modify the PDU session, e.g. due to the change in QoS subscription information.

#### 6.25.2.x Support of slicing for FN-CRG

There is no new requirement on the 5GC to support slicing with FN-CRG. The general concept works as follows: The W-5GCAN is provisioned with a slice ID. The UDM links the slice ID with the HFC_Identifier. 5GCAN can provision a PDU session with that slice ID.

The W-5GCAN supports a PDU session with the slice ID on the AN. The mapping of the AN resource to the slice ID is defined by CableLabs.

END of CHANGE