SA WG2 Meeting #129Bis	S2-1812087

# 26 - 30 November, 2018, West Palm Beach, USA	(revision of S2-18xxxx)

Source:	Huawei, HiSilicon

Title:	        update to Solution#3 and Solution#4

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC / Rel-16

Abstract of the contribution: This contribution proposes to update solution#3 and solution#4 to resolve the ENs.

# 1 Discussion

The following contribution discusses the editor’s notes added for solution #3 and solution #4 in last meeting, and updates solution #3 and #4 according to the discussion.

1. The editor’s notes to be resolved in solution #3:

- Editor's note: It is FFS whether and how to coordinate with other redundancy mechanisms if such mechanisms were used in DN.

In IEEE802.1 CB, it supports flexible positioning of the FRER function. As show in Figure 1, End System A is transmitting a Stream, but has no FRER functions. Relay system B transforms the TSN stream into Relay system C and D by sequencing the packets and splitting them into 2 Member Streams 26 and 31.

Figure 1 Frame Replication and Elimination for Reliability flexible positioning

In Solution #3 or #4, if IEEE 802.1CB was deployed in the DN, a relay system can be deployed close to or co-located with the UPF to terminate IEEE FRER as defined in above figure.

Figure 2 depicts the transmission for TSN Stream. For the DL, the relay system combines the two Member Streams received from different Bridges over N6, and obtains a single copy of each packet of the TSN Stream. Then the UPF replicates each packet as described in solution #3 or #4. For the UL, the relay system will transforms the packet received from 5GS into two Member Streams and sends to different paths in TSN domain.

Figure 2 Coordination with redundancy mechanism used in DN

Proposal 1: If IEEE FRER are used in DN, a TSN relay system can be deployed close to or co-located with the UPF to terminate TSN FRER as defined in IEEE 802.1CB.

- Editor's note: It is FFS how the UE becomes aware which flows redundancy should be applied to.

- Editor's note: It is FFS how the limitation of one QoS flow per DRB is known and enforced in RAN. It is FFS whether the limitation of one QoS flow per DRB imply also a limitation on the number of QoS flows, and if so, how is that limitation known in the core network?

The two ENs both related to how the UE/RAN knows the redundancy transmission described in solution #3 is applied to the QoS flow.

It’s proposed to let the SMF to include a “CN redundancy indicator” in the N2 SM information in PDU session establishment or modification procedure. For option 1 case, RAN then informs UE about the CN redundant transmission during RRC reconfiguration procedure. For option 2 (HRP protocol) case, the UE is informed by the CN redundancy indicator included in the NAS messages (PDU Session Establishment/Modification Accept).

With this, both UE and RAN know the solution is applied to the QoS Flow, then UE and RAN can apply the mechanisms described in the solution accordingly.

Proposal 2: A “CN redundancy indicator” is included in the N2 SM information for the QoS Flow during PDU session establishment or modification procedure. For option 1 case, RAN then informs UE about the CN redundant transmission during RRC reconfiguration procedure. For option 2 (HRP protocol) case, the UE is informed by the CN redundancy indicator included in the NAS messages (PDU Session Establishment/Modification Accept). With the indication, UE and RAN shall handle the QoS Flow as described in clause 6.3.1a.

2. The editor’s notes to be resolved in solution #4

5) Editor's note: It is FFS which tunnel is applied for a flow that does not need redundant handling.

If there are some flows that does not need redundant transmission, it is proposed to use a default N3 tunnel for these flows. The default tunnel is decided during the tunnel establishment

Proposal 3: One of the N3 Tunnel is marked as default tunnel during the tunnel establishment. For those QoS Flows of the same PDU session that don’t need redundant transmission, the default N3 Tunnel is used.

.

6) Editor's note: It is FFS how the RAN node capability to support the solution is indicated to the CN.

Supporting of solution 4# is a capability of RAN node, hence it’s possible to perform capability negotiation between RAN and CN as usual. However, in most deployments, supporting of URLLC should be homogeneous in a specific area or network slice. For the sake of simplification, it proposed to pre-configure the capability to support the solution in SMF per SMF service area or network slice.

Proposal 4: The RAN node capability to support the solution can be configured in the SMF per network slice or per SMF service area.

# 2 Proposal

It is proposed to make the following changes to the TR 23.725.

* * * * Start of Changes * * * *

## 6.3	Solution #3 for Key Issue #1: Supporting redundant data transmission via single UPF and two RAN nodes

### 6.3.1	Description

This solution addresses the KI#1 of How to "Supporting high reliability by redundant transmission in user plane".

This solution focuses on how to realize the reliability of user plane between UPF and UE. For control plane network functions, the existing network device failure recovery mechanisms, e.g. N+1 resilience, should be enough to fulfil the high reliability requirements on control plane. Though the failover may cause a very short interrupt (typically on xx ms level), but this control plane failover will not impact the traffic routing on user plane.

Typically telecom-level equipments are able to provide up to five-nine reliability. However, in many cases the end-to-end reliability of real deployed network might be lower than that considering the susceptible deployment environments, especially in the last-mile.

To realize ultra-high reliability for URLLC services in the commercial deployed network, this solution proposes to use redundant transmission over different physical transport paths in CN and RAN to enhance the reliability of upper layer service.

Whether redundant transmission need to be activated for a QoS Flow can be decided by the SMF based on local policies for the DNN and/or S-NSSAI, or by the PCF based on its QoS requirement, UE's subscription and condition of network deployment.

In this solution, it is assumed that the reliability of UPF and CP NFs are high enough to fulfill the reliability requirement of URLLC services served by these NFs, and the UE is under overlapping coverage of the two RAN nodes. The reliabilities of these NFs can be realized based on implementation (e.g. redundant mechanisms provided by NFV platform), which is out of scope of this solution. However a single RAN node cannot provide enough reliability in air interface for the QoS flow, so the redundant packets will be transferred between UPF and UE via two independent N3 tunnel and two RAN nodes to enhance the reliability of service.

To ensure the two N3 tunnels can be transferred via disjointed transport layer paths, the M-RAN (Master RAN) node, SMF or UPF may provide different routing information in the tunnel information (e.g. different IP addresses or different Network Instances), and these routing information will be mapped to disjoint transport layer paths according to network deployment configuration.

The redundant transmission is based on the DC architecture, except that same PDU is transferred via both M-RAN and S-RAN (Secondary RAN) nodes. With two different NG-RAN nodes, separate transport layer paths are used for redundant data transmission in user plane.

Figure 6.3.1-1: Redundant transmission with two N3 tunnels between a UPF and two NG-RAN nodes

Figure 6.3.1-2: Redundant transmission with two N3 and N9 tunnels between a UPF and two NG-RAN nodes

In deployments where the M-RAN/S-RAN needs to connect to the anchor UPF via intermediate UPF nodes, one or two Intermediate UPFs (I-UPFs) may be inserted between the anchor UPF and the M-RAN/S-RAN nodes separately to support the redundant transmission with one or two N3 and N9 tunnels, as shown in figure 6.3.1-2.

NOTE 1:	The I-UPFs on the leg of the redundant paths shall not behave in an UL CL or Branching Point role anymore.

For DL traffic, without changing sequence number in GTP-U packet, the two I-UPFs receive the traffic duplicated by the anchor UPF via N9 tunnels and without any further duplication forward them to the M-RAN and S-RAN via N3 tunnels separately. In case of UL traffic, by keeping sequence number in GTP-U packet unchanged, the two I-UPFs without any further duplication forward the traffic received from the M-RAN and S-RAN via N3 tunnels to the anchor UPF via N9 tunnels separately.

Figure 6.3.1-3: Redundant transmission with two NG-RAN nodes in HR roaming scenario

As shown in figure 6.3.1-3, in home routed roaming scenario, the M-RAN/S-RAN nodes are connected to the anchor UPF in the HPLMN via two intermediate UPF nodes in the VPLMN separately to support redundant transmission with two N3 and N9 tunnels. The V-SMF selects the two intermediate UPFs and setup the redundant transmission tunnels on N3 and N9 in case it receive two CN tunnel info for a URLLC QoS flow from H-SMF.

NOTE 2:	Supporting the above HR redundant transmission requests two disjointed transport paths exist between the two I-UPFs and PSA UPF.

NOTE 3:	If FRER are used in DN, a TSN relay system can be deployed close to or co-located with the PSA UPF to terminate FRER as defined in IEEE 802.1CB[6].

### 6.3.1a	User Plane Protocol Stack options

6.3.1a.1	Option 1: Enhancement of PDCP and GTP-U protocols

In this option, PDCP protocol and GTP-U protocol are enhanced to support packet replication and elimination function.

Figure 6.3.1a.1-1: User Plane Protocol Stack - Option1

In this option, it is required there is only one QoS Flow per DRB.

In this option, for DL traffic, the UPF replicates the packet received from the DN and includes the same sequence number in GTP-U headers of the replicates for the redundant transmission. When the M-RAN and S-RAN receive the packets from UPF, they set the DL PDCP sequence number of the PDCP packet according to the sequence number in the GTP-U headers received from UPF. With that, the two PDCP packets that UE received from M-RAN and S-RAN carries same PDCP sequence number, so that the UE can eliminate the duplicated packets at PDCP layer based on the PDCP sequence number.

NOTE: The detail of how to map SNs between GTP-U and PDCP packets can be decided in stage 3. Each QoS Flow have an individual GTP-U SN space. One possible way is to reuse GTP-U extension header "PDCP PDU number" together with QFI to directly carry the PDCP sequence number per QoS Flow in the GTP-U packet.

For UL traffic, the UE replicates the packet and assigns the same PDCP sequence number to them for the redundant transmission. One of these packets is transmitted to the M-RAN, and the other to the S-RAN. The sequence number in GTP-U header for the packet sent to the UPF is assigned by the M-RAN and S-RAN based on the PDCP sequence number received from UE. The UPF eliminates the duplicated packets at GTP-U layer based on the sequence number in GTP-U header.

6.3.1a.2	Option2: HRP protocol layer

In this option, a HRP (High Reliability Protocol) layer is implemented on the UE and UPF to support packet replication and elimination function.

Figure 6.3.1a.2-1: User Plane Protocol Stack - Option2

In this option, for DL traffic, the UPF replicates the DL packet and assigns the same HRP sequence number to them for the redundant transmission. When the UE receives the replicates from M-RAN and S-RAN, it eliminates the duplicated packets at HRP layer based on the HRP sequence number.

For UL traffic, the UE replicates the packet and assigns the same HRP sequence number to them for the redundant transmission. The UPF eliminates the duplicated packets at HRP layer based on the HRP sequence number.

NOTE: In this option, the HRP layer may cause additional delay for internal handlings within UE and UPF.

### 6.3.2	Procedures

#### 6.3.2.1	Activate Redundant Transmission during PDU Session Establishment procedure

The redundant transmission will be established during a PDU Session establishment procedure or a PDU Session Modification procedure.

Figure 6.3.2.1-1 depicts the establishment of redundant transmission during the PDU session establishment procedure in the non-roaming and roaming with local breakout cases.

Figure 6.3.2.1-1: Activate Redundant Transmission during PDU Session Establishment procedure

1.	Steps 1-12 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

In step 7b, the PCF indicates the SMF and the NG-RAN to activate redundant transmission for the QoS flow by the 5QI if the UE requested PDU Session Establishment procedure is initiated for a URLLC service.

In step 10, the SMF then initiates an N4 Session Establishment procedure with the selected UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

In step 11, the N2 SM information includes the QFI(s), QoS Profile(s), CN redundancy indicator, CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel. For option 2 (HRP protocol) case, the CN redundancy indicator is included in the PDU Session Establishment Accept as well.

The CN redundancy indicators are used to indicate that redundant transmission described in the solution is applied to the QoS Flow, so that UE and RAN shall handle the QoS Flow as described in clause 6.3.1a.

2.	RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The Secondary NG-RAN node Addition procedure is performed as defined in TS 37.340 [7]. For option 1 case, RAN informs UE about the CN redundant transmission during the procedure, so that UE shall handle the QoS Flow as described in clause 6.3.1a.

3.	Steps 14-20 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. Each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint. The M-RAN node assigns the appropriate CN tunnel information to each of the NG-RAN nodes.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

During the course of PDU Session Establishment procedure, in case N2 PDU Session Request message contains one CN Tunnel Info, based on radio resource, wireless conditions or local policy, if the NG-RAN may determine to send N2 PDU Session Failure Response to the SMF via the AMF with the cause indicating redundant transmission establishment. Considering it, the SMF may initiates redundant transmission with two CN Tunnel Info, or the SMF may report subscribed event to the PCF, it is up to the PCF to determine whether redundant transmission needs to be activated.

NOTE: The above mechanism may apply to PDU Session Modification and mobility procedure.

Editor's note:	whether the NG-RAN can trigger the 5GC to initiate redundant transmission is FFS.

#### 6.3.2.2	Activate Redundant Transmission during PDU Session Modification procedure

Figure 6.3.2.2-1 depicts the establishment of redundant transmission during the UE or network requested PDU session modification procedure in the non-roaming and roaming with local breakout cases.

Figure 6.3.2.2-1: Activate Redundant Transmission during PDU Session Modification procedure

1.	Steps 1-4 from clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The PDU Session Modification procedure is triggered by the UE or the SMF to add a QoS flow for a URLLC service.

In step 1b or step 2, the PCF indicates the SMF to activate redundant transmission for the QoS flow by the 5QI.

The SMF initiates an N4 Session modification procedure with the UPF and a new CN Tunnel for redundant transmission may be allocated by the SMF or UPF.

In step 4, the N2 SM information includes the QFI(s), QoS Profile(s) for each N3 tunnel, CN redundancy indicator, and the CN Tunnel Info corresponding to the new N3 tunnel. For option 2 (HRP protocol) case, the CN redundancy indicator is included in the PDU Session Modification Accept as well.

The CN redundancy indicator is used to indicate redundant transmission described in the solution is applied to the QoS Flow.

2.	RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The Secondary NG-RAN node Addition procedure can be performed as defined in TS 37.340 [7]. For option 1 case, RAN informs UE that the CN redundant transmission is used during the procedure.

3.	Steps 6-12 from clause 4.3.3.2 (UE or network requested PDU Session Modification) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info for the redundant transmission is provided by the NG-RAN. The AN Tunnel Info includes a tunnel endpoint for the NG-RAN node used for redundant transmission, and the QFIs assigned to the tunnel endpoint.

The SMF provides AN Tunnel Info to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

#### 6.3.2.3	Handover procedure

In this solution, the UE can connect to two NG-RAN nodes simultaneously when redundant transmission is used. Either the M-RAN node or the S-RAN node may perform handover at a time.

In the case that the S-RAN node handover, the M-RAN Node will initiates a Secondary Node Change procedure as defined in TS 37.340 [7]. The transmission between UE and UPF via M-RAN node will not be impacted during the handover procedure.

In the case that the M-RAN node handover, the Master Node handover without Secondary Node change procedure will be applied as defined in TS 37.340 [7]. The transmission between UE and UPF via S-RAN node will not be impacted during the handover procedure.

In the case that both M-RAN node and S-RAN node handover, it follows the procedures defined in TS 37.340 [7].

### 6.3.3	Impacts on Existing Nodes and Functionality

PCF:

-	The PCF can determine whether redundant transmission needs to be activated for a QoS Flow based on its QoS requirement, UE's subscription and condition of network deployment.

AMF:

-	No impact in this solution.

SMF:

-	The SMF can determine whether redundant transmission needs to be activated for a QoS Flow based on local policies for the DNN and/or S-NSSAI and inform that to the RAN and UE (for option 2).

-	In case the SMF allocates CN Tunnel Info, it shall provide the CN Tunnel Info for two tunnels of the redundant transmission paths.

-	The SMF shall indicate the UPF to replicate the downlink packet and send the duplicate packets to the two N3 tunnels, and indicate the UPF to eliminate the duplicate uplink packets.

UPF:

-	In case the UPF allocates CN Tunnel Info, it shall provide the CN Tunnel Info for two tunnels of the redundant transmission paths.

-	The UPF shall be able to replicate the downlink packet and send the duplicate packets to the two N3 tunnels, and eliminate the duplicate uplink packets, based on either sequence numbering in GTP-U header (option 1) or a new protocol (option 2).

RAN:

-	The RAN node shall support redundant transmission via DC architecture with two N3 tunnels.

-	In case protocol stack option 1 is adopted, RAN need to ensure there is only one QoS Flow per DRB, and maps or reuses PDCP SN and sequence number in GTP-U header, and informs UE that the CN redundancy transmission is used during RRC reconfiguration procedure.

-	RAN needs to be able to combine dual connectivity setup with the establishment or modification of radio bearers within the session management procedures.

UE:

-	The UE shall be able to replicate the uplink packets and eliminate the downlink packets for a URLLC service.

-	The UE can determine whether to replicate the uplink packets for a QoS Flow based on its QoS profile.

-	In case of option 1, UE needs to support a new variant of PDCP replication and elimination compared to the existing RAN PDCP duplication mechanism.

-	In case of option 2, UE needs to support a new protocol for replication and elimination.

### 6.3.4	Solution Evaluation

This solution provides high reliability transporting mechanism by performing redundant transmission between Anchor UPF and UE via disjointed CN tunnels and two different RAN nodes. The solution has the following properties:

-	The solution provides the same level of redundancy on the air interface as the existing PDCP packet duplication function in Rel-15 (see TS 38.300 [11] clause 16.1.3).

-	The solution avoids the MgNB being a single point of failure in the user plane.

-	UPF is a single point of failure in the user plane. In case of UPF failure, the session is completely lost.

-	The operator can control the activation of redundant transmission or not by PCC mechanism.

-	This solution has no dependency on the protocol used in application layer, which is out of control of the operator.

-	The redundant transmission can be activated on demand per QoS Flow but limits to single QoS Flow per DRB.

-	Current existing PDU session/QoS Flow management mechanisms are reused in this solution with limited extensions shown in 6.3.3. No further impact on existing CN control plane mechanisms, however RAN is impacted by the need to be able to combine dual connectivity setup with the establishment or modification of radio bearers within the session management procedures.

-	If protocol stack option 1 is adopted, there is no additional replication protocol defined for the UE. If protocol stack option 2 is adopted, there is no additional replication protocol functionality for the RAN node. But interactions with existing Rel-16 RAN PDCP duplication is not investigated yet.

## 6.4	Solution #4 for Key Issue #1: Supporting redundant data transmission via single UPF and single RAN node

### 6.4.1	Description

This solution addresses the KI#1 of How to "Supporting high reliability by redundant transmission in user plane". It focuses on the redundant transmission between RAN and UPF.

In this solution, it is assumed that the reliability of NG-RAN node, UPF and CP NFs are high enough to fulfil the reliability requirement of URLLC services served by these NFs. The reliabilities of these NFs can be realized based on implementation (e.g. redundant mechanisms provided by NFV platform), which is out of scope of this solution. However, the reliability of single N3 tunnel is considered not high enough, e.g. due to the deployment environment of backhaul network, so the redundant packets will be transferred between UPF and RAN via two independent N3 tunnels, which are associated with a single PDU Session, over different transport layer path to enhance the reliability of service.

To ensure the two N3 tunnels can be transferred via disjointed transport layer paths, the NG-RAN node, SMF or UPF will provide different routing information in the tunnel information (e.g. different IP addresses or different Network Instances), and these routing information will be mapped to disjoint transport layer paths according to network deployment configuration.

This solution supports the redundant transmission based on two N3 tunnels between a single NG-RAN mode and the UPF. The RAN node and UPF shall support the packet replication and elimination function.

Figure 6.4.1-1: Redundant transmission with two N3 tunnels between the UPF and a single NG-RAN node

Packet replication and elimination can be realized by modifying the GTP-U protocol. In case of DL traffic, the UPF replicates the packet from the DN and assigns the same GTP-U sequence number to them for the redundant transmission. These packets are transmitted to the NG-RAN via the N3 Tunnel 1 and the N3 Tunnel 2 separately. In order to eliminate the duplicated packet, the NG-RAN forwards the packet received first from either tunnel to the UE and drops the replicated packet which has the same GTP-U sequence number as the forwarded packet.

In case of UL traffic, the NG-RAN replicates the packet and assigns the same GTP-U sequence number to them and the UPF eliminates the duplicated packet based on the GTP-U sequence number.

One of the N3 Tunnel is marked as default tunnel during the tunnel establishment. For those QoS Flows of the same PDU session that don’t need redundant transmission, the default N3 Tunnel is used.

NOTE 1: 	The RAN node capability to support the solution can be configured in the SMF per network slice or per SMF service area.

The solution can be extended by inserting two Intermediate UPFs (I-UPFs) between the UPF acting as the PDU Session Anchor and the NG-RAN to support the redundant transmission based on two N3 and N9 tunnels between a single NG-RAN mode and the UPF. The RAN node and UPF shall support the packet replication and elimination function.

Figure 6.4.1-2 Two N3 and N9 tunnels between NG-RAN and UPF for redundant transmission

In figure 6.4.1.2, there are two N3 and N9 tunnels between NG-RAN and UPF for redundant transmission. The UPF interfacing the DN and acting as the Traffic Distributor for DL traffic duplicates the packet of the URLLC service from the DN and assigns the same GTP-U sequence number to them. These duplicated packets are transmitted to I-UPF1 and I-UPF2 via N9 Tunnel 1 and N9 Tunnel 2 separately. Each I-UPF forwards the packet with the same GTP-U sequence number which receives from the UPF to NG-RAN via N3 Tunnel 1 and N3 Tunnel 2 respectively. The NG-RAN eliminates the duplicated packet based on the GTP-U sequence number. In case of UL traffic, the NG-RAN acting as the Traffic Distributer for UL traffic duplicates the packet of the URLLC service for the UE and the UPF eliminates the duplicated packet.

NOTE 2:	The I-UPFs inserted on one leg of the redundant paths shall not behave in an UL CL or Branching Point role.

Figure 6.4.1-3: Redundant transmission with one NG-RAN node in HR roaming scenario

As shown in figure 6.4.1-3, in home routed roaming scenario, the NG RAN node is connected to the anchor UPF in the HPLMN via an intermediate UPF node in the VPLMN to support redundant transmission with two N3 and N9 tunnels. The V-SMF selects the two intermediate UPFs and setup the redundant transmission tunnels on N3 and N9 in case it receive two CN tunnel info for a URLLC QoS flow from H-SMF.

NOTE 3:	Supporting the above HR redundant transmission requests two disjointed transport paths exist between I-UPF and PSA UPF.

### 6.4.2	Procedures

The Procedures for activate redundant transmission during PDU session establishment/modification are same as corresponding procedures shown in clause 6.3.2 except for both of two AN tunnels are terminated on M-RAN nodes and S-RAN node addition is not needed.

The handover procedures in this solution can refer to the handover procedures defined in TS 23.502 [3]. The only difference is that the NG RAN node shall provide the CN with the AN Tunnel Info for the two N3 tunnels and the CN will also provide the NG RAN node with the CN Tunnel Info for the two N3 tunnels.

### 6.4.3	Impacts on Existing Nodes and Functionality

PCF:

-	The PCF can determine whether redundant transmission need to be activate for a QoS Flow based on its QoS requirement, UE's subscription and condition of network deployment.

AMF:

-	No impact in this solution.

SMF:

-	The SMF can determine whether redundant transmission need to be activated for a QoS Flow based on local policies for the DNN or S-NSSAI.

-	In case the SMF allocates CN Tunnel Info, it shall provide the CN Tunnel Info for two tunnels of the redundant transmission paths.

-	The SMF shall indicate the UPF to replicate the downlink packet and send the duplicate packets to the two N3 tunnels, and indicate the UPF to eliminate the duplicate uplink packets.

UPF:

-	In case the UPF allocates CN Tunnel Info, it shall provide the CN Tunnel Info for two tunnels of the redundant transmission paths.

-	The UPF shall be able to replicate the downlink packet and send the duplicate packets to the two N3 tunnels, and eliminate the duplicate uplink packets.

RAN:

-	The RAN shall be able to replicate the uplink packet and send the duplicate packets to the two N3 tunnels, and eliminate the duplicate downlink packets.

UE:

-	No impact in this solution.

### 6.4.4	Solution Evaluation

This solution provides high reliability transporting mechanism by performing redundant transmission between Anchor UPF and RAN node via disjointed CN tunnels. The solution has the following properties:

-	The operator can control the activation of redundant transmission or not by PCC mechanism.

-	Use of certain transport network (e.g. TSN) in the backhaul (i.e. over N3/N9) can address this issue.

-	The solution provides a 3GPP solution for setting up redundant paths on N3 in case transport layer cannot meet the reliability requirement.

-	The UPF is a single point of failure in the user plane. In case of UPF failure, the session is lost.

-	This solution has no dependency on the protocol used in application layer, which is out of control of the operator.

-	The redundant transmission can be activated on demand per QoS Flow.

-	Current existing PDU session/QoS Flow management mechanisms are reused in this solution with limited extensions shown in 6.4.3. No further impact on existing control plane mechanisms.

-	No impact on air interface and UE.

* * * End of Change * * *