# SA WG2 Meeting #129bis	S2-1812102

# 26-30 November, 2018, West Palm Beach, FL, USA	(revision of S2-18xxxxx)

Source:	Apple

Title:	23.793: Update of MPTCP procedures - Policies

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS / Rel-16

Abstract of the contribution: This contribution proposes an update for the Policy description of the MPTCP procedures and related conclusions.

# 1.	Discussion

#1. It was concluded that ATSSS rules are mapped from PCC rules and are sent to UE via the AMF.

Solution 5 is updated to move Steering Modes out of URSP.

#2. It was also concluded that the network may indicate to UE the list of applications for which MPTCP should be applied, however it was left FFS how the list of applications can be provided to the UE.

Solution 5 is updated to include an optional URSP extension that provides information to the UE about the list of applications for which MPTCP should be applied.

#3 Traffic Descriptor of an ATSSS rule can include Destination Domain or Destination FQDN.

Clause 7 is updated to clarify that IP descriptors within Traffic Descriptor of an ATSSS rule can include Destination Domain or Destination FQDN.

# 2.	Proposal

It is proposed to modify TR 23.793 as follows:

***** First Change *****

## 6.5	Solution 5: Multipath TCP (MPTCP) proxy architecture

### 6.5.1	General

This solution addresses Key Issues #1 and #4.

Multipath TCP (MPTCP) defined in [8] is a set of extensions to regular TCP to provide a Multipath TCP service, which enables a transport connection to operate across multiple paths simultaneously.

MPTCP defines how to setup multiple paths ("subflows"), manage these subflows, reassemble of data and terminate sessions.

The simultaneous use of multiple disjoint (or partially disjoint) paths across the network increases the resilience of the connectivity by providing multiple paths, protects UEs from the failure of one path, and it also increases the efficiency of the resource usage, and thus increases the network capacity available to UEs.

### 6.5.2	MPTCP proxy

When communicating with traditional TCP servers, MPTCP capable devices have to fall back to traditional TCP and cannot fully utilize their MPTCP capability. In order to support the use of MPTCP session between an MPTCP host and a TCP host, or to help to aggregate MPTCP subflows, an MPTCP proxy needs to be introduced.

Figure 6.5.2-1: Illustration of an MPTCP proxy

The MPTCP proxy makes it possible to achieve the benefits of MPTCP for access aggregation and route selection without requiring MPTCP support in all TCP servers.

### 6.5.3	Addition of MPTCP proxy to 5GC

In order to introduce the MPTCP proxy to 5GC and allow 3GPP UEs to make use of the MPTCP capability, the UPF shall incorporate the MPTCP proxy functionality for the PDU session.

Figure 6.5.3-1: Addition of MPTCP proxy to 5GC

The MPTCP proxy shall implement at least one of the following protocol options to communicate with the UE:

1.	SOCKS v4 or v5 Proxy, RFC 1928 [9];

-	Advantages: Well-known, well-supported.

-	Disadvantages: Additional connection-setup latency, protocol chattiness.

2.	Transport converter, TS 38.215 [11];

-	Advantage: Well supported (adopted by the IETF TCPM (TCP Maintenance) Working Group), designed specifically for the MPTCP use case, provides 0-RTT protocol conversion service, no overhead, can support other TCP conversion services such as TCPinc.

-	Disadvantage: IETF standardization process is ongoing (milestone Dec 2018).

### 6.5.4	MPTCP PDU session establishment procedure

This solution supports the Multi-Access PDU session establishment defined in clause 6.2. Each of the PDU sessions may have its own set of UPFs, but both child PDU sessions share a common PDU session anchor (PSA).

This procedure is based on the procedure defined in clause 6.2.2.2 with the following exceptions:

1.	The UE requests the network to select an MPTCP proxy on the user plane;

2.	The MPTCP proxy address is transported back to the UE;

3.	The child PDU sessions do not share a common IP address but they are allocated separate IP addresses.

Figure 6.5.4-1: UE-requested MPTCP PDU Session Establishment for non-roaming and roaming with local breakout

Editor's note:	It is FFS whether instead of the above PDU session setup procedure the UE could setup two independent PDU sessions for MPTCP procedures as defined in clause 6.2.2.1. In this case it is also FFS how the UE receives the address of the MPTCP proxy.

1.	The UE initiates the "UE Requested PDU Session Establishment" procedure as specified in TS 23.502 [6] clause 4.3.2.2.1 on 3GPP access with an additional preference to establish an MPTCP PDU Session. The UE can also first initiate the PDU Session Establishment procedure on untrusted non-3GPP access.

2.	If the network supports MPTCP procedures, the AMF selects an SMF that supports MPTCP procedures.

3.	The AMF sends an Nsmf_PDUSession_CreateSMContext Request to SMF.

4.	The SMF registers with the UDM.

5.	The SMF sends an Nsmf_PDUSession_CreateSMContext Response to AMF.

6.	Optional Secondary authorization/authentication.

7a.	If dynamic PCC is deployed, the SMF performs PCF selection.

7b.	The SMF may perform a Session Management Policy Establishment.

8.	The SMF selects an UPF that incorporates an MPTCP proxy functionality.

9.	SMF may perform a Session Management Policy Modification procedure.

10-17.	The user plane of the PDU session over 3GPP access is established and the UE creates the first MPTCP subflow over the 3GPP access. The first MPTCP subflow can also be created over the non-3GPP access once the PDU Session is established over the untrusted non-3GPP access.

-	In step 11, the SMF indicates to AMF the MPTCP proxy address along with the list of protocols the MPTCP proxy supports (SOCKSv4, v5 or Transport converter).

-	In step 13, the UE receives a PDU Session Establishment Accept message containing the MPTCP proxy address (if the network supports MPTCP procedures) along with the list protocols the MPTCP proxy supports, thus, the UE determines that the MPTCP PDU session over 3GPP access is established and selects one protocol to contact the MPTCP proxy from the protocol options provided by the network.

19-27.	The user-plane of the PDU session over untrusted non-3GPP access is established and the UE creates an additional MPTCP subflow over the untrusted non-3GPP access.

NOTE:	The steps 10-17 and 19-27 can be executed in parallel.

### 6.5.5	ATSSS and URSP rules for MPTCP

### 6.5.5.1 URSP rules for MPTCP

In Rel-15, the UE is responsible for establishing new PDU session(s) based on URSP rules provided by the network (PCF) and/or preconfigured in the UE as described in TS 23.503 [7] clause 6.6.2. As such, URSP rules provide SDF level steering capability for UE to establish new PDU session(s) over the preferred access network based on traffic descriptor and rule precedence.

The steering decision is performed by taking into account the congestion and delay information of the individual TCP subflows of the MPTCP connection. The UE may also take into account Access Measurement Reports, as defined in clause 6.1.5.3 or 6.6.3.2, to complement the steering and path-management decisions.

In this solution, the URSP rules described in TS 23.503 [7] clause 6.6.2 are extended to indicate the list of applications to the UE for which MPTCP should be applied:

-	The MPTCP field in the Route Selection Descriptor specifies whether to use MPTCP for the matching application;

-	If present, in case of WLAN access, the Limited  SSID(s) for MPTCP field specifies a list of SSID(s) that are allowed on the WLAN access when MPTCP is used.

The PCF may select the URSP rules applicable for each UE based on local configuration and operator policies taking into consideration e.g. UE location or time of the day.

The PCF may provide the URSP rules at the initial Registration procedure, or when the operator policies indicate that the conditions for updating the UE are met, i.e. at change of UE location or at mobility with change the AMF, or at any time, as defined in TS 23.502 [6]. Operator defined policies in the PCF may depend on input data such as UE location, time of day, information provided by other NFs, etc.

Table 6.5.5-1: UE Route Selection Policy (from TS 23.503 [7] table 6.6.2.1-1)

| Information name | Description | Category | PCF permitted to modify in a URSP | Scope |
| --- | --- | --- | --- | --- |
| URSP rules | 1 or more URSP rules as specified in table 6.6.2.1-2 | Mandatory | Yes | UE context |

Table 6.5.5-2: UE Route Selection Policy Rule (from TS 23.503 [7] table 6.6.2.1-2)

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the URSP rule is enforced in the UE. | Mandatory (NOTE 1) | Yes | UE context |
| Traffic descriptor | This part defines the traffic descriptors for the policy |  |  |  |
| Application identifiers | It consists of OSId and OSAppId(s). (NOTE 2) | Optional | Yes | UE context |
| IP descriptors | IP 3 tuple(s) (destination IP address or IPv6 network prefix, destination port number, protocol ID of the protocol above IP) | Optional | Yes | UE context |
| Non-IP descriptors | Descriptor(s) for non-IP traffic | Optional | Yes | UE context |
| DNN |  | This is the DNN information provided by the application | Optional | Yes | UE context |
| Connection Capabilities |  | This is the information provided by a UE application when it requests a network connection with certain capabilities. | Optional | Yes | UE context |  |
| List of Route Selection Descriptors | A list of Route Selection Descriptors. The components of a Route Selection Descriptor are described in table 6.6.2.1-3 | Mandatory |  |  |
| NOTE 1: Rules in a URSP shall have different precedence values. NOTE 2: The information is used to identify the Application(s) that is(are) running on the UE's OS. |  |  |  |  |

Table 6.5.5-3: : Route Selection Descriptor (from TS 23.503 [7] table 6.6.2.1-3)

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory (NOTE 1) | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory (NOTE 2) |  |  |
| SSC Mode Selection | One single value of SSC mode. | Optional | Yes | UE context |
| Network Slice Selection | Either a single value or a list of values of S-NSSAI(s). | Optional (NOTE 3) | Yes | UE context |
| DNN Selection | Either a single value or a list of values of DNN(s). | Optional | Yes | UE context |
| PDU Session Type Selection | One single value PDU Session Type | Optional | Yes | UE context |
| Non-seamless Offload indication | Indicates if the traffic of the matching application is to be offloaded to non-3GPP access outside of a PDU Session. | Optional (NOTE 4) | Yes | UE context |
| Access Type preference | Indicates the preferred Access Type (3GPP or non-3GPP) when the UE establishes a PDU Session for the matching application. | Optional | Yes | UE context |
| MPTCP | Indicates whether to use MPTCP for the matching application. | Optional | No | UE context |
| Limited SSID(s) for MPTCP | If present, in case of WLAN access, it indicates a list of SSID(s) on the WLAN access that are allowed when MPTCP is used. | Optional (NOTE 5) | Yes | UE context |
| NOTE 1: Every Route Selection Descriptor in the list shall have a different precedence value. NOTE 2: At least one of the route selection component shall be present. NOTE 3: When the Subscription Information contains only one S-NSSAI in UDR, the PCF needs not provision the UE with S-NSSAI in the Network Slice Selection information. NOTE 4: If this indication is present in a Route Selection Descriptor, no other components shall be included in the Route Selection Descriptor. NOTE 5: This field shall only be present if the MPTCP field is present. |  |  |  |  |

#### 6.5.5.2 ATSSS rules for MPTCP

The SMF maps the PCC rules into ATSSS rules which are sent to UE via the AMF.

An ATSSS rule includes the following:

a)	A Precedence value, which identifies the priority of this ATSSS rule with respect to other ATSSS rules.

b)	A Traffic Descriptor, which identifies an SDF. It may include e.g. an Application ID, IP descriptors, non-IP descriptors, etc.

c)	Steering Mode(s), which identifies how the matching SDF should be steered across 3GPP and non-3GPP accesses. The following Steering Modes will be supported:

-	Active-Standby: An MPTCP service that provides seamless handover between non-3GPP and 3GPP access in order to preserve the connection;

-	Smallest Delay: A service whereby MPTCP attempts to use the lowest-latency interface; or

-	Load-Balancing: An MPTCP service that aggregates the capacities of the access networks in an attempt to increase throughput and minimize latency;

The steering decision is performed by taking into account the congestion and delay information of the individual TCP subflows of the MPTCP connection. The UE may also take into account Access Measurement Reports, as defined in clause 6.1.5.3 or 6.6.3.2, to complement the steering and path-management decisions.

***** Next Change *****

# 7	Conclusions

The ATSSS solution in Rel-16 shall be based on the following principles:

Editor's note:	These principles below do not specify a complete ATSSS solution for Rel-16, but they provide the guidelines, which the complete solution should be based on. It is FFS how these principles can be amended or modified.

Support of MA-PDU sessions

1.	The solution shall support ATSSS with a Multi-Access PDU (MA-PDU) session, i.e. ATSSS procedures shall be applied after a MA-PDU session is established.

Editor's note:	If and how ATSSS can be applied without a MA-PDU session is FFS.

2.	A MA-PDU session is established with the Separate Establishment procedure, as specified in clause 6.2.3.1, or, with the Combined Establishment procedure, as specified in clause 6.2.3.2.

Editor's note:	It is FFS whether only one establishment procedure (Separate or Combined) or both will be supported.

3.	When the UE sends a NAS message to request a single-access PDU session and the UE supports MA-PDU sessions, the UE includes an "MA-PDU capability" indication in the NAS message. This indication may be used by the network to establish a multi-access PDU session, instead of the requested single-access PDU session, as specified in clause 6.2.4.

Policy for ATSSS Control

4.	During the establishment of a MA-PDU session, the PCF may create PCC rules that contain information for controlling the traffic steering across multiple accesses used by a MA-PDU session. In addition to the existing information in the PCC rule such as QoS and charging information, these rules as described in solution#13, also specify how the packets matching the service data flows template of the PCC rule should be routed across the 3GPP and non-3GPP accesses.

5.	The SMF maps the PCC rules into (a) ATSSS rules which are sent to UE via the AMF, and (b) Packet Detection Rules and other N4 rules which are sent to UPF. The ATSSS rules are used by the UE for uplink traffic steering and the N4 Rules are used by the UPF for downlink traffic steering.

6.	The ATSSS rules are sent to UE with a NAS message when the MA-PDU session is created or when they are updated by SMF/PCF. Similarly, the Packet Detection Rules are sent to UPF when the MA-PDU session is created or when they are updated by SMF/PCF.

7.	An ATSSS rule includes the following:

a)	A Precedence value, which identifies the priority of this ATSSS rule with respect to other ATSSS rules.

b)	A Traffic Descriptor, which identifies a service data flow (SDF). It may include e.g. an Application ID, IP descriptors (Destination Address, Destination Port, Destination Domain, Destination FQDN), non-IP descriptors, etc.

c)	A Steering Mode, which identifies how the matching SDF should be steered across 3GPP and non-3GPP accesses. The following Steering Modes will be supported:

-	Active-Standby: It is used to steer a SDF on one access (the Active access), when this access is available, and to switch the SDF to the other access (the Standby access), when Active access becomes unavailable. When the Active access becomes available again, the SDF is switched back to this access. If the Standby access is not defined, then the SDF is only allowed on the Active access and cannot be transferred on another access.

-	Smallest Delay: It is used to steer a SDF to the access that is determined to have the smallest Round-Trip Time (RTT). As defined below, measurements may be conducted to determine the RTT over 3GPP access and over non-3GPP access.

-	Load-Balancing: It is used to split a SDF across both accesses. With a 50/50 load-balancing, the SDF traffic is equally split across the two accesses. With an 80/20 load-balancing, about 80% of the SDF traffic is sent on one access and 20% on the other access.

d)	A Steering Function, which identifies whether the MPTCP or the ATSSS function shown in Fig. 7-1 should be used to steer the traffic of the matching SDF. This is useful in case the UE supports multiple functions for traffic steering.

Editor's note:	It is FFS (a) if additional steering modes are needed, and (b) if the structure of the ATSSS rule needs to be modified. The details of the Steering Function are also FFS.

8.	As an example, the following ATSSS rules could be provided to UE:

a)	"Traffic Descriptor: UDP, DestAddr 1.2.3.4", "Steering Mode: Active-Standby, Active=3GPP, Standby=non-3GPP"

-	This means "steer UDP traffic with destination IP address 1.2.3.4 to the active access (3GPP), if available. If the active access is not available, use the standby access (non-3GPP)".

b)	"Traffic Descriptor: TCP, DestPort 8080", "Steering Mode: Smallest Delay"

-	This means "steer TCP traffic with destination port 8080 to the access with the smallest delay". The UE needs to occasionally measure the RTT over both accesses, in order to determine which access has the smallest delay.

c)	"Traffic Descriptor: Application-1", "Steering Mode: Load-Balancing, 3GPP=20%, non-3GPP=80%", "Steering Function: MPTCP"

-	This means "send 20% of the traffic of Application-1 to 3GPP access and 80% to non-3GPP access by using MPTCP".

d)	"Traffic Descriptor: TCP, DestDomain google.com", "Steering Mode: Smallest Delay"

-	This means "steer TCP traffic with destination domain "google.com" to the access with the smallest delay".

Support of Measurements

9.	It shall be possible for the UE and the network to measure the RTT over both accesses. Such measurements may be required only under certain conditions, e.g. only when the UE has a valid ATSSS rule using a "Smallest Delay" steering mode.

10.	Measurements between the UE and the network take place over the user-plane.

Editor's note:	It is FFS if the network can provide measurement policy to UE to assist the UE in taking measurements. It is also FFS if additional measurements (other than RTT) are needed.

Support of MPTCP

11.	The solution will support MPTCP as follows:

a)	During the MA-PDU session establishment, if the UE wants to use MPTCP for traffic steering, the UE provides an "MPTCP Request" indication.

b)	If the network agrees to enable MPTCP for the MA-PDU session then:

i)	The network allocates two IP addresses for the MA-PDU session. This is required, otherwise MPTCP cannot be used. As specified in RFC 6824, "... there must be multiple addresses at least at one endpoint, for MPTCP to be used".

Editor's note:	Whether the two IP addresses are allocated to UE or to the MPTCP proxy is FFS.

ii)	The network may send MPTCP proxy information to UE, e.g. the IP address(es), port and type (SOCKS5 [9] or TCP Convert Protocol, TS 38.215 [11]) of the MPTCP proxy.

iiii)	The network may indicate to UE the list of applications for which MPTCP should be applied as described in subclause 6.5.5.1.

Editor's note:	It is FFS whether the MPTCP proxy information must be provided to UE, and/or whether a transparent MPTCP proxy can be used.

12. Within the same MA-PDU session, if MPTCP is enabled, it is possible to steer the MPTCP flows by using the MPTCP protocol (the MPTCP function) and, simultaneously, to steer all other flows by using a lower-layer steering functionality, called the "ATSSS function". This is schematically illustrated in Fig. 7-1, which shows an example UE model with an MPTCP function and an ATSSS function. The MPTCP flows is the traffic of the applications for which MPTCP can be applied (see bullet 11.b.iii). Note that in Fig. 7-1, the MPTCP traffic goes through the MA-PDU session but is not handled by the ATSSS function.

13.	The MPTCP proxy functionality is located in the UPF.

14.	The same set of ATSSS rules is applied for steering decisions by the MPTCP function and by the ATSSS function.

Figure 7-1: An example of a UE supporting the MPTCP function and by the ATSSS function

***** End of Changes *****