SA WG2 Meeting #129bis	S2-1812239

November 26 - 30, 2018, West Palm Beach, USA

Source:	CATT

Title:	Discussion on Xn handover issue in network sharing case

Document for:	Discussion

Agenda Item:	6.5.7

Work Item / Release:	5GS_Ph1 / Rel-15

Abstract of the contribution: This paper would like to discuss the solutions to solve network sharing issue.

1.	Discussion

It is specified in TS 23.501 that, in Rel 15, only RAN is shared in 5GS:

“In this Release of the specification, only the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported.”

Based on this, it is observed that:

Operators sharing the RAN must deploy different AMFs.

In such network sharing architecture, when an inter-PLMN change happens, i.e., the UE moves from operator A’s network to operator B’s network, the AMF serving the UE will be changed.

If AMF changes, then Xn handover doesn’t not work, at least in current specification.

To solve this issue, we have two options:

Option 1: Remove Xn handover for inter-PLMN change;

Option 2: Enhance Xn handover to support AMF change.

Option 1 seems an easy way, but if 5GC is also shared in future release, it will require the RAN to know whether 5GC is shared or not.

Option 2 seems a future-proof solution, but it brings impacts to 5GC.

2.	Proposal

It is proposed to discuss which option is preferred by the WG. Based on the conclusion, we will bring CRs in the next meeting.