SA WG2 Meeting #129-bis	S2-1812330

26 – 30 November, West Palm Beach, FL, US	(revision of S2-18xxxx)

Source:	Vodafone

Title:	Evaluation of solution 5

Document for:	Approval

Agenda Item:	6.29

Work Item / Release:	FS_UDICOM / Rel-16

Abstract of the contribution: This pCR proposes evaluation of solution #5 for FS_UDICOM

# 1 Proposal

This pCR proposes evaluation for solution #5.

******************First change***********************

### 6.5.5	Evaluation

Solution 5 proposes an architecture that does not require direct interaction between the UDM and the HSS FE or interaction between EPS UDR and 5G UDR.

The solution makes use of an adaptation layer, named Access Data Layer, that separates the applications front end logic from database technology/protocol in the back end repositories.

The solution addresses all interworking scenarios depicted in TS 23.501 [x] and TS 23.502 [2], including support for IMS procedures.

The advantages of this solution is that:

-	It addresses effective separation of HSS FE and UDM and EPS UDR from 5G UDR to enable multivendor deployments

-	It avoids new interface and new operations specification between the HSS and UDM

-	It is possible to introduce only UDM and 5G UDR when deploying a 5GS system, without having to update/upgrade either HSS FE or the EPS UDR, or having to introduce these functionalities in deployments with single core network.

-	Standardisation effort is minimised and restricted to potential incorporation of:

- 	Sh/Cx operations for UDM, in case FS_eIMS study does not incorporate the changes needed for P-CSCF restoration and T-ADS information retrieval.

- 	For EPS to 5GS handover without N26 support in single registration mode, alternative 1, update of procedure in order to reflect AMF makes use of 5GC procedures based on SBA/SBI (instead of using p2p interfaces towards legacy systems).

-	Introduction of this solution does not impact other network functions that may be accessing the different repositories, while it provides the means to further split other network functions defined in rel 15 (i.e PCF/PCRF).

The disadvantage of this solution is to support proprietary Ud implementations.

******************End of changes***********************