SA WG2 Meeting #129BIS	S2-1812393

West Palm Beach, USA, 26-30 November 2018	(revision of S2-1811565)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0656 | rev | 5 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network controlled NSSAI for SR-related Access Stratum connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NEC, Telecom Italia, Huawei, InterDigital, AT&T, Verizon, Sprint, Orange, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2018-11-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed in tdoc S2-1810390 we need to conclude on the topic of what NSSAI value to include in the RRC connection establishment when this is triggered by a SR (whithout this, the Specification is incomplete). This CR proposes a way forward in alignment with S2-1810390. This CR also addresses the need to not send NSSAI in RCC if any of the HPLMN or VPLMN have any privacy concerns on inlcuding NSSAI in RRC as there is no confidentiality protection of RRC msg 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A PLMN can instruct the UE on which behaviour to adopt when triggering a Service Request procedure, by including Access Stratum Connection Establishment NSSAI Inclusion Mode information in a Registration Accept Message that instructs the UE to: 1.- Include in Access Stratum Connection Establishment a NSSAI = Allowed NSSAI for the UE, or 2. Include in Access Stratum Connection Establishment only the S-NSSAIs of the Network Slices that trigger the RRC Connection Establishment, or 3. Include no NSSAI for Service Request and Registration Request not due to mobility or initial access. 4. Never include any NSSAI in RRC regardless of the procedure . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined UE behaviour with respect to providing NSSAI to the RAN in Access Stratum connectivity establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | If SA3 deems that it is a security issue to send S-NSSAI in cleartext in AS signaling and to send encrypted S-NSSAI in the NAS signaling, then it is an option to remove S-NSSAI from NAS signaling, instead RAN can send S-NSSAI in the N2 signaling towards AMF. |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.15.x	Operator-controlled inclusion of NSSAI in Access Stratum Connection Establishment

The Serving PLMN, based on information received from the UDM, can control, per access type, whether the UE is allowed to include NSSAI information in the Access Stratum. The UE shall not include NSSAI information in the Access Stratum unless it has been informed that it is allowed to do so.

During a Registration procedure the AMF may, if informed by the UDM, send an Access Stratum NSSAI Inclusion parameter to the UE in the Registration Accept message. This parameter indicates whether the UE can include NSSAI information in the Access Stratum Connection Establishment (e.g. an RRC connection Establishment defined in TS 38.331).

In the case of roaming the Serving PLMN can, based local policies, set the Access Stratum NSSAI Inclusion parameter to indicate that the UE is not allowed to include NSSAI information in the Access Stratum Connection Establishment, even if the information in from the UDM in the Home PLMN indicates that the UE is allowed to.

If the UE is allowed to include NSSAI information, according to the parameter above, then the NSSAI information is set depending on the cause of the Access Stratum Connection Establishment.

If it is caused by a Periodic Registration Update, or a Registration procedure used to update the UE capabilities the NSSAI is set to the Allowed NSSAI, if available.

If it is caused by a Mobility Registration Update or Initial Registration the NSSAI is set to the Requested NSSAI provided by the NAS layer and defined in clause 5.15.5.2.

If it is caused by a Service Request it is set as follows:

- The NSSAI is set to the S-NSSAI(s) of the Network Slice(s), or;

- The NSSAI is set to the Allowed NSSAI, if available.

The UE may store the required behaviour for a PLMN per Access Type as part of the network slicing configuration.

**** END OF CHANGES ****