3GPP TSG-SA WG2 Meeting #129BIS	S2-1812455

West Palm Beach, USA, 26-30 November 2018

Source:	Orange, ETRI

Title:	Updates of Solution #6

Document for:	Approval

Agenda Item:	6. 11

Work Item / Release:	FS_eNA

Abstract of the contribution: The proposal updates Solution #6 in order to resolve editor’s notes.

# Discussion

The modifications address the following topics:

| Topic | Proposal |
| --- | --- |
| "Editor’s note: The update of subscription for the other parameters (i.e., CDI scope, etc.) is FFS." | Addition of a note: The update of subscription for the other parameters (i.e., CDI scope, etc.) is left for the normative phase. |
| “Editor's note: How to smooth out the peaks of data transfer in the NEF is FFS.” | Addition of a note: The exact definition of alternatives for data transfer from AF through NEF is left for the normative phase. |

# Proposal

It is proposed to modify the following clause in TR 23.791.

**** Start of Change ****

#### 6.6.1.11	Procedure Interactions with AFs for update of Data Collection Subscription to AF

AF

NEF

NWDAF

2.

Naf_DataCollectionService_

Notify()

1.

Naf_DataCollectionService_Update()

Figure 6.6.1.11-1: Procedure for Interactions with AFs for update of Data Collection Subscription to AF

- To update of subscription for data to be collected from AF, the NWDAF invokes a Naf_DataCollectionService_Update service operation from AF with the subscription ID, newly requested subscription duration, new time frame for measurement, optionally new sampling interval, and the newly requested period between two notifications as described in clause 6.6.1.2. The AF responds the request service operation with the indication of success or failure of the subscription update request.

NOTE: The update of subscription for the other parameters (i.e., CDI scope, etc.) is left for the normative phase.

2.	To report the data to be collected from AF when it is available to be collected, the AF invokes a Naf_DataCollectionService_Notify service operation to notify NWDAF with the requested CDI(s) according to revised time interval.

NOTE 2:	If the AF is not allowed by the operator to access directly the NF(s) as specified in clause 6.2.10, TS 23.501 [2], the service operations in step 1-2 should use the NEF to interact between the AF and the NWDAF.

NOTE: The exact definition of alternatives for data transfer from AF through NEF is left for the normative phase.

**** End of Changes ****