3GPP SA WG2 Meeting #129	S2-1810271

15 – 19 October, 2018, Dongguan, China	(revision of S2-18xxxx)

Source:	Ericsson

Title:	Solution 5 update

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution updates solution 5 in TR 23.742

# 1 Discussion

## 1.1 Separation of business logic and discovery and selection

.

Nearly all proposed solutions suggest a separation of service business logic (BL) from discovery and selection, On an abstract level these are very similar in that the consumer business logic should not need to discover or select a producer, it should only send a request. In solution #5 consumers do discovery, but selection may be done in the Centralized Service Framework (CSF). Solution #4 and #6 allow for consumers to also do discovery.

Common for all solutions is that is the CSF south of the BL need to implement 3GPP specific discovery (except solution #5) and selection functions if rel 15 NF/NF services are used. E.g. in rel 15 SUPI may be used when discovering UDM/UDR, and when selecting AMF, the AMF set is used as one input. Thus, whatever is south of the BL must be 3GPP NF/NF service aware. This seems like a bad idea. It would be better if the CSF would be 3GPP agnostic. This is reflected in proposed updates of solution #5.

## 1.1 how to handle subsequent request if consumer is divided into BL and selection?

One of the challenges to separate the business logic from discovery and selection is how to treat subsequent requests. See example in figure 1.2-1 where AMF creates a SMF context and later does an update to this context.

Figure 1.2-1

If discovery is done by AMF, which is an option in solution #4 and #6, and mandatory in solution #5, then AMF is able to create a unique URI per NF/NF service instance based on received discovery result, and it could use this URI for subsequent messages regarding this SMF context.

In #5 a pseudo producer is used, and thus if more than one pseudo producer address needs to be created per service, the NRF may automatically create these pseudo producers to reduce the need for configuration. Alternatively, a there could be a proxy configuration in the NF as in solution #4. This is reflected in proposed updates of solution #5.

If there is no 3GPP specific discovery and selection, thus an arbitrary producer service instance can be selected then the CSF may do discovery and selection. To solve the problem of subsequent messages the following solutions are identified:

- A service producer’s instances have only one resource URI, which is used by consumers in initial requests. When the consumer sends a request using this URI, the CSF discovers and selects a producer service instance. Then the CSF or the producer may either:

  - Send a redirect in a HTTP 201 message with a URI identifying selected resource; or

  - respond with a HTTP 308 (permanent redirect) with the URI of the selected resource.

The selected resource URI is stored by the consumer in its context state record (local or external), and the consumer will use this URI in all subsequent messages regarding this producer context.

- A service producer’s instances have only one resource URI, which is used by consumers in initial requests When a consumer sends a request using this URI, the CSF discover and selects a service producer instance, the CSF understands the specific contents of a request and if it is a create context type of request, the CSF stores the binding between consumer instance and producer instance per consumer and producer context.

- Same as in 1) but instead of a resource URI, cookies are used

Bullet 2) need specific understanding of the content in a HTTP body, also it introduces that dynamic states needs to be stored in CSF.

Using URIs instead of cookies is regarded as more RESTful.

Solution 1) is proposed. This is reflected in proposed updates of solution #5.

# 2 pCR

******************** Start change ********************

## 6.5	Solution 5: Flexible Service Framework Deployment

### 6.5.1	Introduction

This solution addresses key issues 3 "Improvements to service framework related aspects" and key issue 5 "SBA backward and forward compatibility".

### 6.5.2	High level description

In Rel-15 the service framework consists of the functional components illustrated by the figure 6.5.2-1 below.

Figure 6.5.2-1: Rel15 service framework

The NRF handles the service registry and the consumer authorization. The NF may consist of one or several NF Service consumers and NF Service producers. An NF service consumer consists of the service business logic, service discovery and service instance selection. An NF producer consists of service business logic, service registration and optionally load balancing. It is implementation dependent if there is a load balancer. NF service consumers communicates directly with NF service producers. The ovals in figure 6.5.2-1 can be seen as components of the service framework.

NOTE:	An NF service may include both the consumer and producer roles in the same NF service. They are discussed and shown separately in figures in order to clarify the functionality needed for the 2 different roles.

In a new service framework, these types of components will still need to exist in one way or the other. The solution in 6.2 introduces a service framework where all of these components are moved to a logically centralized service framework. This does not mean that in deployment time that the service framework is centralized. For simplicity reasons in this clause 6.5 centralized framework mean that it is logically centralized.

To enable co-existence of existing Re. 15 service and the possibility to choose at network design which type of service frame work is wanted by an operator, the following model is proposed as shown in figure 6.5.2-2 below.

Figure 6.5.2-2: Flexible service framework

The NF/NF services in Rel-15 can be kept as -is. If a centralized service framework is selected in the network design (deployment), the service consumers will either get pseudo addresses of the producer leading to the centralized service framework (CSF) or it will be configured with a HTTP proxy address leading to the centralized service framework. By this all the consumers’ service invocations will go via the CSF. Still, consumers will do selection, which may be a selection of a service set/NF. If a centralized service framework is not selected in the network design (in deployment), then rel. 15 procedures will be used.

NOTE: Using proxy may inhibit use of TLS. E.g. use of SEPP required change of FQDN in the HTTP header

Standardized consumers always do discovery of a service producer. The reason for this is backward compatibility and the possibility to support both services frameworks as well as potentially lesser configuration. The discovery procedure is done seldom, and the basic part can be implemented in a few lines of code, and depending on implementation architecture used by a vendor, it may be a common SW entity that is re-used by all the vendor's consumer implementations. However, in Rel-15 there are some special behaviour for discovery of for example UDM where MNC and MCC are important parameters to consider, and thus it may not be so that all consumers can have just a generic discovery procedure.

One should observe here that instance selection is not always generic. For example, when AMF selects an SMF instance the following should be considered:

-	Selected Data Network Name (DNN).

-	S-NSSAI.

-	Subscription information from UDM, e.g.

-	per DNN: whether LBO roaming is allowed

-	per S-NSSAI: the subscribed DNN(s)

-	per (S-NSSAI, subscribed DNN): whether LBO roaming is allowed

-	Local operator policies.

-	Load conditions of the candidate SMFs.

-	Access technology being used by the UE

This is different from when AMF selects another AMF then the following shall be considered:

-	AMF Region ID and AMF Set ID derived from GUAMI.

-	Requested NSSAI.

-	Local operator policies.

-	Availability of candidate AMFs.

-	Load balancing across candidate AMFs (e.g. considering weight factors of candidate AMFs in the AMF Set).

As can be seen some of the factors to consider for AMF when selecting SMF could be generalized, but some are unique per selection. Therefore a consumer will do selection based on 3GPP unique parameters, if a CSF is deployed, a consumer may just do selection of NF instance or service instance set. When the consumer has made the selection, the CSF may select a service instance.

If it is arbitrary which producer instance is selected, i.e. no 3GPP parameters are to be considered in the selection, the discovery result may be one SBI address representing all the producers and the CSF will select a producer instance. The producer may reply with a resource URI identifying the specific producer URI which shall be used by the consumer for subsequent requests.

### 6.5.3	Services and illustrated Procedures

#### 6.5.3.1	Registration and de-registration of NF instances/NF service instances

This clause describes the high-level procedures for the registration of NF instances/NF service instances at the service framework. In general, the procedures for the registration of NF/NF service (instances) are unchanged compared to Rel-15.

Registration of NF instances/NF service instances:

-	the NF instances/NF service instance uses the Rel-15 mechanism to register it self.

De-Registration of NF instances/NF service instances:

-	the NF instances/NF service instance uses the Rel-15 mechanism to deregister itself.

#### 6.5.3.2	Discovery and selection of NF services and communication between consumer and producer when no CSF

A legacy consumer uses the Rel-15 mechanism to discover producers. New standardized consumers services always do service discovery. See clause 6.5.2. For examples of consumer to producer communication see below figures 6.5.3.2-1 and 6.5.3.2-2 for non-roaming and 6.5.3.2-3 and 6.5.3.2-4 for roaming.

Figure 6.5.3.2-1: Consumer to producer communication, no centralized service framework

Figure 6.5.3.2-1 is the Rel-15 behaviour, and will be used if no centralized service framework is wanted.

##### 6.5.3.x	Discovery and selection of services and communication between consumer and producer with CSF

##### 6.5.3.x.1	Example when 3GPP specific parameters are needed in the selection

Figure 6.5.3.x.1-1: Consumer to producer communication, with centralized service framework, 3GPP specific parameters needed for selection

In figure 6.5.3.x.1-1 represent an example of a consumer x instance #1 that wants to communicate with the producer y. The consumer is configured with a proxy address or the NRF has generated pseudo addresses for producer y as shown in figure, which leads to the CSF. The producer y has 3GPP specific parameters needed for selection. The consumer does discovery and gets a list of pseudo network addresses of service instance sets (or NFs instances if rel 15) The consumer selects a service instance set (or NF instance) of producer y, set #1 in this example, and sends the request to pseudo address for the service set (or NF)which is the an ingress address of CSF. The CSF will select a producer instance among the selected service instance set (or NF instance) to be used for this operation, producer service y set #1 service instance #2 in this example

##### 6.5.3.x.2	Example when 3GPP specific parameters are not needed in the selection (and discovery)

Figure 6.5.3.x.2-1 shows an example flow for a request where no 3GPP specific selection (nor discovery) is needed

Figure 6.5.3.x.2-1

A consumer x instance #1 wants to communicate with the producer y.. The consumer does not need to consider 3GPP specific parameters for selection The consumer does discovery and gets only one address representing all instances and sets (NFs if rel 15) of producer y this address is either a pseudo address, or if proxy is used as in this example a real address representing all producers. The consumer selects this address and sends the request to the configured proxy address, which is the ingress address of CSF. The CSF will select a producer instance of producer y and sends the request to this instance. The producer responses and includes a URI that shall be used for all following requests regarding this context. In this case it is a URI that represent the selected set.

In a subsequent request regarding this context, the consumer x uses the received URI (uri of set #1) and the CSF uses this to route the message to an instance within the service set. In this example instance #1 of service set #1 of service y

If pseudo address is used representing the consumer, then similar procedure is used..

##### 6.5.3.x.3	Examples of roaming

The following figures shows roaming examples of how a logically centralised service framework deployed the visited network interacts with a Rel-15 based Service framework in the home network and vice versa.

Figure 6.5.3.x.3-1: Consumer to producer communication over roaming interface, centralized service framework in VPLMN

Figure 6.5.3.x.3-2: Consumer to producer communication over roaming interface, centralized service framework in HPLMN

The SEPP in figures 6.5.3.x.3-1 and 6.5.3.x.3-2 may of course do topology hiding. These figures illustrated the principles.

#### 6.5.3.3	Non-standardized services

Non-standardized services could be implemented targeting a specific network design model. These non-standard services may utilize a centralized service framework by for example to not do service discovery or instance selection. They may interact with the centralized service framework in a different way compared to the standardized services.

### 6.5.4	Impacts on existing NF/NF Services and Interfaces

No impact on existing NF/NF services and interfaces

### 6.5.5	Evaluation of the Solution

Editor's note:	This clause provides an evaluation of the solution.

********************  End  change  ********************