3GPP TSG-SA2 Meeting #129	S2-1810613

15 - 19 October 2018, Dongguan, China	(was S2-188050)

Source:	Huawei, HiSilicon

Title:	KI#15: Update solution 42 and solution 43 for redirection from one CN type to another

Document for:	Approval

Agenda Item:	6.9

Work Item / Release:	FS_CIoT_5G / Rel-16

Abstract of the contribution: This paper proposes to update solution 42 and solution 43 to remove EN and also some issue consideration, e.g. how to solve the case without N26 and load condition are added.

# 1	Introduction

Key Issue 15: Core Network selection for Cellular IoT aims to study how to inform the UE whether and which 5GS CIoT features are available in 5GC, and how to steer the UE to EPC or 5GC based on operator policies or load condition if the CIoT features required by the UE are supported in both EPC and 5GC. The requirements regarding this KI include:

- “It shall be possible for a UE supporting CIoT features to discover whether 5G CIoT feature(s) are supported in 5GC via NB-IoT/WB-E-UTRA access, and to perform core network selection between EPC and 5GC based on such discovery.”

- “It shall be possible for the RAN to take into account CIoT feature indication(s) from the UE to perform AMF selection.”

- “Upon UE selecting 5GC, it shall be possible for the 5GC, based on e.g. operator policy or load conditions, to redirect the UE to EPC, and vice versa.”

- “Upon redirecting the UE to EPC, it shall be possible for the EPC to prevent idle and connected mode mobility to 5GC, and vice versa.”

# 2	Discussion

Two mechanisms have been proposed on how to redirect the UE to the other CN type. Solution 42: NAS based Redirection between Core Networks and Solution 43: RRC based Redirection between Core networks apply to KI 15. In particular they address the requirements c), d) above.

There is the following editor’s notes in Solution 42/43:

Editor's note: How the MME knows the 5GC is available for the UE is FFS.

Editor’s Note: How to re-enable S1 capabilities if disabled is FFS.

Editor's note: How the AMF knows the EPC is available for the UE is FFS.

Editor’s Note: How to re-enable N1 capabilities if disabled is FFS.

## 2.1 	UE capability for redirection decision

When the AMF or MME determines to redirect UE between 5GC and EPC, it shall take the 5GS CIoT features or EPS CIoT optimisations supported by the UE into account. For example if a UE doesn’t support EPS CIoT optimisations, the AMF can’t redirect the UE to EPC. This information is not same as whether the UE supports S1/N1 mode. The Preferred Network Behaviour includes the CIoT optimisations the UE supports and what it would prefer to use. It is proposed to clarify that the Preferred Network Behaviour can be used by the AMF/MME to judge whether it can redirect the UE to other type of CN, i.e., whether the 5GC/EPC is available for the UE.

## 2.2	Re-enablement for N1/S1 mode

TS 24.501 provides how to re-enable N1 mode, in clause 4.9.1:

The UE shall re-enable the N1 mode capability when the UE powers off and powers on again or the USIM is removed.

As an implementation option, the UE may start a timer for re-enabling N1 mode capability, after the N1 mode capability was disabled. On the expiry of this timer, the UE should re-enable the N1 mode capability.

And in clause 4.9.2:

When the UE supporting both N1 mode and S1 mode needs to stay in E-UTRA connected to EPC, in order to prevent unwanted handover or cell reselection from E-UTRA connected to EPC to NG-RAN connected to 5GCN, the UE operating in single-registration mode shall disable the N1 mode capability for 3GPP access and:

a)	shall set the N1mode bit to "N1 mode not supported" in the UE network capability IE (see 3GPP TS 24.301) of the ATTACH REQUEST message and the TRACKING AREA UPDATE REQUEST message in EPC; and

b)	the UE NAS layer shall indicate the access stratum layer(s) of disabling of the N1 mode capability for 3GPP access.

c)	if another RAT of the registered PLMN or a PLMN from the list of equivalent PLMNs cannot be found, or the UE does not have a registered PLMN, then perform PLMN selection as specified in 3GPP TS 23.122 [5]; or

d)	if no other allowed PLMN and RAT combinations are available, then the UE may re-enable the N1 mode capability for 3GPP access and remain camped in NG-RAN of the registered PLMN, and may periodically scan for another PLMN and RAT combination which can provide EPS services or non-EPS services (if the UE supports non-EPS services).

The UE shall re-enable the N1 mode capability for 3GPP access when the UE performs PLMN selection.

The UE should memorize the identity of the PLMN where N1 mode capability for 3GPP access was disabled and should not consider this PLMN in subsequent PLMN selections as specified in 3GPP TS 23.122

For the re-enablement of N1 or S1 mode, it is proposed reuse the same mechanism. A clarification is added that the existing procedure in TS 24.501 covers the re-enablement of N1 mode in this case.

## 2.3	RRC based redirection

For the RRC based redirection, the UE N1/S1 mode capability is not required to be disabled. Thus when the UE moves to the other CN type, it need consider how to prevent the UE is redirected back to the original CN type.

This CN type redirection may not be subscription based, e.g. it can be based on the load condition or operator policy. From the target CN point of view it does not know why the UE was redirected, e.g. load conditions, operator policy, radio conditions etc. Depending upon when the UE is rejected by the CN there may not be a context available to transfer to the target CN type the UE was redirected to (e.g. when the redirection occurs during initial registration or attach), or the N26 interface may not exist between the 2 CN types, therefore to enable the target CN to know that the UE was redirected it is proposed to introduce a “CIoT Inter CN Redirection” indicator in the Registration/TAU/Attach request messages to indicate the redirection to the target CN.

When the target CN receives a “CIoT Inter CN Redirection” indication it sets an EPC/5GC forbidden indication in its UE context. The “CIoT Inter CN Redirection” is not transferred over N26 when the CN type is changed to prevent confusion in the target CN type when the context is retrieved.

We also see this information as useful when the UE is moving between the different CN NFs in the target CN domain, as a new CN NF may not be aware that the UE is redirected from different CN system. For example, when the UE moves from MME1 to MME2, MME2 is not aware that the UE has been redirected from 5GC. If MME1 also transfers the 5GC forbidden indicator to MME2, MME2 can also set the correct RFSP/HRL.

## 2.4	Load condition consideration

In case the redirection is triggered due to load conditions (e.g. network congestion), after a period of time, 5GC may allow the UE return. In this case the redirection should be regarded as temporary.

It is proposed that the redirection is enhanced with a timer to indicate for how long the UE is not allowed to return to the original CN type. As such we see the benefit to introduce this mechanism, the enhancement can be listed as below:

- For the NAS based redirection, the AMF or MME may provide a redirection timer to UE. After the redirection timer expiry, the UE can re-enable the N1/S1 mode capability for 3GPP access, similar to the implementation option timer handling in TS 24.501 clause 4.9.1, except the timer value is provided by the network.

- For RRC based redirection, when the MME receives the TAU (or Attach) message with “CIoT Inter CN Redirection”, the MME additionally starts a redirection timer associated with the 5GC forbidden indicator. After the redirection timer expiry, the MME removes the 5GC forbidden indicator. Similar can also be done at the AMF.

## 2.5	Other Notes and ENs

1) In clause 6.42.2:

Editor's note:	It is FFS how the solution interoperates with the steering of roaming as defined in TS 23.122 [21], e.g. whether the CN redirection or PLMN redirection takes precedence.

It is proposed to resolve the EN by clarifying that the Steering of roaming is not applied in 5GC if the UE is redirected to EPC. This means that the CN redirection takes precedence over Steering of Roaming.

2) In clause 6.43.2

Editor's note:	RFSP/SPID has not been specified for NB-IoT UEs.

It is proposed to replace this by a NOTE.

3) In clauses 6.43.4.1.2 & 6.43.4.1.3

NOTE 2:	As per current procedures the AS layer indicates the available core networks to the NAS layer. If the target CN is not available for the UE, the UE stays registered in the source core network.

This note has been replaced with more descriptive text of the UE behaviour.

# 3	Proposal

It is proposed to approve the changes to TR23.724 listed below.

*** Start of changes ***

## 6.42	Solution 42: NAS based Redirection between Core Networks

### 6.42.1	Introduction

This NAS based solution applies to Key issue 15 of load balancing the UEs between the two systems.

### 6.42.2	Functional Description

This solution is a procedure to steer the UE during the (re-)registration procedure.

In EPC, when UE sends the Attach Request to the MME and MME determines that the UE needs to move to the 5G system, the MME shall reject the Attach Request and send an appropriate EMM cause indicating the target CN system to the UE. After, the UE shall disable S1 mode and re-enable the N1 mode if it was disabled and perform registration in the target 5G system. If redirection is triggered by load conditions, the MME may include a redirection timer in the rejection message. When the timer expires, the UE re-enables S1 mode regardless of whether PLMN selection is performed. The timer handling in the UE is similar to the implementation option described in TS 24.501 [10] clause 4.9.1 for 5GS.

Similar, in case of 5GC, the AMF will send a Registration Reject with a cause indicating to the UE to register in the EPC network. Upon the indication, the UE shall disable the N1 mode and re-enable the S1 mode if was disabled. If redirection is triggered by load conditions, the AMF may include a redirection timer in the rejection message. When the timer expires, the UE re-enables N1 mode regardless of whether PLMN selection is performed. The timer handling in the UE is as the described implementation option in TS 24.501 [10] clause 4.9.1.

### 6.42.3	Support of EPC interworking

TS 23.501 [5], clause 5.17 applies when a registered/connected UE(s) is moved to 5GC or EPC systems.

### 6.42.4	Procedures

#### 6.42.4.1	Redirection using NAS indication

##### 6.42.4.1.1	General

When a UE triggered registration either in EPC or 5GC network, network redirects the UE to target system by indication in the NAS layer.

##### 6.42.4.1.2	Redirection from EPC to 5GC network

Figure 6.42.4.1.2-1: Redirection from EPC to 5GC network

1.	The UE sends the Attach or TAU Request as step#1-2 of Figure 5.3.2.1-1 in TS 23.401 [4]. The UE includes an indication that it supports N1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports N1 mode and that Preferred Network Behaviour, the MME knows the UE can be redirected to 5GC.

2.	If the MME decides to redirect the UE to the 5GC network, then the MME rejects the UE's attach request and sends an Attach / TAU Reject with appropriate EMM cause to indicating the UE to be registered in the 5GC network.

3.	Upon target CN type indication in the Attach Reject the UE disables S1 mode and re-enables the N1 mode if disabled, and the UE starts the 5GC Registration procedure, see Figure 4.2.2.2.2-1 in TS 23.502 [7].

If redirection is triggered by load balancing, the MME may provide a redirection timer to UE in Attach / TAU Reject message. When the redirection timer expires, the UE re-enables S1 mode regardless of whether PLMN selection is performed.

If the UE is provided with a redirection timer, the UE should store the identity of the PLMN where S1 mode was disabled and should not consider this PLMN in subsequent PLMN selections for EPC until the redirection timer expires. Otherwise, the UE should store the identity of the PLMN where S1 mode capability was disabled and should not consider this PLMN in subsequent PLMN selections for EPC.

If the UE cannot find a cell supporting connectivity to 5GC, the UE re-enables the S1 mode and attempts to Attach to EPC. The UE includes the “Inter-CN redirection” indication in the Attach request indicating that the UE has been redirected from EPC to 5GC. Based on the indication, the MME ceases the redirections to 5GC for this UE.

4.	After, the 5GC will accept the UE registration as described in the Figure 4.2.2.2.2-1 in TS 23.502 [7].

##### 6.42.4.1.3	Redirection from 5GC to EPC network

Figure 6.42.4.1.3-1: Redirection from 5GC to EPC network

1.	The UE sends the Registration Request as step#1-3 of Figure 4.2.2.2.2-1 in TS 23.502 [7]. This can be initial, mobility or periodic registration. The UE includes an indication that it supports S1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports S1 mode and the Preferred Network Behaviour, the AMF knows the UE can be redirected to EPC.

2.	AMF decides to redirect the UE to the EPC, the AMF rejects the UE's Registration Request and sends a Registration Reject with appropriate MM cause to indicating to the UE to attempt a registration in the EPC network.

3.	Upon CN type indication in the Registration Reject, the UE disables the N1 mode and re-enables the S1 mode if disabled, and the UE starts the EPC Attach or TAU procedure, see Figure 5.3.2.1-1 in TS 23.401 [4].

If redirection is triggered by load balancing, the AMF may provide a redirection timer to UE in Registration Reject message. If the redirection timer expires, the UE re-enables N1 mode regardless of whether the PLMN selection is performed.

If the UE is provided with a redirection timer, the UE should store the identity of the PLMN where N1 mode capability was disabled and should not consider this PLMN in subsequent PLMN selections for 5GC until the redirection timer expires. Otherwise, the UE should store the identity of the PLMN where N1 mode capability was disabled and should not consider this PLMN in subsequent PLMN selections for 5GC.

If the UE cannot find a cell supporting connectivity to EPC, the UE re-enables the N1 mode and attempts to Attach to 5GC. The UE includes the “Inter-CN redirection” indication in the Registration Request indicating that the UE has been redirected from 5GC to EPC. Based on the indication, the AMF ceases the redirections to EPC for this UE.

4.	After, the EPC will accept the UE Attach/TAU as described in the Figure 5.3.2.1-1 in TS 23.401 [4].

##### 6.42.4.1.4	Interaction with Steering of Roaming

If the AMF rejects the registration request and redirects the UE to EPC, the steering of roaming as described in Annex C in TS 23.122 [21] cannot be applied. In this case the UE may be redirected to EPC as described in this solution.

### 6.42.5	Impacts on existing entities and interfaces

EPC impacts:

-	If a MME determines that the UE needs to be redirected to other system, then MME needs to use an appropriate EMM cause and notify to the UE in NAS layer.

-	If redirection is triggered by load condition, the MME may provide a redirection timer to UE in Attach / TAU Reject message. If the redirection timer expires, the UE re-enables S1 mode regardless of whether the PLMN selection is performed.

5GC impacts:

-	AMF uses an appropriate EMM cause and notifies to the UE over NAS.

-	If redirection is triggered by load condition, the AMF may provide a redirection timer to UE in Registration Reject message. If the redirection timer expires, the UE re-enables N1 mode regardless of whether the PLMN selection is performed.

UE impacts:

-	UE needs to understand the NAS rejection value and performs actions accordingly, i.e. disable the S1 or N1 mode.

-	UE needs to support the timer as provided by MME/AMF. The UE needs to indicate the “Inter-CN redirection” indicator in Attach/Tracking Area Update and Registration Requests, when it failed to connect to the target CN and returns back to the source CN.

### 6.42.6	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

## 6.43	Solution 43: RRC based Redirection between Core networks

### 6.43.1	Introduction

This RRC based solution applies to Key issue 15 of load balancing the UEs between the two systems.

### 6.43.2	Functional Description

This solution provides a procedure to redirect the UE between the 5GC or EPC by the network over a RRC notification.

Proposed to re-use the "target CN indication" in the N2 release message by the AMF (in Rel-15 specified for Emergency Services) for the load balancing purpose. In this case, when the AMF is notified (e.g. by O&M) or determines based on local policy, and UE’s Preferred Network Behaviour, then the AMF triggers N2 release with an indication to redirect the UE to the target CN. After, the UE performs Attach/TAU procedure with the “CIoT Inter CN Redirection” indication in EPC. Based on the received “CIoT Inter CN Redirection” indication the MME sets a 5GC forbidden indicator in the MM Context. If redirection is triggered by load conditions (via O&M notification), the MME may start a redirection timer associated with 5GC forbidden indicator. When the timer expires, the MME removes the 5GC forbidden indicator.

In addition, similar indication needs to be introduced in the S1-AP interface i.e. MME will indicate the RAN to perform the redirection of a UE to the target 5GC system which will be notified to the UE in the RRC release. In fact, similar mechanism is defined in TS 36.413 [22] (cause IE "Load Balancing TAU Required") for load balancing between MMEs within the EPC. Similar, upon determining (e.g. by O&M notification or by local policy, and UE’s Preferred Network Behaviour) by the MME that the UE(s) needs to move to target network, then MME will trigger UE context release command with a cause IE i.e. target CN information (i.e. 5GC) to the RAN which is notified to the UE in the RRC release, thus UE can perform the registration procedure with the “CIoT Inter CN Redirection” indication in the target CN system (i.e. 5GC). In addition, MME may provide the RFSP Index and handover restriction list to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in 5GC when it returns to Idle mode, and to prevent handovers to 5GC in Connected mode. After, the UE performs the registration procedure with the “CIoT Inter CN Redirection” indication in the 5G system. Based on received “CIoT Inter CN Redirection” indication the AMF sets the EPC forbidden indicator. If redirection is triggered by load conditions (via O&M notification), the AMF may start a redirection timer associated with EPC forbidden indicator. When the timer expires, the AMF removes the EPC forbidden indicator.

The “CIoT Inter CN Redirection” indication is passed between the CN types by the UE, as depending upon when the redirection occurs a MM context may not exist in the source CN type for the target CN type to retrieve, for example if the redirection occurred during Initial Registration or Attach or N26 not be used.

To prevent confusion in the target CN type (e.g. the UE registers on 5GC as it is the only CN type available for a certain location and the AMF retrieves a MM context from EPC with a 5GC forbidden indicator), the 5GC forbidden/EPC forbidden indication in the MM context is not transferred between 5GC and EPC if mobility with N26 occurs.

NOTE: RFSP/SPID has not been specified for NB-IoT UEs.

### 6.43.3	Support of EPC interworking

TS 23.501 [5], clause 5.17 applies when a registered/connected UE(s) is moved to 5GC or EPC systems.

The EPC forbidden / 5GC forbidden indicator is not transferred in the MM context between the network types.

### 6.43.4	Procedures

#### 6.43.4.1	Redirection using RRC indication

##### 6.43.4.1.1	General

The network will redirect the UEs to the target system based on load balancing determination as below.

##### 6.43.4.1.2	Redirection from EPC to 5GC network

Figure 6.43.4.1.2-1: Redirection from EPC to 5GC network

0.	A UE is attached to the EPC network. As part of Attach / TAU procedure, the UE includes an indication that it supports N1 mode and its Preferred Network Behaviour. The Preferred Network Behaviourr includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports N1 mode and the Preferred Network Behaviour, the MME knows the UE can be redirected to 5GC.

1.	If the MME has determined that the UE need to be redirected to the 5GC, when the MME releases the S1 context, the MME includes an indication of the target CN information (i.e. 5GC) to the Initial Context Release command.

In addition, MME may provide the RFSP Index to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in 5GC when it returns to Idle mode.

NOTE 1:	Defining the appropriate cause value of indication is up to the Stage 3.

2.	RAN node will release the RRC connection with an indication of the target core network type and idle-mode camping policies.

3.	RAN node sends the Release complete to the MME.

4.	After the RRC release, the UE attempts to perform a registration procedure with “CIoT Inter CN Redirection” indication in the registration request to the 5G system. 

If the UE finds a cell supporting connection to 5GC, it sends a Registration Request to 5GC. The AS layer in the UE informs the NAS about the redirection between CN types so “CIoT Inter CN Redirection” can be included in the Registration Request.

The AMF gets the UE context from EPC as described in the clause 4.11 in TS 23.502 [7], if the N26 interface exists.

5.	The AMF sends the Registration Accept to the UE. Afterwards, based on received “CIoT Inter CN Redirection” indication the AMF sets the EPC forbidden indicator and may provide the handover restriction list to the RAN which prevents the handovers to the EPC.

If redirection is triggered by load conditions (via O&M notification), the AMF may start a redirection timer associated with the EPC forbidden indicator. When the timer expires, the AMF removes the EPC forbidden indicator.

##### 6.43.4.1.3	Redirection from 5GC to EPC network

Figure 6.43.4.1.3-1: Redirection from 5GC to EPC network

0.	A UE is registered in the 5G system. As part of Registration procedure, the UE includes an indication that it supports S1 mode and its Preferred Network Behaviour. The Preferred Network Behaviour includes the 5GS/EPS CIoT optimisations the UE supports and what it would prefer to use. Based on the indication that the UE supports S1 mode and that Preferred Network Behaviour, the AMF knows the UE can be redirected to EPC.

1.	If the AMF has determined that the UE need to be redirected to the EPC, when the AMF releases the N2 connection, the AMF includes an indication of the target CN information (i.e. EPC) to the N2 release command. 

In addition, AMF may provide the RFSP Index to the RAN which allows the RAN to set proper camping policies to the UE to ensure the UE stays in EPC when it returns to Idle mode.

2.	RAN triggers RRC release procedure includes target CN indicated received in N2 procedure is conveyed to the UE.

3.	RAN sends N2 release acknowledgement to the AMF

4.	Based on the RRC indication, the UE attempts to perform the registration procedure i.e. mobility Attach or TAU procedure with “CIoT Inter CN Redirection” indication in EPC. 

If the UE finds a cell supporting connection to EPC, it sends a Attach or TAU Request to 5GC. The AS layer in the UE informs the NAS about the redirection between CN types so “CIoT Inter CN Redirection” can be included in the Attach or TAU Request.

The MME retrieves the UE context from 5GC as described in the clause 4.11 in TS 23.502 [7], if the N26 interface exists.

5.	After, the MME sends the Attach/TAU Accept message to the UE. Afterwards, based on received “CIoT Inter CN Redirection” indication the MME sets the 5GC forbidden indicator and may provide the handover restriction list to the RAN which prevents the handovers to the 5GC.

If redirection is triggered by load conditions (via O&M notification), the MME may start a redirection timer associated with the 5GC forbidden indicator. When the timer expires, the MME removes the 5GC forbidden indicator.

### 6.43.5	Impacts on existing entities and interfaces

EPC impacts:

-	If a MME determines that the UE needs to be redirected to other system, then the MME needs to create an appropriate cause and notify to the RAN in S1-AP.

-	RAN needs to provide the redirection indication in RRC to the UE.

-	When the MME receives the Attach/TAU request message with the “CIoT Inter CN Redirection” indication, the MME sets the 5GC forbidden indicator.

-	If redirection is triggered by load conditions, the MME may start a redirection timer associated with 5GC forbidden indicator. When the timer expires, the MME removes the 5GC forbidden indicator.

5GC impacts:

-	If an AMF determines that the UE needs to be redirected to the other CN type, then the AMF needs to create an appropriate cause and notify to the RAN in N2.

-	RAN needs to provide the redirection indication in RRC to the UE.

-	When the AMF receives the Registration request message with the “CIoT Inter CN Redirection” indication, the AMF sets the EPC forbidden indicator.

-	If redirection is triggered by load conditions, the AMF may start a redirection timer associated with EPC forbidden indicator. When the timer expires, the AMF removes the EPC forbidden indicator.

UE Impacts:

-	The UE needs to understand redirection indication and perform actions accordingly. The existing redirection indication in RRC Connection Release (cnType) in WB-E-UTRAN can be reused and a similar indication needs to be added for NB-IoT.

-	The AS layer informs NAS about the redirection so NAS can include “CIoT Inter CN Redirection” in the Registration, Attach or TAU Request.

NOTE:	Defining the appropriate cause value of indication is up to the Stage 3.

### 6.43.6	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

*** End of changes ***