SA WG2 Meeting #129	S2-1810693

15 - 19 October 2018, Dongguan, China

Source:	Huawei, HiSilicon

Title:	        Updates Solution 18 for Key Issue 7: NWDAF assisting Future Background Data Transfer

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes to give an evaluation for the solution 18.

1 Proposal

It is proposed to make the following changes to the Solution 18 in TR 23.791.

* * * * Start of Change * * * *

6.18	Solution 18: NWDAF assisting Future Background Data Transfer (BDT)

6.18.1	Description

This solution is for Key Issue 7: NWDAF assisting Future Background Data Transfer.

At the moment, as specified in clause 4.16.7 Negotiations for future background data transfer in TS 23.502 [3], H-PCF can provide one or more transfer policies for the future background data transfer based on requests from the 3rd party, The transfer policy is generated by the H-PCF based on e.g. analytics information, and some other information listed in TS 23.503 [4]. The transfer policy is stored in UDR, at the time the ASP indicates that transfer of background data to the UE starts. The PCF retrieves the transfer policy and enforces it.

It is considered that the network condition at the UE location could directly impact the transfer policy for future background data transfer provided to the ASP in particular:

-	UE Moving Trajectory;

-	Network performance information such as NF available capacity of 5G NF which serves the UE.

It is obvious that NWDAF could derive the UE Moving Trajectory by collecting the UE level information such as location information from AMF, Network performance information is collected from OAM.

6.18.1.1	Information to support Future Background Data Transfer

The UE level information related to background data transfer per UE is defined in Table 6.18.1.1-1.

Table 6.18.1.1-1: NWDAF input data

| Information | Presence | Source | Description |
| --- | --- | --- | --- |
| UE ID | M | AMF | Could be e.g. SUPI, which is used by NWDAF to correlate the information from AMF/OAM. |
| Location info |  |  |  |
| >Timestamp | O | AMF | The timing for the UE |
| >Location Info | O | AMF | The location info for the UE e.g. Cell ID or TA ID |
| UE Mobility pattern | O | AMF | UE mobility pattern |
| Network performance info |  |  |  |

Editor's note:	Network performance information is required for a BDT policy, we need to ask and cooperate with SA WG5 which NW performance indicators are available, e.g. NF available capacity.

6.18.1.2	Procedure for Future Background Data Transfer

Prerequisite: The PCF subscribes to or periodically requests a network condition list for an ASP or for specific UEs having a subscription with an ASP, where each network condition includes the network performance analytics and Mobility Patterns(s) analytics corresponding to a UE group belonging to the ASP or for specific UEs having a subscription with an ASP.

Table 6.18.1.2-1: EventId to provide analytics to enhance BDT functionality

| Event ID | Event Filter | Description |
| --- | --- | --- |
| Network Condition list | List of UE identities, Time, Date, Network area | As defined in Table 6.18.1.2-2 |

Table 6.18.1.2-2: Content of Network Condition List

| IE/Group Name | Presence | Range | Semantics description |
| --- | --- | --- | --- |
| Network Condition list |  | 1 |  |
| > Network Condition IEs |  | 1 to M |  |
| >>list of UE IDs | M |  |  |
| >> UE Location | M | 1 to N |  |
| >>Network Performance Info |  |  | Performance information on the NF where the UE is expected to be according to the expected moving trajectory |

Figure 6.18.1.2-1: Procedure for future BDT enhanced with Analytics

The existing procedure listed in clause 4.16.7.2 of TS 23.502 [3] applies, except:

1)	In step 1 AF request shall contain optionally UE IDs List, besides an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window and optionally, network area information (e.g. list of TAs/RAs). When the UE ID list is provided and the network area information is not provided, the NWDAF derives the expected location of the UE(s).

2)	Step 5 that needs to be updated to explain that for a Policy decision PCF takes analytics into account to generate a transfer policy. The analytics information is either subscribed to or periodically requested by the PCF for an ASP or for a number of UE and therefore have it available at the time the ASP requests for a transfer policy.

6.18.2	Impacts on Existing Nodes and Functionality

NWDAF:

- Based on the UE ID list from PCF, the NWDAF should derive the BDT analytics i.e. Network Condition list which each Network Condition contains a list of UE IDs, Expected Moving Trajectory, Network Performance Info;

- NWDAF should provide the BDT analytics i.e. Network Condition list to the PCF.

PCF:

- PCF should provide the UE ID list to the NWDAF;

- PCF should take the BDT analytics i.e. Network Condition list by NWDAF into account when generate a BDT policy.

AF: The AF should optionally provide the UE ID list which helps the NWDAF derives the expected location of the UE(s) who will transfer the background data.

6.18.3	Solution Evaluation

This solution is proposed to leverage data analytic in NWDAF to help the PCF determine accurate BDT policy.

* * * End of Change * * *