SA WG2 Meeting #129	S2-1810694

15 - 19 October 2018, Dongguan, China

Source:	Huawei, HiSilicon, Orange

Title:	        New Solution for Analytics Exposure to AF

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution proposes a solution for KI#2 on NWDAF exposing analytics to AF.

# 1 Discussion

Observation 1.

KI#2 address the issues of how AFs can request/subscribe to NWDAF analytics, receive analytics updates from NWDAF and request/receive analytics metadata information. From the three issues to be studied, the current solutions from TR 23.791, only Solution 13 addresses how AF can request and receive analytics metadata.

Observation 2.

Solution 8 (Performance improvement and supervision of mIoT terminals) defines that NWDAF sends analytics to AF, however, it is not defined in this solution, how AF subscribes to NWDAF to receive such analytics.

Proposal. This contribution proposes a new solution for analytics exposure from NWDAF to AF.

# 2 Proposal

It is proposed solution for Key issue 2 to exposure analytics information to AF.

* * * * Start of Changes * * * *

## 6.X	Solution x: Analytics Exposure to AF

### 6.X.1	Description

This solution addresses the issues of how to expose analytics information to AF as described in Key Issue #2 (Analytic Information Exposure to AF).

NOTE: what information could expose to AF is based on the specific use case discussion outcome.

#### 6.X.1.1	Procedure for AF subscription to NWDAF analytics

Figure 6.X.1.1-1: Procedure for AF subscription to NWDAF analytics

1. AF invokes the Nnwdaf_ EventSubscription Subscribe operation for subscribing to NWDAF analytics or Nnwdaf_EventSubscription Unsubscribe operation to unsubscribe for NWDAF notifications. The interaction between AF and NWDAF could also go through 5GC NF e.g. PCF.

2. NWDAF uses the Nnwdaf_ EventSubscription Notify operation to expose the analytics information that AF subscribed to receive.

NOTE: The call flow only shows a subscription-notify model for the simplicity instead of both request-response model and subscription-notification model.

### 6.X.2	Impacts on Existing Nodes and Functionality

NEF: The NEF needs to be enhanced to handle the analytics information exposure between NWDAF and AF.

### 6.X.3	Solution Evaluation

This solution is required in particular to provide NWDAF analytics exposure to AFs.

* * * End of Change * * *