SA WG2 Meeting #129	S2-1811391

15 – 19 October, Dongguan, China	(revision of S2-1811250)

Source:	Ericsson, Vodafone?, Orange, AT&T?,

Title:	Way forward for KI#3

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes interim conclusions for key issue 3.

# 1 pCR – all new text

******************** Start change  ********************

## 8.x	Interim conclusions for the service framework

Principles related to registration and discovery of services

B.1) The Rel-15 NRF is regarded as Rel-15 Service Framework

B.2) The Rel-16 Service Framework shall support the SBI of Rel-15 Service Framework for NF service registration and discovery (e.g. for backwards compatibility)

B.3) The Rel-16 Service Framework shall support in addition Rel-16 SBI for service registration and discovery that shall support registration and discovery for sets of service instances or NFs or other set concept, and other relevant parameters needed for selection, e.g. DNN

Editor’s note: depends on the outcome of which set concept will be adopted in rel-16, that is ffs

B.5)	It shall be possible to get a Rel-15 discovery result for NF services registered via Rel16 registration and vice versa.

B.6) 	It shall be possible that NF Services are discovered and selected by the Service Framework without explicit discovery request from the consumer, i.e. based on relevant metadata provided in the service request.

B.7) 	The Rel-16 Service Framework shall support routing of consumer NF service requests.

B.8)	Interoperable service registration and service discovery is achieved via the Rel-16 Service-Framework.

********************  End  change  ********************