SA WG2 Meeting #129 	S2-1811554

October 15 - 19, 2018, Dongguan, China	(revision of S2-1811438)

Source:	China Mobile

Title:	NG RAN involvement in determination of redundant transmission

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC

Abstract of the contribution: This contribution intends to propose NG RAN involvement in determination of redundant transmission for solution#3.

Discussion

In the current TR23.725 v100, it is only up to 5GC to determine whether to activate redundant transmission for the QoS flow by the 5QI. However, based on radio resource, wireless conditions or local policy, the RAN should also be able to decide whether to enable redundant transmission for the QoS flow by the specific 5QI for a URLLC service.

Therefore, this contribution intends to propose NG RAN involvement in determination of redundant transmission for solution#3.

Proposal

It is proposed to update TR 23.725 as following.

*******FIRST CHANGE*******

6.3.2.1	Activate Redundant Transmission during PDU Session Establishment procedure

The redundant transmission will be established during a PDU Session establishment procedure or a PDU Session Modification procedure.

Figure 6.3.2.1-1 depicts the establishment of redundant transmission during the PDU session establishment procedure in the non-roaming and roaming with local breakout cases.

Figure 6.3.2.1-1: Activate Redundant Transmission during PDU Session Establishment procedure

1.	Steps 1-12 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

In step 7b, the PCF indicates the SMF and the NG-RAN to activate redundant transmission for the QoS flow by the 5QI if the UE requested PDU Session Establishment procedure is initiated for a URLLC service.

In step 10, the SMF then initiates an N4 Session Establishment procedure with the selected UPF and CN Tunnel Info 1& CN Tunnel Info 2 are allocated by the SMF or UPF.

In step 11, the N2 SM information includes the QFI(s), QoS Profile(s), CN Tunnel Info 1& CN Tunnel Info 2 corresponding to each N3 tunnel.

2.	RRC Connection Reconfiguration takes place with the UE establishing the necessary NG-RAN resources related to the QoS Rules for the PDU Session request. The Secondary NG-RAN node Addition procedure is performed as defined in TS 37.340 [7].

3.	Steps 14-20 from clause 4.3.2.2.1(UE requested PDU Session Establishment) in TS 23.502 [3] are performed with the following differences:

The AN Tunnel Info provided by the NG-RAN includes AN Tunnel Info 1 and AN Tunnel Info 2. Each AN Tunnel Info includes a tunnel endpoint for each involved NG-RAN node, and the QFIs assigned to each tunnel endpoint. The M-RAN node assigns the appropriate CN tunnel information to each of the NG-RAN nodes.

The SMF provides AN Tunnel Info 1 and AN Tunnel Info 2 to the UPF as well as the corresponding forwarding rules. The forwarding rules indicates the UPF to replicate the downlink packets and send each replicate via one of DL N3 tunnels separately to NG-RAN, and eliminate the duplicate uplink packets received from the two UL N3 tunnels.

During the course of PDU Session Establishment procedure, in case N2 PDU Session Request message contains one CN Tunnel Info, based on radio resource, wireless conditions or local policy, if the NG-RAN may determine to send N2 PDU Session Failure Response to the SMF via the AMF with the cause indicating redundant transmission establishment. Considering it, the SMF may initiates redundant transmission with two CN Tunnel Info, or the SMF may report subscribed event to the PCF, it is up to the PCF to determine whether redundant transmission needs to be activated.

Note: The above mechanism may apply to PDU Session Modification and mobility procedure.

Editor's note:	whether the NG-RAN can trigger the 5GC to initiate redundant transmission is FFS.

*******END of CHANGE*******