SA WG2 Meeting #129	S2-1810303

15-19, October, 2018, Dongguan, China

Source:	Samsung, SK Telecom

Title:	Solution 4 Update: Provisioning of UE Radio Capability Information

Document for:	Discussion

Agenda Item:	6.28

Work Item / Release:	FS_RACS / Rel-16

Abstract of the contribution: It is proposed to update solution 4 for correction and clarification.

# Discussion

In this paper, following aspects are updated:

1. Correction of service operation

2. Remove option that AMF is able to directly connect to UDR, in order to keep architecture principle of 5GS.

# Proposal

## 6.4	Solution #4: Provisioning of UE Radio Capability using UDR

### 6.4.1	Introduction

The solution addresses key issue #2. It proposes that Application Function (e.g., owned by UE manufacturer, or operator) provisions to the UDR the representation of UE radio capability (e.g., UE Radio Capability ID) and corresponding UE radio capabilities. In this solution, the UE Radio Capability and corresponding UE Radio Capability ID compose the UE Radio Capability Information. The UDR can notify the UE Radio Capability Information to the AMF if the AMF subscribes change event of UE Radio Capability. Also this solution allows the AMF updates the UE Radio Capability Information to the UDR after the AMF resolves the capabilities, the identifier, and the association between them.

### 6.4.2	Functional Description

This solution has the following characteristics:

-	AF provisions UE Radio Capability ID and corresponding UE Radio Capabilities to the Network.

-	NEF exposes API for UE Radio Capability provisioning to the AF.

-	NEF updates the UE Radio Capability Information to the UDR.

-	UDR can notify the updated UE Radio Capability Information to the subscribed AMF.

-	AMF is able to associate the UE Radio Capability ID and the full UE Radio Capabilities from the UE. AMF is also able to update the UE Radio Capability Information to the UDR.

### 6.4.3	Procedures

#### 6.4.3.1	Provisioning UE Radio Capability Information to UDR

In this clause, the procedure for provisioning of UE Radio Capability is described.

Figure 6.4.3-1: Provisioning of UE Radio Capability information to UDR

Editor's Note:	It is FFS whether to use existing service operation for this procedure.

1.	The AMF triggers Nnef_DM_Subscribe service operation to the NEF in order to subscribe notification of the UE Radio Capability Information from the UDR. The UE Radio Capability Information is composed of the UE Radio Capability and corresponding UE Radio Capability Identifier. In the Nnef_DM_Subscribe service operation, the AMF indicates the type of data is for UE Radio Capability Information and includes its AMF ID to be identified by the UDR.

NOTE: In order to synchronize UE Radio Capability Information among AMFs in the PLMN, AMF should request subscription to UDR for the UE Radio Capability Information when the AMF is initiated.

2.	The NEF triggers Nudr_DM_Subscribe service operation based on the information received in the step 1. The UDR performs authorization whether the AMF can subscribe notification of the UE Radio Capability Information. If authorized, the UDR stores received subscription request. If the UDR detects the AMF hasn't been notified any UE Radio Capability Information, the UDR may decide to provision all the UE Radio Capability Information stored in the UDR. In this case, the UDR can notify to the AMF with UE Radio Capability Information as in the step 8.

3.	If there is new type of UE with different UE Radio Capabilities combination, the AF invokes Nnef_RadioCapability_Provisioning Request service operation to the NEF. The AF includes UE Radio Capability Information in this message. The AF can include set of UE Radio Capability Information if there are various type of UEs to provision UE Radio Capability Information.

In the case of architecture without CAPIF support, the AF is locally configured with the API termination points for this service operation. In case of architecture with CAPIF support, the AF obtains the service API information from the CAPIF core function as specified in TS 23.222 [4].

4.	The NEF authorized the AF request based on the operator policy.

5.	The NEF acknowledges the execution of Nnef_RadioCapability_Provisioning_request..

6.	The NEF invokes Nudr_DM_Update to the UDR. In this message, NEF includes the received UE Radio Capability Information from the step 3.

7.	The UDR updates the UE Radio Capability. The UDR sends Nudr_DM_Update response to the NEF.

8.	The UDR detects that the related subscription for notification of the updated UE Radio Capability Information. The UDR notifies to the NEF of the update UE Radio Capability Information by invoking Nudr_DM_Notify service operation. In this message, the UDR includes the update UE Radio Capability Information, and the AMF ID which needs to be notified.

9.	The NEF triggers Nnef_DM_Notify service operation based on the information received in the step 8.

#### 6.4.3.2	Update and sharing of UE Radio Capability Information using UDR

In this clause, the procedure for update and sharing of UE Radio Capability is described.

Figure 6.4.3-2: Update of UE Radio Capability information to UDR and sharing to other AMF

1.	The UE performs registration procedure. If the UE is configured with the UE Radio Capability IDs, it provides it in initial access stratum signalling. The UE Radio Capability ID is conveyed to the AMF in an N2 message.

Editor's note:	Whether the UE sends the UE Radio Capability ID in NAS message depends on other solution.

2.	The AMF determines there is association with the UE Radio Capability ID received in the step 1 in order to retrieve the explicit set of the UE Radio Capabilities. If the AMF cannot figure out its explicit set of the UE Radio Capabilities corresponding to the UE Radio Capability ID, the AMF decides to perform UE Capability Match procedure to the NG-RAN.

3.	The AMF performs UE Capability Match procedure to the NG-RAN to retrieve explicit set of UE Radio Capabilities for the UE.

4.	If the NG-RAN doesn't have the UE Radio Capability for the UE, the NG-RAN performs RRC procedure to retrieve the explicit set of UE Radio Capabilities. If the NG-RAN has the UE Radio Capability for the UE, the NG-RAN performs step 5.

5.	The NG-RAN provides the UE Radio Capabilities to the AMF. The AMF associates the UE Radio Capabilities to the UE Radio Capability ID received from the UE. The AMF stores this information.

6.	The AMF performs rest of registration procedure.

7.	The AMF invokes Nnef_DM_Update to the NEF. In this message, AMF includes the stored UE Radio Capability Information in the step 5. The NEF invokes Nudr_DM_Update service operation based on the received information from the AMF. The UDR updates the received UE Radio Capability Information. The UDR sends Nudr_DM_Update response.

8.	The UDR detects that AMF 2 has subscribed for notification of the updated UE Radio Capability Information. The UDR notifies to the AMF 2 of the update UE Radio Capability Information by invoking Nudr_DM_Notify service operation to the NEF. In this message, the UDR includes the update UE Radio Capability Information. The NEF invokes Nnef_DM_Notify service operation to the AMF based on the received information from the UDR.

NOTE: It is assumed that AMFs in the PLMN have subscribed the notification of update of UE Radio Capability Information to UDR, in order to synchronize updated UE Radio Capability Information.

### 6.4.4	Impacts on existing entities and interfaces

Editor's note:	This clause describes impacts to existing entities and interfaces.

NEF: New API exposure to AF for UE Radio Capability Information provisioning. New Nnef service operation to AMF for subscription and update of the UE Radio Capability Information.

AMF: Use Nnef service for the UE Radio Capability Information Subscription and Update.

The AMF is able to associate unknown UE Radio Capability ID received from the UE with explicit set of the UE Radio Capabilities.

UDR: UDR supports subscription/notification and update of the UE Radio Capability information.

### 6.4.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.