# SA WG2 Meeting #S2-129	S2-1810795

# 15 – 19 October, 2018, Dongguan, P. R. China

Source:	ORANGE

Title:	Evaluation of Solution #10

Document for:	Approval

Agenda Item:	6.20

Work Item / Release:	FS_5G_URLLC

Abstract of the contribution: This contribution proposes an evaluation of Solution #10.

# Proposal

**** Start of change (all new text) ****

### 6.10.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

- The solution can provide disjoint redundant user plane paths through the 3GPP system including UE, RAN and CN.

- The solution ensures that the redundant user plane paths do not share the same risks, whenever possible.

- The solution can be well integrated with end to end redundancy solutions.

- The solution can provide the same level of redundancy as typically applied for fixed industrial deployments today.

- The operator has the means to deploy the solution in its own network by also deploying Solution #10 in combination with e.g., IEEE TSN FRER or other redundancy solutions in case such redundancy solutions are not available in an external data network.

- The solution is under control of the terminals.

- The solution extends the dual connectivity by a CN trigger to request dual connectivity setup on a per session basis.

- The solution uses multiple UEs in the terminal, hence it provides redundant UEs.

- The solution enables a flexible deployment as it can be deployed by defining NRGs based on the already deployed network without requiring the deployment of fully disjoint RANs or CNs.

**** End of Changes ****