SA WG2 Meeting #129	S2-1811114

15 - 19 October 2018, Dongguan, China	(revision of S2-1810696)

Source:	Huawei, HiSilicon, Orange, Nokia

Title:	Updates Solution 12

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	FS_eNA

Abstract of the contribution: This contribution provides the update in the description, impacts and evaluation sections of solution 12.

1 Discussion

The following updates to the existent solution are proposed.

| Topic | Reason | Proposal |
| --- | --- | --- |
| Clarify text | Current text of Section 6.12.1.1 is ambiguous regarding who exactly request the data collection (NWDAF or OAM) | - Update of text from Section 6.12.1.1 clarifying that NWDAF has access to information from OAM, and that data collection itself it an OAM task. |
| Align references to SA5 | SA5 did a restructuring of their specifications (to reduce the number of specs) and the text of solution 12 needs to be updated to reflect SA5 changes | - Removal and inclusion of SA5 specifications |
| Updates on Procedure | Current Figure 6.12.1.2-1 and description of the text are ambiguous on the role of OAM and NWDAF regarding the actual data collection from measurement jobs. | - Update steps of the procedure in Figure 6.12.1.2-1 - Update text describing the steps of the procedure in Figure 6.12.1.2-1. |
| Impacts and Evaluation Section | 3GPP default Editor’s note signalling the missing parts | - Addition of the Impact and Evaluation sections |

2 Proposal

It is proposed to make the following changes to the TR 23.791.

* * * * Start of Changes * * * *

6.12	Solution 12: Data Collection from OAM using the existing SA5 services

6.12.1	Description

This solution is for Data Collection from OAM associated with Key Issue 4: Interactions with OAM for Data Collection and Data Analytics Exposure.

6.12.1.1	General

Based on current definitions from SA5, NWDAF shall have access to the following information provided by OAM :

- NR and NG RAN or 5GC performance measurements as defined in TS 28.552[15]

- 5G End to end KPIs as defined in TS 28.554[17]

NWDAF shall use the following SA5 services to have access to the information provided by OAM:

- Generic performance assurance and fault supervision management services as defined in TS 28.532.

- PM (Performance Management) services as defined in TS 28.550[13]

- FS (Fault Supervision) services defined in TS 28.545[18]

OAM shall be responsible for performing the configuration and data collection from job measurements.

6.12.1.2	Procedure for the support of Data Collection from OAM

The interactions between NWDAF and OAM for data collection are illustrated in Figure 6.12.1.2-1. The data collected depends on the use cases.

OAM shall setup the proper mechanism to guarantee the continuous data collection requested by NWDAF

NOTE: Coordination with SA5 is required to clarify if measurements and KPIs required by NWDAF (as defined in the solutions for the key issues) can be provided.

Figure 6.12.1.2-1: Data collection from OAM services

- NWDAF subscribes to OAM services for receiving information from one of OAM services. The subscription request shall optionally indicate to OAM the need to receive UP network slice instance topology information relevant to the analysis of the requested information

NOTE: SA5 does not restrict the consumers of the 5G management services and therefore NWDAF invokes the existing SA5 service to retrieve the management data.

Editor’s Note: It is FFS if NWDAF can use the existing service to subscribe for data collection.

- OAM perform the required configuration in order to provide the information requested by NWDAF subscription. It is up to the OAM to identify if already deployed/configured support for data collection related to the subscription request will be reused for each request from NWDAF or if new support for data collection of information associated with the subscription needs to be deployed/configured.

- OAM perform the tasks, e.g., data collection, data processing, associated with the subscribed request from NWDAF.

- OAM services send notification to NWDAF about subscribed information.

NOTE: The call flow in Figure 6.12.1.2-1 only shows a subscribe/notify model for the simplicity, however both request-response and subscription-notification models should be supported.

6.12.2	Impacts on Existing Nodes and Functionality

NWDAF is to invoke the current SA5 defined service(s) for data collection

.

6.12.3	Solution Evaluation

This solution defines the framework for data collection from OAM by NWDAF and the detailed parameters of OAM data will be defined in other solutions.

This solution reuses the current SA5 defined service(s) for data collection and NWDAF is to invoke the current SA5 defined service(s) to collect data as SA5 does not restrict the consumers of the 5G management services.

* * * End of Change * * *