SA WG2 Meeting #129	S2-1811186

October 15 - 19, 2018, Dongguan, P. R. China	was S2-181354

Source:	CATT

Title:	Solution for Local LCS architecture

Document for:	Discussion and approval

Agenda Item:	6.13

Work Item / Release:	FS_eLCS / Rel-16

Abstract of the contribution:

This contribution proposes a new solution for local LCS architecture, mainly to address KI#3 and KI#1.

1. Introduction

This contribution proposes a new solution for local LCS architecture and provides the function description and signalling procedures.

///////////////////////////////////////////////////////////////// Start of change /////////////////////////////////////////////////////////////////////////////

6.xx	Solution xx: Local LCS architecture

6.xx.1	Introduction

This solution addresses the Key Issue#1 "Enhancement to LCS architecture" and Key Issue#3 "support of low latency LCS", and proposes local LMF architecture in the NG-RAN, additionally, the impact to the 5GC and NG-RAN are discussed.

6.xx.2	Functional Description

The solution includes Local LMF capabilities in the NG-RAN, as shown in Figure 6.xx.2-1.

# Figure 6.xx.2-1: Local LCS architecture

The NG-RAN which contained local LMF capability includes the following functionalities:

-	Supports location determination for a UE served by the NG RAN contains Local LMF capability.

-	Obtains downlink location measurements or a location estimate from the UE.

-	Obtains uplink location measurements.

-	Obtains non-UE associated assistance data from the neighbour NG RAN(s).

-	Dynamic coordination of positioning resources of the neighbour NG RAN(s).

-	Communication with LMF, AMF and other 5GC LCS function entities.

6.xx.3	Procedures

6.xx.3.1 Local LMF registration

The purpose of the Local LMF capability registration procedure is to register the Local LMF to the NRF , as shown in Figure 6.xx.3-1.

Figure 6.xx.3-1 Procedure for Local LMF registration

- The NG-RAN node issues a N2 transport message to the AMF, which may be a NG setup request or RAN congifuration update message, passing the registration related information of the Local LMF, e.g, the Local LMF support indicator or the capability information of the Local LMF, etc.

- According to the received registration related information, the AMF may generate and send the Local LMF registration request message towards the NRF, which contains the RAN identity and the capability information of the Local LMF, etc.

Editor's NOTE: whether AMF contacts NRF is FFS;

Editor's NOTE: whether and how NRF store the Local LMF capability of NG-RAN is FFS.

- The NRF determines if the registration can be accepted based on the operator's strategy and stores the above registration related information.

- If the registration is accepted, the NRF returns the Local LMF registration ack message to the AMF.

- The AMF returns a N2 transport message to the corresponding NG-RAN node, which may be a NG setup response or RAN congifuration update acknowledge message.

6.xx.3.2 AMF based Local LMF selection

The AMF based Local LMF selection procedure is used when a location service is requested and the serving NG-RAN has the Local LMF capability, as shown in Figure 6.xx.3-2

Figure 6.xx.3-2 Procedure for AMF based Local LMF selection

- The AMF receives the location service request Message from the GMLC or UE.

- The AMF may invoke the Nnrf_NFDiscovery_Request service operation from the NRF to find a suitable LMF to continue the location request procedure.

- The NRF replies with the list of potential LMF(s) and their service capability information. The list contains LLMFs and LMFs. Based on LMF selection factors in section 6.14.3.0, the AMF may chose to use the LLMF within current serving NG-RAN of the target UE to continue the location request..

- The AMF communicates the Location Determine Request to the Local LMF.

- The local LMF initiates location operations with the gNB/UE to obtain positioning measurements, the positioning signalling between the Local LMF and the gNB/UE in this solution is to be defined by RAN.

- The Local LMF computes the position estimate and returns it to the AMF, via Location Determine Response.

- The AMF forwards it to the requesting entity (e.g. UE or GMLC).

# If the local LMF related information is stored in the serving AMF, the above step 2 and 3 should be omitted. The AMF can discover a suitable Local LMF serving this UE by itself.

# Editors' note:	The details of communication between AMF and Local LMF are FFS and needs to be further studied in cooperation with RAN. E.g. in step 4 and 6, the AMF and LLMF may exchange messages directly or via NG-RAN

6.xx.4	Impacts on existing entities and interfaces

The NG-RAN supports positioning capability, e.g, location measurement and location calculation.

6.xx.5	Evaluation

Since the LMF related capabilities are supported in NG-RAN, the signaling transmission process via 5GC can be avoided, the solution can greatly improve the latency performance of the location service. Especially for the location service with stringent latency requirement, compared with the legacy LCS architecture, this solution can obtain the maximum gain. In addition, through the dynamic coordination of position resources, the solution can further improve the utilization rate of radio resources.

///////////////////////////////////////////////////////////////// End of change /////////////////////////////////////////////////////////////////////////////