SA WG2 Meeting #129	S2-1811391

15 – 19 October, Dongguan, China	(revision of S2-1811250)

Source:	Ericsson, Vodafone?, Orange, AT&T?,

Title:	Way forward for KI#3

Document for:	Approval

Agenda Item:	6.19

Work Item / Release:	FS_eSBA / Rel-16

Abstract of the contribution: This contribution proposes interim conclusions for key issue 3.

# 1 pCR

## 8.x	Interim conclusions for the service framework

Principles related to registration and discovery of services

B.1)	The Rel-15 NRF SBIs are regarded as version 1 for NF service registration and discovery

B.2)	Rel-16 will include the version 1 SBIs of NF service registration and discovery.

B.4)	There may be a version 2 service operations related to registration and discovery which include new constructs such as sets of service instances or NFs, , and other relevant parameters needed for selection, e.g. DNN

Editor’s note: depending on the architecture constructs adopted in rel-16, whether it is NF/NF Service/Service, the service framework version needs to be adapted but the actual choice is FFS.

B.5)	It shall be possible to get a version 1 discovery result for services registered via version 2 registration and vice versa.

B.6)	Interoperable service registration and service discovery is achieved via the NRFand/or SBI

Principles related to roaming:

E.1)	SEPP defined in R15 shall be supported to enable inter-PLMN service discovery operations between PLMNs

E.2)	SEPP defined in R15 shall be supported for communication between service consumers in VPLMN and service producers in HPLMN.

********************  End  change  ********************