3GPP TSG-SA WG2 Meeting #130	S2-1900179

Kochi, India, 21 - 25 January, 2019	(revision of S2-19XXXXX)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 0890 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-Access PDU Session Establishment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  |  | Date: |  | 2019-01-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is concluded that Multi-Access PDU Session Establishment procedure is based the procedure specified in TR 23.793 clause 6.2.3.3 "UE Requested MA-PDU Session Establishment". Based on the procedure, it is proposed to add a new procedure for Multi-Access PDU Session Establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure for Multi-Access PDU Session Establishment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ATSSS functionality cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * * All New Text

4.3.2.X	UE Requested MA-PDU Session Establishment

This procedure is used in case of non-roaming and roaming with local breakout scenarios. The UE may use this procedure when the UE is registered via single access only (i.e. 3GPP access or non-3GPP access) or when the UE is registered to the same PLMN via both accesses.

NOTE 1:	This procedures assumes that the UE is registered via 3GPP access. If the UE is registered via non-3GPP access only, the UE sends a PDU Session Establishment request message over non-3GPP access.

Figure 4.3.2.X-1: UE Requested Multi-Access PDU Session Establishment for non-roaming and roaming with local breakout

1.	Step 1 from clause 4.3.2.2.1 with the following modifications and clarifications:

When the UE wants to establish a MA-PDU Session,

-	the UE sets Request Type to "initial request", and;

-	in the NAS message, the UE includes MA-PDU Request indication.

-	the UE includes ATSSS and/or MPTCP Capability in the PDU Session Establishment request message.

NOTE 2:	If the UE is registered via both access and wants to establish a MA-PDU Session, the UE is expected to send PDU Session Establishment request message over 3GPP access to avoid paging over 3GPP access.

2.	Step 2 from clause 4.3.2.2.1 with the following modifications and clarifications:

-	The AMF sets Access Type to "Multi-Access" and stores it.

-	If MA-PDU Request indication is included in the NAS message, the AMF selects an SMF supports ATSSS functionality.

3.	Step 3 from clause 4.3.2.2.1 with the following modifications and clarifications:

-	If the AMF determines that the UE is registered via both accesses but the requested S-NSSAI is not allowed on both accesses, then the AMF shall reject the Multi-Access PDU Session establishment.

-	The AMF determines Access Type in Nsmf_PDUSession_CreateSMContext Request to the access the NAS message is received over.

-	If the AMF supports ATSSS functionality and MA-PDU Request indication is included in the NAS message, the AMF includes MA-PDU Request indication and RM states of each access in Nsmf_PDUSession_CreateSMContext Request.

4-6.	Steps 4-6 from clause 4.3.2.2.1.

7.	Steps 7a and 7b from clause 4.3.2.2.1 with the following modifications and clarifications:

-	If MA-PDU Request indication is included in the NAS message, the SMF selects and PCF support ATSSS functionality.

-	If the UE included ATSSS Capability and/or MPTCP Capability the SMF may accept the PDU Session Establishment request based on MA-PDU Request indication from the AMF and operator policy. The SMF may accept the PDU Session Establishment request as a sigle-access PDU Session.

-	If dynamic PCC is deployed and if both the AMF and the SMF support MA-PDU sessions, then the SMF sends an MA-PDU Request indication and associated access information to PCF in the SM Policy Control Create message. The PCF decides whether the MA-PDU session is allowed or not based on operator policy and subscription.

-	The PCF provides PCC rules related with the MA-PDU session during the Policy Association Establishment.

-	From the received PCC rules, the SMF derives (a) ATSSS rules, which will be sent to UE for controlling the traffic steering, switching and splitting in the uplink direction, and (b) N4 rules, which will be sent to UPF for controlling the traffic steering, switching and splitting in the downlink direction.

8.	Step 8 from clause 4.3.2.2.1 with the following modifications and clarifications:

-	The SMF selects an UPF supports ATSSS Capability and/or MPTCP Capability according to the indicated UE capability.

9.	Step 9  from clause 4.3.2.2.1.

10-17. Steps 10-17 from clause 4.3.2.2.1 with the following modifications and clarifications:

-	The SMF initiates User Plane resource setup over the target access (e.g. non-3GPP) different from the access the PDU Session Establishment request is received (e.g. 3GPP) if the UE is registered over the target access. If the UE is not registered over the target access, the SMF skips these stpes.

-	The SMF includes Access Type in Namf_Communication_N1N2MessageTransfer to indicate the target access of N2 SM information. The SMF does not include N1 SM Container (PDU Session Establishment Accept).

18-25. Steps 10-17 from clause 4.3.2.2.1 with the following modifications and clarifications:

-	The SMF initiates User Plane resource setup over the access the PDU Session Establishment request is received.

-	The SMF includes Access Type in Namf_Communication_N1N2MessageTransfer to indicate the target access of N2 SM information. If the SMF accepts the PDU Session establishment, the SMF sends N1 SM Container (PDU Session Establishment Accept) with ATSSS Capability and/or MPTCP Capability indication and provides ATSSS rules.

26-28. Steps 18 and 20 from clause 4.3.2.2.1.