3GPP TSG-SA WG2 Meeting #130	S2-1900180

Kochi, India, 21 - 25 January, 2019	(revision of S2-19XXXXX)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0770 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS for Multi-Access PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  |  | Date: |  | 2019-01-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is concluded that Multi-Access PDU Session Establishment procedure supports non-GBR and GBR QoS Flows as descirbe in TR 23.793. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a description on QoS support for Multi-Access PDU Session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MA-PDU Session does not support QoS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * * All New Text

5.X.Y	QoS of Multi-Access PDU Session

The 5G QoS model for the Single-Access PDU Session is also applied to the Multi-Access PDU Session, i.e. the QoS Flow is the finest granularity of QoS differentiation in the Multi-Access PDU Session. The QoS Flow is not associated with specific access, i.e. access agnostic. When a data flow is steered between two accesses, the same QoS Flow is used. The SMF shall provide the same QFI in 3GPP and non-3GPP accesses so that the same QoS is supported when a data flow is switched between both accesses.

A QoS Flow of the Multi-Access PDU Session may either Non-GBR or GBR depending on its QoS profile. For the Non-GBR QoS Flow, the SMF provides QoS profile in both accesses if the UE is in CM-CONNECTED in both accesses. For the GBR QoS Flow, the SMF may provide QoS profile to the single access as follows:

-	If the PCC rule allows a GBR QoS Flow in a single access, the SMF provides QoS profile for the GBR QoS Flow over the access allowed by the PCC rule.

-	If the PCC rule allows a GBR QoS Flow in both accesses, the SMF decides which access to provide QoS profile for the GBR QoS Flow based on its local policy.

When the Multi-Access PDU Session is established, the SMF provides QoS rule(s) to the UE. The UE performs the classification and marking of UL User Plane traffic, i.e. the association of UL traffic to QoS Flows, based on QoS rule(s). The QoS rule(s) are commonly used for both 3GPP and non-3GPP access, so the QoS classification is independent of ATSSS steering rules.

In this release of the specification, Notification control is not supported for the Multi-Access PDU Session.

* * * * End of Changes * * * *