3GPP TSG-SA WG2 Meeting #130	S2-1900462

# January 21 - 25, 2019, Kochi, India

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0852 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of MA-PDU session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSlicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  |  | Date: |  | 2018-12-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the TR 23.793 conclusion, introduces the MA-PDU session definition in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines the MA-PDU session, including the MPTCP function, ATSSS function and link performance measurement function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MA-PDU session cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	IETF RFC 1027: "Using ARP to Implement Transparent Subnet Gateways".

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.274: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[x]	IETF RFC 6824: "TCP Extensions for Multipath Operation with Multiple Addresses".

[y]	IETF RFC 1928: "SOCKS Protocol Version 5".

[z]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

* * * * Next Change * * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Access Network: An access network comprising a NG-RAN and/or non-3GPP AN connecting to a 5G Core Network.

5G Core Network: The core network specified in the present document. It connects to a 5G Access Network.

5G QoS Flow: The finest granularity for QoS forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.). Providing different QoS forwarding treatment requires separate 5G QoS Flow.

5G QoS Identifier: A scalar that is used as a reference to a specific QoS forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a 5G QoS Flow. This may be implemented in the access network by the 5QI referencing node specific parameters that control the QoS forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Access Traffic Steering: Selecting an access network for a new data flow and transfers the traffic of this data flow over the selected access network. Access traffic steering is applicable to 3GPP and non-3GPP accesses.

Access Traffic Switching: Moving all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow. Access traffic switching is applicable between 3GPP and non-3GPP accesses.

Access Traffic Splitting: Splitting the traffic of a data flow across multiple access networks. When traffic splitting is applied to a data flow, some traffic of the data flow is transferred via one access and some other traffic of the same data flow is transferred via another access.

Access traffic splitting is applicable between 3GPP and non-3GPP accesses.

Allowed NSSAI: NSSAI provided by the Serving PLMN during e.g. a Registration procedure, indicating the S-NSSAIs values the UE could use in the Serving PLMN for the current registration area.

Allowed Area: Area where the UE is allowed to initiate communication as specified in clause 5.3.2.3.

AMF Region: An AMF Region consists of one or multiple AMF Sets.

AMF Set: An AMF Set consists of some AMFs that serve a given area and Network Slice(s). AMF Set is unique within an AMF Region and it comprises of AMFs that support the same Network Slice(s). Multiple AMF Sets may be defined per AMF Region.

Application identifier: An identifier that can be mapped to a specific application traffic detection rule.

AUSF Group ID: This refers to one or more AUSF instances managing a specific set of SUPIs.

Configured NSSAI: NSSAI provisioned in the UE applicable to one or more PLMNs.

DN Access Identifier (DNAI): Identifier of a user plane access to one or more DN(s) where applications are deployed.

Emergency Registered: A UE is considered Emergency Registered over an Access Type in a PLMN when in limited service state and registered for emergency services only over this Access Type in this PLMN.

Endpoint Address: An address used by a NF service consumer to access the NF service (i.e. to invoke service operations) provided by a NF service provider. An Endpoint Address is represented in the syntax of Uniform Resource Identifier (e.g. part of Resource URI of the NF service API).

Expected UE Behaviour: Set of parameters provisioned by an external party to 5G network functions on the foreseen or expected UE behaviour, see clause 5.20.

Forbidden Area: An area where the UE is not allowed to initiate communication as specified in clause 5.3.2.3.

GBR QoS Flow: A QoS Flow using the GBR resource type or the Delay-critical GBR resource type and requiring guaranteed flow bit rate.

Initial Registration: UE registration in RM-DEREGISTERED state as specified in clause 5.3.2.

Local Area Data Network: a DN that is accessible by the UE only in specific locations, that provides connectivity to a specific DNN, and whose availability is provided to the UE.

Local Break Out (LBO): Roaming scenario for a PDU Session where the PDU Session Anchor and its controlling SMF are located in the serving PLMN (VPLMN).

Mobility Pattern: Network concept of determining within the AMF the UE mobility parameters as specified in clause 5.3.2.4.

Mobility Registration Update: UE re-registration when entering new TA outside the TAI List as specified in clause 5.3.2.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

Multi-Access PDU Session: A PDU session with multi-access, traffic of which can be sent over 3GPP access, or over non-3GPP access, or over both accesses.

NGAP UE association: The logical per UE association between a 5G-AN node and an AMF.

NGAP UE-TNLA-binding: The binding between a NGAP UE association and a specific TNL association for a given UE.

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 2:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Network Instance: Information identifying a domain. Used by the UPF for traffic detection and routing.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

Network Slice instance: A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice.

Non-GBR QoS Flow: A QoS Flow using the Non-GBR resource type and not requiring guaranteed flow bit rate.

NSI ID: an identifier for a Network Slice instance.

NF instance: an identifiable instance of the NF.

NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service instance: an identifiable instance of the NF service.

NF service operation: An elementary unit a NF service is composed of.

NG-RAN: A radio access network that supports one or more of the following options with the common characteristics that it connects to 5GC:

1)	Standalone New Radio.

2)	New Radio is the anchor with E-UTRA extensions.

3)	Standalone E-UTRA.

4)	E-UTRA is the anchor with New Radio extensions.

Non-Allowed area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3.

Non-Seamless Non-3GPP offload: The offload of user plane traffic via non-3GPP access without traversing either N3IWF or UPF.

PDU Connectivity Service: A service that provides exchange of PDUs between a UE and a Data Network.

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service.

PDU Session Type: The type of PDU Session which can be IPv4, IPv6, IPv4v6, Ethernet or Unstructured.

Periodic Registration Update: UE re-registration at expiry of periodic registration timer as specified in clause 5.3.2.

(Radio) Access Network: See 5G Access Network.

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Routing Indicator: Indicator that allows together with SUCI/SUPI Home Network Identifier to route network signalling to AUSF and UDM instances capable to serve the subscriber.

Service based interface: It represents how a set of services is provided/exposed by a given NF.

Service Continuity: The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.

Service Data Flow Filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet (IP or Ethernet) flows constituting a Service Data Flow.

Service Data Flow Template: The set of Service Data Flow filters in a policy rule or an application identifier in a policy rule referring to an application detection filter, required for defining a Service Data Flow.

Session Continuity: The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.

Subscribed S-NSSAI: S-NSSAI based on subscriber information, which a UE is subscribed to use in a PLMN

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs.

UDR Group ID: This refers to one or more UDR instances managing a specific set of SUPIs.

UPF Service Area: The area within which PDU Session associated with the UPF can be served by (R)AN nodes via a N3 interface between the (R)AN and the UPF without need to add a new UPF in between or to remove/re-allocate the UPF.

Uplink Classifier: UPF functionality that aims at diverting Uplink traffic, based on filter rules provided by SMF, towards Data Network.

* * * * Next Change * * * *

5.6.x	Multi-Access PDU session

5.6.x.1	General

The 5GC su pports a PDU Session with multi-access, i.e. Multi-Access PDU session (MA-PDU session). The traffic transported via the Multi-Access PDU session can be steered, switched, or split between the 3GPP and non-3GPP accesses by the enforcement of the ATSSS rule.  The MA-PDU session supports any type of access, including Untrusted non-3GPP access, Trusted non-3GPP access, Wireline 5G Access, Wireline 5G Cable Access, etc., as long as a MA-PDU session can be established using this type of access.

The establishment of the MA-PDU session is triggered by the UE when the UE is registered to the same PLMN via both 3GPP and non-3GPP accesses, or when the UE is registered via one access only. At any given time, a MA-PDU session may have user-plane resources on both 3GPP and non-3GPP accesses, or on one access only, or may not have user-plane resources on any access. The MA-PDU Session is supported only in non-roaming and LBO scenario. The MA-PDU session establishment procedure is defined in 3GPP TS 23.502 [3] subclause 4.3.2.

The Access Traffic Steering, Switching, and Splitting (ATSSS) for the MA-PDU session is performed by the MPTCP function as defined in subclasue 5.6.x.2 or ATSSS function as defined in sbuclasue 5.6.x.3. If MPTCP is enabled, it is possible to steer the MPTCP flows by applying the MPTCP function and, simultaneously, to steer all other flows by using a lower-layer steering functionality, called the "ATSSS function". If the MA-PDU session is supported by the network, both the MPTCP proxy function and ATSSS function shall be supported by the network. For the same N4 Session in the UPF, if MPTCP proxy functionality is enabled by the indication from SMF, the UPF shall steer the MPTCP flows by using the MPTCP protocol and, simultaneously, steer other flows without MPTCP, as defined for the "ATSSS function". It shall also be possible for the UE and the network to support the link performance measurement over both accesses as defined in subclause 5.6.x.4.

The same set of ATSSS rules as defined in 3GPP TS 23.503 [45] is applied for steering decisions by the MPTCP function and by the ATSSS function respectively.

5.6.x.2	MPTCP function

For the TCP traffic, the ATSSS can be supported by the MPTCP function in the UE side and the MPTCP proxy functionality in the network side. In the UE, the MPTCP function is defined as the higher layer steering function implemented by the Operation System. In the network, the MPTCP proxy functionality is located in the UPF.

During the MA-PDU session establishment, if the UE is capable to use MPTCP for traffic steering, the UE provides an "MPTCP Capability" indication. If the MA-PDU session is supported by the network, the network allocates to UE one IP address/prefix for the MA-PDU session and two additional IP addresses, called "link-specific multipath" addresses. Additionally, the network shall send MPTCP proxy information to UE, i.e. the IP address(es), a port number and the type of the MPTCP proxy (e.g. SOCKS5 (see RFC 1928 [y])). The IP address/prefix for the MA-PDU session is associated with both 3GPP access and non-3GPP access, and one "link-specific multipath" address is associated with 3GPP access and another "link-specific multipath" address is associated with the non-3GPP access.  These two "link-specific multipath" addresses are used only by the MPTCP function and may not be routable via N6. To be specific, the MPTCP function in the UE shall use "link-specific multipath" addresses to establish MPTCP sub-flows to the MPTCP proxy over non-3GPP access and over 3GPP access as defined in the RFC 6824 [x], and the MPTCP proxy functionality in the UPF may use the IP address/prefix for the MA-PDU session to establish the TCP connection to the application server in a data network. The application server IP address is obtained by the MPTCP proxy from UE side as defined in the RFC 1928 [y].

The network may indicate to UE the list of applications for which MPTCP should be applied. This is achieved by using the Steering Function component of an ATSSS rule as defined in TS 23.503 [45], which indicates to the UE whether MPTCP should be applied for steering the traffic of one or more applications.

When the UE requests a MA-PDU session and indicates it is capable to support the MPTCP function only, the link performance measurements are taken by the MPTCP protocol in the UE and the MPTCP proxy.

5.6.x.3	ATSSS function

For any type of traffic (e.g. IP traffic, or Ethernet traffic), the ATSSS can be supported by the ATSSS function in the UE side and the network side. In the UE, the ATSSS function is defined as the lower layer steering functions compared with the higher layer steering MPTCP function. In the network, the ATSSS function in UPF is based on the N4 rules supporting ATSSS provided by SMF (PDR etc).

During the MA-PDU session establishment, if the UE is capable to use ATSSS function for traffic steering, the UE provides an "ATSSS Capability" indication. If the MA-PDU session is supported by the network, the network allocates to the UE one IP address/prefix for the MA-PDU session. The IP address/prefix for the MA-PDU session is associated with both 3GPP access and non-3GPP access.

If ATSSS function is used in the MA-PDU session for some data traffic, the UE and the UPF apply the link performance measurement taken by the Link performance measurement function as defined in subclause 5.6.x.4.

The UPF internal handling of the packet flows in an MA-PDU Session is based on the packet processing model described in the TS 29.244 [z].  For the downlink traffic, the N3 UPF selects the access type based on the MAR rules provided by the SMF and routes the downlink data packets to the N3 tunnel associated with the selected access type.

Editor's note:	The implementation of traffic switching and splitting for the UDP and Ethernet data packets based on the IETF protocol is FFS.

5.6.x.4	Link performance measurement function

If the ATSSS function is used in the MA-PDU session for some data traffic, the RTT measurement over both accesses (i.e. 3GPP and Non-3GPP) is performed by the Link performance function in the UE side and UPF side.  Such measurements may be required only under certain conditions, e.g. only when the ATSSS rule is "Smallest Delay" steering mode.  Based on the measurement result, the ATSSS function of UE side or/and UPF side may adjust the enforcement of the traffic steering, switching and splitting policies.

The SMF provides the link performance measurement rules to the UE or/and the UPF during the PDU Session Establish/Modification procedure to establish the link performance measurement association between the Link performance function in the UE side and UPF side. The measurement rule may contain the measurement parameters and the measurement period of each access to assist the Link performance function of UE side and UPF side to initiate link performance measurement over both accesses. The SMF also sends the IP address and port number of Link performance function of UPF to the UE during the PDU Session Establish/Modification procedure.

The measurement messages are transported between the Link performance function of UE side and UPF side via the user plane over each access. The UE or the UPF send(s) the measurement request messages based on the received measurement rule, and the opposite node sends the measurement response messages after receiving the measurement request messages.  The Link performance function of UE side or/and UPF side could calculate the RTT based on the time stamp of the point when the measurement request messages are sent and the time stamp of the point when the measurement response messages are received. The Link performance function of UE side or/and UPF side should be able to correlate the measurement messages with the corresponding access type in order to get the accurate measurement result for each access.

Editor's note:	Whether other link performance parameters than RTT, e.g. jitter and/or packet loss ratio for GBR QoS Flow, are to be measured is FFS.

* * * * End of Change * * * *