SA WG2 Meeting #130	S2-1901200

January 21 - 25, 2019, Kochi, India	rev of S2-1900649

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.725 | CR | 0017 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Solution #16 for automated GBR service recovery after handover into congested cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_URLLC |  |  |  |  |  |  | Date: |  | 2019-01-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1 Several Editor’s Notes are present in Solution #17. With regard to: Editor's note: what RRC Configuration is used for any GBR flow accepted with best effort QoS is FFS. When QoS Flows are established or modified, the core network synchronises the N2 interface request for RAN resources with the N1 signalling sent to the UE. (c.f. steps 4 and 5 of section 4.3.3 “PDU Session Modification” in TS 23.502). The RAN sends the N1 signalling message to the UE at the same time as (re)configuring the radio resources. This N1 signalling message contains information used by the UE to associate each uplink packet with the correct QFI. The RR configuration contains information on which flows are mapped to which PDCP entities; which PDCP entities relate to which radio bearers; and which uplink radio bearers are mapped to which logical channel groups. The UE reports its amount of uplink queued data on a per-logical-channel-group basis in the uplink Buffer Status Reports. When accepting a GBR flow with best effort QoS at the target RAN node, the above procedures do not need to change. It can remain a local target-NG-RAN node decision as to how to configure the radio interface to serve the “GBR QFI”. E.g. If the target NG-RAN node maps the “GBR QFI” on to the same Logical Channel Identifier as some non-GBR QFIs, then an RRC Reconfiguration is likely to be needed when GBR QoS can be restored. Alternatively, the target NG-RAN node can map the “GBR QFI” to its own Logical Channel Group and then no RRC Reconfiguration would be needed when the GBR QoS can be restored. The second Editor’s note states: Editor's note: whether and how the UE's NAS is informed that the GBR flow has not been allocated GBR radio resources is FFS. Note that with Release 15 notification control without handover, the UE's NAS does not seem to be aware that the RAN has "notified" the core network about the unavailability of the GRB QoS. In general, the UE has no knowledge of what radio QoS (latency, priority, data rate) will actually be allocated to its flows by the NG-RAN. All the UE knows (for the uplink) is a set of mappings from packet characteristics to QFI; QFI to PDCP entity; PDCP entity to radio bearer; and, radio bearer to logical channel group; along with instructions on whether or not to use e.g. RLC acknowledged or unacknowledged mode. The volume of un-transmitted data in the uplink buffer for the logical channel group associated with the GBR flow’s QFI can give the UE some indication of when a GBR QoS is not being delivered. When radio conditions degrade badly for existing GBR flows such as QCI=1 (voice), the RAN is likely to maintain the radio (and N3 tunnel) configuration for a time similar to that of the “RADIO-LINK-TIMEOUT” that is broadcast by the local GSM cells (TS 44.018), e.g. probably around 4-8 seconds. Thus it can be assumed that the UE’s NAS and/or User Plane handling software is used to data being queued on a logical channel group used by GBR services. With the exception of the GFBR and MFBR bit rates and their time averaging windows, it can be noted that the signalling in TS 24.501 means that the UE is generally unaware of the QoS that the network intends to use for a QFI. This is particularly the case for non-standardised QFI values. In Release 15, when not performing handover, the ‘notification’ that the GBR QoS cannot be delivered by the RAN is only sent to the core network and no indication is sent to the UE. Hence it is suggested that while there is no absolute need for new signalling from the network to the UE’s NAS when the RAN is not able to deliver the requested QoS, new RRC signalling could be added so that the UE’s NAS is kept aware of signalling sent to the core network. Any such RRC signalling should be consistently applied to both the handover (key issue #7) situation and the established Release 15 ‘notification’ situation. 2 Impacts on existing nodes section needs completion 3 Solution evaluation needed 4 Some aspects from S2-1900495 (Qualcomm) incorporated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Editor’s Notes addressed 2 Impacts section completed. 3 solution evaluation added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Outages of unnecessary length for GBR flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Section 5.7 is attached only “for information” |  |  |  |  |  |  |  |  |

******** for information - Unmodified text from TR 23.725 *********

## 5.7	Key Issue #7: Automatic GBR service recovery after handover

### 5.7.1	General description

When a machine needs a Guaranteed Bit Rate service, it is likely to need that quality level in order to do its job. Hence if the QoS level cannot be maintained (e.g. handover into a congested cell; temporary cell overload due to higher priority services; etc), the QoS level should be restored as soon as possible, and, without a storm of signalling messages.

Release 15 5GC has partially solved this issue by introducing a "notification" mechanism that allows the core network to request the RAN to not release the radio bearer/QoS Flow when the QoS guarantee cannot be met, but, instead to notify the Core Network when the QoS drops, AND, when the RAN has restored the QoS level. This allows the controller of the machine (e.g. car/train) to adapt its behaviour (e.g. reduce speed) and ensures that the RAN continues to try to restore the QoS level while the device is in that cell.

However, at Xn or N2 handover, the target RAN node applies admission control and if it cannot support the required GBR QoS, the target RAN node does not establish that QoS Flow. In such a case, if, e.g. due to movement of the UE, the source RAN node has no choice but to handover to that target RAN node, then the target RAN node will complete the handover but will NOT notify the CN if and when the GBR QoS can be supplied to that UE.

As the machine needs to have its GBR service restored as soon as possible, the CN needs to repeatedly attempt to re-establish the GBR service. These re-attempts involve a considerable number of signalling messages, and are sent without any awareness of RAN congestion or potential link quality.

This is an inadequate solution for any mobile "non-human device" that needs to maintain a GBR data link during mobility (e.g. a car, a train, robots moving around a factory).

***** start of changes ***********

## 6.16	Solution #16 for Key Issue #7: Automatic GBR service recovery after handover

### 6.16.1	Description

During the Handover Preparation phase, an additional attribute "GBR required as soon as possible" is proposed to be sent by the Source RAN node to the Target RAN node in the Xn/N2 interface signalling when the existing "notification control" QoS attribute is sent.

With this new QoS attribute, at handover, the Target RAN node will perform admission control for the GBR flow. If the GBR admission control fails, the Target RAN node performs admission control for that flow as a 'best effort' (e.g. QFI=8) flow.

The signalling from the Target RAN to the Source RAN node is extended to indicate which GBR flows are able to be established with the correct target QoS, and which GBR flows are accepted with a best effort QoS.

If the GBR flow(s) could not be admitted with the proper GBR QoS, the Source RAN node can consider whether other candidate Target RAN nodes are available. The Source RAN may then perform Handover Preparation with other candidate Target RAN nodes, and if successful, the Source RAN node cancels the earlier handover attempt.

Upon handover completion in a congested RAN node the new RAN node notifies the CN that the GBR QoS cannot be met and sends another notification when the GBR QoS can again be guaranteed for that flow.

In the handover instruction that the target RAN node prepares for the source RAN node to send to the UE, the target RAN node may map the GBR 5QI (that is accepted with a best effort QoS) to a Logical Channel Identifier that is not shared with other flows, or, may make a different choice. After the UE accesses the target RAN Node:
- in the latter case, an RRC Reconfiguration is likely to be needed when the GBR QoS is subsequently restored. - in the former case the RAN node can choose whether or not to allocate any uplink resources,
- in both cases, the RAN can choose whether or not to allocate any downlink resources to that GBR QFI. If downlink resources are not going to be allocated, then, as an option, the SMF could be requested (via a notification command) to request the UPF to discard the downlink packets (and record corresponding information on the CDR)

The target RAN node to UE signalling may also carry an RRC indication that is to be passed to the UE’s NAS layer to inform the UE’s NAS that the GBR QoS will not be immediately available. After the handover, when the GBR QoS is able to be restored, the RAN node sends an indication of the restoration of QoS to the UE, e.g. using new R16 RRC signalling.

NOTE: 	The above new R16 RAN signalling functionality that informs the UE’s NAS layer about N2 interface GBR Notification signalling should be extended to also cover the non-handover ‘Notification’ situations that exist in R15.

### 6.16.2	Procedures

These are readily derivable from the above description.

### 6.16.3	Impacts on Existing Nodes and Functionality

AMF: 	no impact unless the “Notification of lack of GBR QoS” is piggy-backed on the Path Switch Request rather than being sent immediately afterwards.

PCF: 	no impact if this is developed as an extension of R15 Notification Control. Impact expected if this is developed as an alternative to R15 Notification Control.

SMF	Impact expected if this is developed as an alternative to R15 Notification Control. 
Impact also expected if RAN wants SMF to get UPF to discard downlink packets.

UPF:	existing data forwarding functionality can probably be re-purposed if the RAN wishes to have the core network discard downlink packets.

RAN: 	a) the main “radio resource scheduler” functionality is common with R15. 
b) Updates to the “controller” of the admission control process are needed, e.g. to re-run the admission control process once every 500 ms for UE’s with “notified” (suspended) GBR QoS Flows.
c) new RRC signalling to keep the UE’s NAS aligned with the Notification status in the Core Network.
d) additional Xn/N2 signalling parameter to indicate to the source RAN node the new handling of this  GFBR flow at the target RAN node.

UE (that co-exists with R15 Notifications sent for GBR flows)
a) new RRC signalling to keep the UE’s NAS aligned with the Notification status in the Core Network

### 6.16.4	Solution Evaluation

Editor's note:	This clause provides an evaluation of this solution.

******* end of changes *****