3GPP TSG-SA WG2 Meeting #131	S2-1901641

# Tenerife (Spain), 25 Feb - 1 March 2019

Agenda Item:	6.15

Source:	OPPO

Title:	Discussion and proposal for Key Issue 3.2 Time Sync in TSN

Document for:	Discussion, Decision

- Introduction

In TR23.725, there are some alternative solutions for time sync in TSN which have different pros and cons.

In this contribution, we analyze the impact from each solutions and propose to adopt solution#11 option 2 in SA2.

- Discussion

There are 6 different solutions which are outlined as below with their elaborate description:

1) Conveying timing to the UE that act as boundary master clocks towards connected TSN device via 5G specific signalling, e.g. via 5G broadcast/5G unicast frame structure. In this option, the 5G RAN utilizes its fine-frame structure (e.g. below PHY symbol level) to convey precise timing to the UE. The 5G RAN receives the TSN timing information via direct connectivity with the TSN master clock, e.g. via underlying transport network by having an embedded TSN client within the gNB (this option does not use UE specific 802.1AS messages). (Refer Solution #11 Option 2)

2) One time-aware relay implemented with Solution#8 5G RAN is unaware of the TSN. The timing information from TSN working domain (external clock) is delivered via the UEs to the respective End stations.  The 5G internal system clock will keep these network elements synchronized so that the timestamping of the gPTP event messages is done correctly. The 5G internal system clock can be made available to UE with signalling of time information related to absolute timing of radio frames (i.e. using SIB/RRC based methods described for LTE Rel-15). The timing information (gPTP messages, including the information on the incoming sync message timestamping) can be carried to the UE as data packets (e.g. payload). The time stamp is based on the 5G internal system clock. (Refer Solution #11 Options 3).

3) For multiple time domains: “Multiple time domains merged into one domain using 5G clock “, the UE only receives 5G timing information through gNB, and acts as master clock to the TSN end stations. In this solution each interface of the 5G system is seen by the connected TSN networks and by the End stations, as separate GMs, each of them operating in independent gPTP domains, but providing the same time to all the connected networks. (Refer Solution #11 Options 4).

4) 5GS gives deterministic delays between UPF and UE and 5GS acts as a link or as a TSN Bridge. When 5GS acts as a link, PTP messages containing clock information are passing through 5GS experiencing deterministic delays. 5GS need not perform any measurement/synchronization processes. When 5GS acts as a TSN bridge, the deterministic delays between UPF and UE make the residence time in 5GS be easily calculated so that 5GS can make proper correction of the PTP header’s “correctionField” with the residence time. (See Solution #17).

5) Based on the internal synchronization, the 5GS transparently pass the external PTP message through and makes proper correction of the PTP header’s “correctionField” with the known residence time (See solution #19).

6) 5GS acts as transparent clock with independent internal clock achieving common concept of time between UEs and UPF as well as among different UEs. This is to allow one-way measurement and control of the E2E delay (Refer Solution #28).

From UE vendor perspective, the impact comparison is made in the table below:

|  | Solution #11 Option 2 | Options other than Solution #11 Option 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Solution #11 Option 3 | Solution #11 Option 4 | Solution #17 | Solution #19 | Solution #28 |
| Time information sent by | SIB/RRC (broadcast/Unicast) | Unicast | SIB/RRC (broadcast/Unicast) | Unicast | Unicast | Unicast |
| Time information on Uu interface | TSN reference time | 802.1AS PTP messages | TSN reference time | 802.1AS PTP messages | Two time stamps, one is related to PDU layer, one is related to transport layer | 802.1AS PTP messages |
| CP/UP impact on air interface | CP impact | UP impact | CP impact | UP impact | UP impact | UP impact |
| UE needs to support gPTP-like procedure on Uu interfasce | No | Yes | No | Yes | Yes, Packet with 2 time stamp on two layers | Yes |
| Overhead | fixed overhead for broadcast method, large overhead for unicast method | Large overhead on Uu interface; Increased when UE number increased. | fixed overhead for broadcast method, large overhead for unicast method | Large overhead on Uu interface; Increased when UE number increased. | Large overhead on Uu interface; Increased when UE number increased. | Large overhead on Uu interface; Increased when UE number increased. |
| 5GS impact | Small Reuse existing time sync in LTE with limited extension. | None Only TSN Translator is impacted | None Only TSN Translator is impacted | Large Deterministic delay shall be supported | Large PDCP and GTP-U requires to be extended. New functionality in UE is required to get time information from two layer (i.e. PDCP and PDU layer) | Large Deterministic delay shall be supported |
| Others |  | Synchronization latency to TSN clock when UE turns to wake-up. | Synchronization latency to TSN clock when UE turns to wake-up; Need TSN time domain information. | Synchronization latency to TSN clock when UE turns to wake-up; Only support deterministic air delay and backhaul delay. | Synchronization latency to TSN clock when UE turns to wake-up; Need to enhance PDCP layer; Need to enhance PDU layer functionality, which is out of 3GPP scope. | Synchronization latency to TSN clock when UE turns to wake-up; Need to measure the residence time of each PTP message, or use fixed residence time similar as Solution #17. |

if solution #11 Option 2 is adopted, it is possible for RAN to use existing 5G internal time sync mechanism to make the TSN clock in UE be synchronized to the boundary TSN clock on RAN node.

Solution #11 Option 3 requires the enhancement on user-plane.

Solution #11 Option 4 requires the cooperation from all TSN domain and ask all TSN domains work under the same clock. It is difficult for implementation.

Solution #17 and #28 are highly coupled with deterministic delay as it requires a certain delay within 5GS. It seems not feasible as it based on the assumption that TSN QoS enhancement mechanism is accomplished.

Solution #19 requires the support both from 3GPP and upper (i.e. PDU) layers rather than reusing IEEE 802.1 AS. It causes high complexity in PDCP layer. And it is unclear how to collect the two time stamps at UE adaptor from UE PDCP layer.

Solution#11 option-3/4 has nearly zero impact to 5GS and solution#11 option-2 has small impact to 5GS, while other solutions have large impact to 5GS.

For scalability impact, gPTP messages transport over the air is not supported in legacy system.

It is unclear how to identify the packets with data and packets with gPTP message for all user-plane based solutions.

It is preferred to adopt Solution #11 Option 2 in SA2

Leave the time sync over air interface to RAN WGs, and let them decide if a broadcast and/or unicast signalling is required in air interface.

- Proposal

Based on the discussion above, we made the following proposal:

- It is preferred to adopt Solution #11 Option 2 in SA2.

Leave the time sync over air interface to RAN WGs, and let them decide if a broadcast and/or unicast signalling is required in air interface.