3GPP TSG-SA WG2 Meeting #131	S2-1901741

# 25 Feb – 1 Mar, 2019, Santa Cruz – Tenerife, ES							(revision of S2-1901337)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0747 | rev | 3 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Session Continuity and UP Path management Support for 5G LAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR addresses Editor’s note: “Further description of how Service Continutiy and UP Path management principles described in TS 23.502 [3] apply to 5G LAN-type services is expected to be needed in other clauses of this TS” |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR captures system modification required to enable Service Continutiyand UP path management in a 5G-LAN type service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5G Access Network: An access network comprising a NG-RAN and/or non-3GPP AN connecting to a 5G Core Network.

5G Core Network: The core network specified in the present document. It connects to a 5G Access Network.

5G QoS Flow: The finest granularity for QoS forwarding treatment in the 5G System. All traffic mapped to the same 5G QoS Flow receive the same forwarding treatment (e.g. scheduling policy, queue management policy, rate shaping policy, RLC configuration, etc.). Providing different QoS forwarding treatment requires separate 5G QoS Flow.

5G QoS Identifier: A scalar that is used as a reference to a specific QoS forwarding behaviour (e.g. packet loss rate, packet delay budget) to be provided to a 5G QoS Flow. This may be implemented in the access network by the 5QI referencing node specific parameters that control the QoS forwarding treatment (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Allowed NSSAI: NSSAI provided by the Serving PLMN during e.g. a Registration procedure, indicating the S-NSSAIs values the UE could use in the Serving PLMN for the current registration area.

Allowed Area: Area where the UE is allowed to initiate communication as specified in clause 5.3.2.3.

AMF Region: An AMF Region consists of one or multiple AMF Sets.

AMF Set: An AMF Set consists of some AMFs that serve a given area and Network Slice(s). AMF Set is unique within an AMF Region and it comprises of AMFs that support the same Network Slice(s). Multiple AMF Sets may be defined per AMF Region.

Application identifier: An identifier that can be mapped to a specific application traffic detection rule.

AUSF Group ID: This refers to one or more AUSF instances managing a specific set of SUPIs.

Configured NSSAI: NSSAI provisioned in the UE applicable to one or more PLMNs.

DN Access Identifier (DNAI): Identifier of a user plane access to one or more DN(s) where applications are deployed.

Emergency Registered: A UE is considered Emergency Registered over an Access Type in a PLMN when in limited service state and registered for emergency services only over this Access Type in this PLMN.

Endpoint Address: An address used by a NF service consumer to access the NF service (i.e. to invoke service operations) provided by a NF service provider. An Endpoint Address is represented in the syntax of Uniform Resource Identifier (e.g. part of Resource URI of the NF service API).

Expected UE Behaviour: Set of parameters provisioned by an external party to 5G network functions on the foreseen or expected UE behaviour, see clause 5.20.

Forbidden Area: An area where the UE is not allowed to initiate communication as specified in clause 5.3.2.3.

GBR QoS Flow: A QoS Flow using the GBR resource type or the Delay-critical GBR resource type and requiring guaranteed flow bit rate.

Initial Registration: UE registration in RM-DEREGISTERED state as specified in clause 5.3.2.

Local Area Data Network: a DN that is accessible by the UE only in specific locations, that provides connectivity to a specific DNN, and whose availability is provided to the UE.

Local Break Out (LBO): Roaming scenario for a PDU Session where the PDU Session Anchor and its controlling SMF are located in the serving PLMN (VPLMN).

Mobility Pattern: Network concept of determining within the AMF the UE mobility parameters as specified in clause 5.3.2.4.

Mobility Registration Update: UE re-registration when entering new TA outside the TAI List as specified in clause 5.3.2.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NGAP UE association: The logical per UE association between a 5G-AN node and an AMF.

NGAP UE-TNLA-binding: The binding between a NGAP UE association and a specific TNL association for a given UE.

Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

NOTE 2:	A network function can be implemented either as a network element on a dedicated hardware, as a software instance running on a dedicated hardware, or as a virtualised function instantiated on an appropriate platform, e.g. on a cloud infrastructure.

Network Instance: Information identifying a domain. Used by the UPF for traffic detection and routing.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

Network Slice instance: A set of Network Function instances and the required resources (e.g. compute, storage and networking resources) which form a deployed Network Slice.

Non-GBR QoS Flow: A QoS Flow using the Non-GBR resource type and not requiring guaranteed flow bit rate.

NSI ID: an identifier for a Network Slice instance.

NF instance: an identifiable instance of the NF.

NF service: a functionality exposed by a NF through a service based interface and consumed by other authorized NFs.

NF service instance: an identifiable instance of the NF service.

NF service operation: An elementary unit a NF service is composed of.

NG-RAN: A radio access network that supports one or more of the following options with the common characteristics that it connects to 5GC:

1)	Standalone New Radio.

2)	New Radio is the anchor with E-UTRA extensions.

3)	Standalone E-UTRA.

4)	E-UTRA is the anchor with New Radio extensions.

Non-Allowed area: Area where the UE is allowed to initiate Registration procedure but no other communication as specified in clause 5.3.2.3.

Non-Seamless Non-3GPP offload: The offload of user plane traffic via non-3GPP access without traversing either N3IWF or UPF.

PDU Connectivity Service: A service that provides exchange of PDUs between a UE and a Data Network.

PDU Session: Association between the UE and a Data Network that provides a PDU connectivity service.

PDU Session Type: The type of PDU Session which can be IPv4, IPv6, IPv4v6, Ethernet or Unstructured.

Periodic Registration Update: UE re-registration at expiry of periodic registration timer as specified in clause 5.3.2.

(Radio) Access Network: See 5G Access Network.

Requested NSSAI: NSSAI provided by the UE to the Serving PLMN during registration.

Routing Indicator: Indicator that allows together with SUCI/SUPI Home Network Identifier to route network signalling to AUSF and UDM instances capable to serve the subscriber.

Service based interface: It represents how a set of services is provided/exposed by a given NF.

Service Continuity: The uninterrupted user experience of a service, including the cases where the IP address and/or anchoring point change.

Service Data Flow Filter: A set of packet flow header parameter values/ranges used to identify one or more of the packet (IP or Ethernet) flows constituting a Service Data Flow.

Service Data Flow Template: The set of Service Data Flow filters in a policy rule or an application identifier in a policy rule referring to an application detection filter, required for defining a Service Data Flow.

Session Continuity: The continuity of a PDU Session. For PDU Session of IPv4 or IPv6 or IPv4v6 type "session continuity" implies that the IP address is preserved for the lifetime of the PDU Session.

Subscribed S-NSSAI: S-NSSAI based on subscriber information, which a UE is subscribed to use in a PLMN

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs.

UDR Group ID: This refers to one or more UDR instances managing a specific set of SUPIs.

UPF Service Area: The area within which PDU Session associated with the UPF can be served by (R)AN nodes via a N3 interface between the (R)AN and the UPF without need to add a new UPF in between or to remove/re-allocate the UPF.

Uplink Classifier: UPF functionality that aims at diverting Uplink traffic, based on filter rules provided by SMF, towards Data Network.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.261 [2] apply:

# 5G LAN-type service

# Private communication

**** Next Change ****

### 5.X 	Support for 5G LAN-type service

### 5.X.1 	General

The service requirements for 5G LAN-type service are specified in TS 22.261 [2].

A 5G-LAN group consists of a a set of UEs using private communication for 5G LAN-type services.

### 5.X.2 	5G LAN group management

5G System supports 5G LAN Group Management can be configured by a network administrator or can be managed dynamically by AF.

The 5G LAN group member is uniquely identifed by GPSI. The group as described in clause 5.2.3.3.1 of TS 23.502 [4] is used to identify the 5GLAN group. In order to support dynamic 5GLAN Group Management, the NEF exposes a set of service to manage (e.g. add/delete) 5G LAN group and 5G LAN member.

The 5G LAN group configuration is either provided by OA&M or provided by an AF to the NEF. The 5G LAN group configuration is stored in the UDR.

When configuration is provided by an AF, the NEF service operations information flow procedure described in TS 23.502 [3] clause 4.15.6.2 applies. The 5G LAN group configuration may include the following parameters: GPSI (i.e. UE Identities of the 5G LAN group), PDU session type, DNN, S-NSSAI.

The third party AF may update the UE Identities of the 5G LAN group at any time after the initial provisioning.

Editor’s note: Further description of 5G LAN group configuration parameters is expected to be needed in other clauses of this TS as well as other TSs and they could then be referred to.

Editor’s note: Relation between DNN and 5G LAN group, e.g. whether and how a 1:1 and/or 1:N relation is supported, is FFS.

Some of the 5G-LAN group configuration is delivered from the PCF to the UE using the UE Configuration Update procedure for transparent UE Policy delivery as described in TS 23.502 [3] clause 4.2.4.3.

### 5.X.3 	PDU Session management and user plane handling

Session management as defined for 5GS in clause 5.6 is applicable to 5G-LAN-type services with the following clarifications:

-	A UE gets access to 5G LAN-type services via a PDU Session of IP PDU Session type or Ethernet PDU Session type.

-	A PDU Session provides access to one and only one 5G-LAN group. A UE can use more than one PDU Sesssion to access the same 5GLAN group.

-	A dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G-LAN group.

Editor’s note: Whether and how the above bullet needs to be updated due to eSBA is FFS

NOTE: 	Having a dedicated SMF serving a 5G-LAN group does not contradict that redundancy solutions can be used to achieve high availability.

-	A DNN is associated with a 5G-LAN group

Editor’s note: The above bullet is FFS

-	The UE provides a DNN associated with the 5G-LAN group to access the 5G LAN-type services for that 5G-LAN group, using the PDU Session Establishment procedure described in TS 23.502 [3], clause 4.3.2. The AMF shall reject the NAS message contatining PDU Session Establishment request when the DNN associated to a 5GLAN group is not configured for the S-NSSAI(s) provided by the UE. (E.g., the DNN associated with the 5GLAN group is not in the Subscribed DNN List for the S-NSSAI.

-	The UE may provide a 5GLAN member GPSI list of 5GLAN group members to request path-based forwarding. The SMF may use the 5GLAN member GPSI list to determine the corresponding pathID for each 5GLAN group member included in the GPSI 5GLAN if path-based forwardin is supported. The SMF may also use the 5GLAN member GPSI list of 5GLAN group members to correlate PDU Session that are affected by a PSA reallocation.

-	The UE may use individual pathIDs, provided by the SMF, in the PDU Session Establishment Accept message, as bitfields, from one or more 5GLAN group members to build a multicast pathID address.

-	During establishment of the PDU Session, secondary authentication as described in clause 5.6.6 and in TS 23.502 [3], clause 4.3.2.3, may be performed in order to authenticate and authorize the UE for accessing the DNN associated with the 5G-LAN group. Authentication and authorization for a DNN using secondary authentication implies authentication and authorization for the associated 5G-LAN group. There is no 5G-LAN group specific authentication or authorization defined.

- 	The SM level subscription data for a DNN and S-NSSAI available in UDM, as described in clause 5.6.1, applies to a DNN associated to a 5G-LAN group.

- 	Session management related policy control for a DNN as described in TS 23.502 [3], is applicable to a DNN associated to a 5G-LAN group. This includes also usage of URSP, for the UE to determine how to route outgoing traffic to a PDU Session for a DNN associated to a 5G-LAN group.

There are three types of traffic forwarding methods for 5G-LAN communication:

-	N6-based, where the UL/DL traffic for the 5G-LAN communication is forwarded to/from the DN;

-	Nx-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via Nx. Nx is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G LAN group;

Editor’s note: Further description of Nx and local switch forwarding is expected to be needed in other clauses of this TSas well as other TSs and they could then be referred to.

The SMF may configure the UPF(s) to apply different traffic forwarding methods to route traffic between PDU Sessions for a single 5G LAN group. For example, depending on the destination address, some packet flows may be forwarded locally, while other packet flows are forwarded via Nx and other packet flows are forwarded to N6.

The SMF may provide the UE with packet forwarding information

When a SSC mode 2 PDU Session associated to a 5GLAN group is released and a new PDU Session is re-established a described in TS 23.502, clause 4.x.11, the SMF shall update forwarding rules on affected PSA(s) belonging to the 5GLAN group. The PSA correlates old and new destination address of UEs for which a new PSA has been allocated and it forwards packets intended for the old destination, to the new destination address

Editor’s note: How the SMF determines what traffic forwarding method(s) to apply for a PDU Session corresponding to a 5G-LAN group is FFS, e.g. whether based on local configuration and/or information from PCF.

**** End of Changes ****