3GPP TSG-SA WG2 Meeting #131	S2-1901468

# Santa Cruz, Tenerife, Spain, 25 February – 1 March 2019 	(revision of S2-19xxxxx)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0909 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Control of traffic forwarding in 5G-LAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a need to describe how SMF can control the UPF for handling user plane belonging to a 5G-LAN group |  |  |  |  |  |  |  |  |
| Summary of change: |  | The N4 rules are updated to support new 5G-LAN requirements for local switching and Nx based forwarding |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.X (new), 5.8.2.11.3, 5.8.2.11.6, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

##### 5.8.2.X 	Support for 5G-LAN group communication

In addition to N6-based forwarding of user plane traffic for for 5G-LAN group communication, the SMF and UPF may support local switch and Nx-based forwarding:

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G LAN group;

-	Nx-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via Nx. Nx is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G LAN group.

The local switch and Nx-based forwarding methods require that a common SMF is controlling the PSA UPFs for the 5G-LAN group. If separate SMFs are controlling the PSA UPFs for two 5G LAN group members, N6 based forwarding applies.

To enable local switch in a UPF, the SMF provides FARs for 5G LAN group members’ N4 Sessions (as originating side) with Destination Interface set to “5G LAN internal” to instruct the UPF to again perform classification of packets using relevant PDRs. The SMF also provides PDRs for 5G LAN group members’ N4 Session (as terminating side) with Source Interface set to “5G LAN internal” to instruct the UPF to detect packets that have been sent to classification via the “local switch”.

To enable Nx-based forwarding in a UPF, the SMF may create a N4 session to each involved UPF for forwarding packets to a Nx tunnel as well as processing packets received from a Nx tunnel. Such a N4 session is created per UPF and 5G-LAN group and SMF configures the UPF with relevant N4 rules (e.g. PDRs, QERs, URSs and FARs) applicable for the Nx tunnel in the UPF. In such N4 session, the SMF may configure a default PDR to capture the packets pertaining to 5G LAN group communication with an unknown destination address and a URR to trigger reporting of such events, to enable the SMF to configure relevant PDR and FAR to forward the packets to another UPF via Nx.

**** Next Change ****

##### 5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR |  |
| Rule ID |  | Unique identifier to identify this rule |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN", “5G LAN internal”, “5G LAN Nx” | Combination of UE IP address (together with Network instance, |
| detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length | if necessary), CN tunnel info, packet filter set, application ID, |
| information | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet | Ethernet PDU Session Information and QFI are used for traffic detection. |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID |  |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | Details like all the combination possibilities on N3, N9 interfaces |
|  | Application ID |  | are left for stage 3 decision. |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI | Source interface identifies the interface for incoming packets where the PDR applies, e.g. from access side (i.e. up-link), from core side (i.e. down-link), from SMF, from N6-LAN (i.e. the DN or the local DN), from “5G LAN internal” (i.e. local switch), or from “5G LAN Nx” (i.e. Nx interface). |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains - UPF “local switch” and Nx forwarding is used for different 5G LAN groups. |  |  |  |

***** Next Change *****

##### 5.8.2.11.5	Usage Reporting Rule

The following table describes the Usage Reporting Rule (URR) that defines how a packet shall be accounted as well as when and how to report the measurements.

Table 5.8.2.11.5-1: Attributes within Usage Reporting Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this URR |  |
| Rule ID | Unique identifier to identify this information. | Used by UPF when reporting usage. |
| Reporting triggers | One or multiple of the events can be activated for the generation and reporting of the usage report. | Applicable events include: - Start/stop of traffic detection with/without application instance identifier and deduced SDF filter reporting; Deletion of last PDR for a URR; Periodic measurement threshold reached; Volume/Time/Event measurement threshold reached; Immediate report requested; Measurement of incoming UL traffic; Measurement of discarded DL traffic; MAC address reporting in the UL traffic; unknown destination MAC/IP address. |
| Periodic measurement threshold | Defines the point in time for sending a periodic report for this URR (e.g. timeofday). | This allows generation of periodic usage report for e.g. offline charging. It can also be used for realizing the Monitoring time of the usage monitoring feature. It can also be used for realizing the Quota-Idle-Timeout, i.e. to enable the CP function to check whether any traffic has passed during this time. |
| Volume measurement threshold | Value in terms of uplink and/or downlink and/or total byte-count when the measurement report is to be generated. |  |
| Time measurement threshold | Value in terms of the time duration (e.g. in seconds) when the measurement report is to be generated. |  |
| Event measurement threshold | Number of events (identified according to a locally configured policy) after which the measurement report is to be generated. |  |
| Inactivity detection time | Defines the period of time after which the time measurement shall stop, if no packets are received. | Timer corresponding to this duration is restarted at the end of each transmitted packet. |
| Event based reporting | Points to a locally configured policy which is identifies event(s) trigger for generating usage report. |  |
| Linked URR ID(s) | Points to one or more other URR ID. | This enables the generation of a combined Usage Report for this and other URRs by triggering their reporting. See clause 5.2.2.4, TS 29.244 [65]. |
| Measurement Method | Indicates the method for measuring the network resources usage, i.e. the data volume, duration, combined volume/duration, or event. |  |
| Measurement information | Indicates specific conditions to be applied for measurements | It is used to request: - measurement before QoS enforcement, and/or - to pause or set to active a measurement as for the Pause of charging described in clause 4.4.4 of TS 23.502 [3], and/or - to request reduced reporting for application start/stop events. |

***** Next Change *****

##### 5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. |  |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included. |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). |  |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", “5G LAN internal” or “5G LAN Nx”. | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN or the local DN), to 5G LAN internal (i.e. local switch), or to 5G LAN Nx (i.e. Nx interface). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP+QFI, VLAN tag) to the outgoing packet. | Contains the CN tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF). Any extension header stored for this packet shall be added. |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. |  |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to appropriated N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF “local switch” and Nx forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 in TS 29.244 [65]. NOTE 7: The use of “5G LAN Internal” instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session. To avoid that the packet matches again the same PDR, it is assumed that the packet has been modified in some way (e.g. tunnel header has been removed, the packets being received from the source interface 5G LAN Internal). |  |  |

***** Next Change *****

##### 5.8.2.11.7	Usage Report generated by UPF

The UPF sends the usage report to inform the SMF about the measurement of an active URR or about the detection of application traffic of an active Packet Detection Rule. For each URR, the usage report may be generated repeatedly, i.e. as long as any one of the valid event triggers applies. A final usage report is sent for a URR when it is no longer active, i.e. either the URR is removed or all the references to this URR in any of the Packet Detection Rules belonging to the N4 session.

Following attributes can be included in the usage report:

Table 5.8.2.11.7-1: Attributes within Usage Report

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Uniquely identifies a session. | Identifies the N4 session associated to this Usage Report |
| Rule ID | Uniquely identifies the Packet Detection Rule or Usage Reporting Rule within a session which triggered the report. | Packet Detection Rule is only indicated when Reporting trigger is Detection of 1st DL packet for a QoS Flow or Start/stop of traffic detection. Usage Reporting Rule is indicated for all other Reporting triggers. |
| Reporting trigger | Identifies the trigger for the usage report. | Applicable values are: Detection of 1st DL packet for a QoS Flow; Start/stop of traffic detection with/without application instance identifier and deduced SDF filter reporting; Deletion of last PDR for a URR; Periodic measurement threshold reached; Volume/Time/Event measurement threshold reached; Immediate report requested; Measurement of incoming UL traffic; Measurement of discarded DL traffic; MAC address reporting in the UL traffic; reporting of unknown destination MAC/IP address. |
| Start time | Provides the timestamp, in terms of absolute time, when the collection of the information provided within Usage-Information is started. | Not sent when Reporting trigger is Start/stop of traffic detection. |
| End time | Provides the timestamp, in terms of absolute time, when the information provided within Usage-Information is generated. | Not sent when Reporting trigger is Start/stop of traffic detection. |
| Measurement information | Defines the measured volume/time/events for this URR. | Details refer to TS 29.244 [65]. |

**** End of Changes ****