3GPP TSG-SA2 Meeting #131	S2-1901662

# Santa Cruz - Tenerife, Spain, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1040 | rev | - | Current version: | 15.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Undelivered Steering of Roaming information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-02-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During Registration or UE Parameters Update, UDM may send Steering of Roaming information to the UE via AMF. If the AMF fails delivering the Steering of Roaming information to the UE (example the UE is unreachable), then UDM in the HPLMN needs to be notified. and the UDM in the HPLMN may wait for the acknowledgement of SoR from UE (if it asked for SOR acknowledgment) and/or expect the UE to register to another PLMN. This delivery failure case leads to a situation where the HPLMN UDM is not informed on which VPLMN the UE is registered to, resulting in insufficient SoR distribution in each destination. Therefore, the UDM needs to be timely informed by the AMF in case of failed delivery of SoR information to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AMF sends Nudm_SDM_Info also to indicate to the UDM a failed delivery of SoR information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPLMN UDM is not informed on which VPLMN the UE is registered to, resulting in insufficient SoR distribution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

**************** Start of Change ****************

##### 5.2.3.3.6	Nudm_SDM_Info service operation

Service Operation name: Nudm_SDM_Info

Description: Consumer NF provides UDM with information about the status of the subscription data management procedures. This service operation is used for:

-	providing acknowledgement from the UE to UDM about successful delivery of Steering of Roaming information via the AMF as defined in TS 23.122 [22].

-	informing the UDM when the delivery of Steering of Roaming information to the UE was not successful, sending back the SoR information to the UDM for validation.

-	providing acknowledgement from the UE to UDM about successful Network Slicing Configuration subsequent to delivery of the Network Slicing Subscription Change Indication via the AMF.

Inputs, Required: SUPI, Info (e.g. UE acknowledgment of SoR information from UDM via AMF, undelivered SoR information, UE acknowledgement of successful Network Slicing Configuration subsequent to delivery of the Network Slicing Subcription Change Indication via the AMF).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

**************** End of Changes ****************