SA WG2 Meeting #131 	S2-1902268

# 25 February - 1st March 2019, Santa Cruz de Tenerife, Spain				 (revision of S2-1901992)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 0998 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment with RAN3 about providing Network Instance to the 5G-AN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | R3-187086 “Correction of dedicated PDU Session transport resources” approved by last RAN plenary introduces support of a Network Instance in the PDU Session Resource Setup Request For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource Source: Nokia, Nokia Shanghai Bell, Huawei, LG Electronics, Orange, China Telecom, China Unicom |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the SMF asks the 5G-AN for UP resources for a PDU Session, the SMF may provide the 5G-AN with the Network Instance it has allocated to the 5GC termination of the N3 interface of this PDU Session. The Network Instance associated with the PDU Session is provided to the 5G-AN as part of CN Tunnel Info. This ensures that the 5G-AN can select N3 resources for a PDU Session on the same transport network than selected by the SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5GC and 5G-AN may use different transport networks on either side of the N3 interface for a PDU Session, leading to no UP transfer on this PDU Session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.6.12	Use of Network Instance

The SMF may provide a Network Instance to the UPF in FAR and/or PDR via N4 Session Establishment or Modification procedures.

NOTE 1:	a Network Instance can be defined e.g. to separate IP domains, e.g. when a UPF is connected to 5G-ANs in different IP domains, overlapping UE IP addresses assigned by multiple Data Networks, transport network isolation in the same PLMN, etc.

The SMF may determine the Network Instance for N3, N9 and N6 interfaces, based on the e.g. UE location, registered PLMN ID of UE, S-NSSAI of the PDU Session, DNN, etc.

NOTE 2:	As an example, the UPF can use the Network Instance included in the FAR, together with other information such as Outer header creation (IP address part) and Destination interface in the FAR, to determine the interface in UPF (e.g. VPN or Layer 2 technology) for forwarding of the traffic.

When the SMF asks the 5G-AN for UP resources for a PDU Session, the SMF may provide the 5G-AN with the Network Instance it has allocated to the 5GC termination of the N3 interface of this PDU Session. The Network Instance associated with the PDU Session is provided to the 5G-AN as part of CN Tunnel Info.

NOTE 3: 	This ensures that the 5G-AN can select N3 resources for a PDU Session on the same transport network than selected by the SMF for this PDU Session.

NEXT CHANGE

END OF CHANGES