SA WG2 Meeting #132 	S2-1903655

08 – 12 April, 2019, Xi’an, PRC

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1215 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSC deployment configuration – resolution of EN |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  | Date: |  | 3/30/2019 |
| Category: | F |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support for hybrid model was left as FFS during SA2#131. It was decided that centralized model will be supported and distributed model will not be supported in Rel-16. Since hybrid model comprises of both, it could only be supported partially (i.e. centralized part and not the other mode which is distributed) thus for simplicity it has been decided not to support hybrid model in Rel-16. |  |  |  |  |  |
| Summary of change: |  | Clarify Hybrid Model is not supported in Rel-16 |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s note for hybrid model remains unresolved. |  |  |  |  |  |

### 4.4.8	Time Sensitive Communication

#### 4.4.8.1	General

The 5G System is extended to support Time sensitive communication as defined in IEEE P802.1Qcc [95].

IEEE TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks as in Time-Sensitive Networking (TSN) of the IEEE P802.1Qcc [95]. Integration of 5G System with TSN networks that are based on IEEE TSN (IEEE P802.1Qcc [95]) is supported.

In this Release of the specification, it is about integration of 5G System with IEEE TSN (IEEE P802.1Qcc [95]).

#### 4.4.8.2	Architecture to support Time Sensitive Communication

The 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator (TT) functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as any other TSN Bridge, the 5GS provides TSN ingress and egress ports via the TSN Translator (Device) functionality in the UE side and via the "TSN Translator" (CP and UP) functionality on the 5GC side towards the DN. TSN Translator on the UE side and UPF side supports hold and forward functionality for the purpose of de-jittering.

There are three TSN configuration models defined in IEEE P802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this release of the specification

-	hybrid model is not supported in this release of the specification.

-	fully distributed model is not supported in this release of the specification.

NOTE:	This release only supports interworking with TSN using IEEE 802.1Qbv [96] based QoS scheduling.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

Editor's note:	If TSN Translator is outside the UE/UPF or not is FFS. Whether 3GPP specifies UE to TSN Translator interface (N60) is FFS. Support for functionalities other than hold and forward within the TSN Translator is FFS.