SA WG2 Meeting #133 	S2-1905197

# May 13 – 17, 2019 ; Reno, Nevada, USA								 (revision of S2-1903311)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1100 | rev | 2 | Current version: | 16.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G LAN group communication with UPF autonomous traffic forwarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2019-04-24 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR focuses on the Ethernet PDU Session type in a 5G VN group communication The method specified in CR 0909 agreed at SA2 131 assumes that the UPF needs to report to the SMF the (MAC) addresses it detects in PDU Session traffic for the SMF to be able to update all UPF serving the 5G VN group with traffic filters (at local switch level / at Nx=N19 level) with the detected MAC address. This causes un necessary high signaling load. Assuming a VN group with 100 PDU Sessions each serving 20 MAC address and equally spread over 5 sites (5 PSA UPF), the amount of siignaling load is - UPF report of a new MAC Address: 100*20 = 2000 N4 reports - SMF configuration of the PDR/FAR for the N4 sessions related to these PDU Sessions (on the local UPF/PSA where the MAC address has been discovered)= 2000 configuration operations - SMF configuration of the PDR/FAR for the N4 sessions related to N19 on other UPF/PSA = (5-1) UPF * 2000 = 8000 configuration operations |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Ethernet PDU Sessions, - the UPF may be instructed by the SMF to locally switch traffic targeting any MAC address within the MAC addresses used behind a UE (member of the group) by its own without the need for SMF instructions to do so: The N4 rules (PDR) are updated to support MAC address learning by the UPF; this reuses the Ethernet PDU Session Information already defined in rel15 - the UPF may be instructed to forward packets with an unknown destination address towards all UPFs involved in the 5G VN group, i.e. over all N19 tunnels setup by the SMF for the corresponding 5G VN group. - FAR definition supports requesting to distribute replicates of the same packets towards multiple destinations (i.e. multiple N19 tunnels). - When doing so, the UPF adds in the N19 tunnel header an indication that the packet has already been forwarded over N19 A similar traffic replication capability is defined to handle Ethernet brodacast traffic: - Associated with a PDR identifying Ethernet broadcast traffic on the access side of N4 Sessions corresponding to PDU Sessions of 5G VN group members: a FAR requiring to replicate this packet over all N19 interfaces (together with the addition that indication that the packet has been forwarded over N19) and access interfaces related with the group except the incoming N4 access session - When forwarding traffic over N19, the UPF adds in the N19 tunnel header an indication that the packet has already been forwarded over N19 - a PDR capturing any traffic from the local switch with a broadcast Ethernet destination address together with an indication that the packet has been forwarded over N19; this PDR is associated with a FAR requiring to replicate this packet over all access interfaces related with the group . the FAR definition is upgraded to support requesting to distribute replicates of the same packets towards multiple destinations (supports e) and g)) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.13, 5.8.2.11.3, 5.8.2.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Modification wrt previous version - Quantify the signaling impacts of the current solution - terminology (N19, 5G VN) - Support of Ethernet broadcast - clarifications, |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.8.2.13	Support for 5G-LAN group communication

In addition to N6-based forwarding of user plane traffic for 5G-LAN group communication, the SMF and UPF may support local switch and N19-based forwarding:

-	Local switch, where traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VN group;

-	N19-based, where the UL/DL traffic for the 5G-LAN communication is forwarded between PSA UPFs of different PDU sessions via N19. N19 is based on a shared User Plane tunnel connecting PSA UPFs of a single 5G VN group.

Editor's note:	The way to distinguish the traffic applying N6-based forwarding and traffic applying local switch/N19-based forwarding in a PDU Session is FFS.

In this clause “the UPF” refers to a UPF that acts as a PSA for a PDU Session related to a 5G VN group.

The SMF may configure the UPF(s) to apply different traffic forwarding methods to route traffic between PDU Sessions for a single 5G VN group. For example, depending on the destination address, some packet flows may be forwarded locally, while other packet flows are forwarded via N19 and other packet flows are forwarded to N6.

The local switch and N19-based forwarding methods require that a common SMF set is controlling the PSA UPFs for the 5GVN group. This is further defined in clause 5.29.3.

To enable local switch in a UPF, the following applies:

-	The SMF provides FARs for 5G VN group members' N4 Sessions (i.e. N4 Session corresponding to PDU Session) with  Destination Interface set to "5G VN internal" and Network Instance set to represent the 5G VN group in order to instruct the UPF to again perform classification of packets using the corresponding 5G VN group relevant PDRs.

Both point to point and broadcast traffic may be directed to the "5G VN internal" local switch. The handling of broadcast traffic is detailed later

-	The SMF also provides PDRs for 5G VN group members' N4 Session (i.e. N4 session corresponding to PDU Session) with with Source Interface set to "5G VN internal" and Network Instance set to represent the 5G VN group in order to instruct the UPF to detect packets that have been sent to classification via the local switch. This is done by configuring the PDRs in the target 5G VN group members' N4 Session to include the destination IP/MAC address(es) of these UEs or for Ethernet PDU Sessions to include an Ethernet PDU Session Information that refers to all MAC addresses used as source address within UL traffic of that PDU Session.

To enable N19-based forwarding in a UPF, the following applies:

-	SMF creates a group-level N4 session to each UPF involved by a 5G VN group for handling traffic to and from  a N19 tunnel or N6 interface associated with the 5G VN group.. Such a N4 session is created per UPF and 5G-LAN group (i.e. a group-level N4 session not corresponding to a PDU Session). In case of multiple N19 tunnels between from one UPF to different UPFs involed in a 5G VN group, the SMF may provide this UPF with N4 rules corresponding to the different N19 tunnels in a single group-level N4 Session.

-	The SMF provides relevant N4 rules (e.g. PDRs, QERs, URSs and FARs) for the group-level N4 session applicable for the N19 tunnel(s) and N6 interface in the UPF. The SMF may provides PDRs for the group-level N4 session with Source Interface set to "5G VN N19" or “N6-LAN” to instruct the UPF to detect packets and associate them with a FAR with  Destination Interface set to "5G VN internal" and a Network Instance set to represent the 5G VN group in order  to instruct the UPF to again perform classification of packets using the corresponding 5G VN group relevant PDRs . In case of multiple Nx tunnels between different PSA UPFs, the SMF may provide N4 rules corresponding to the different Nx tunnels in a single group-level N4 Session.

-	To match the traffic of the UE(s) whose traffic should be forwarded across a N19 tunnel, the PDRs of the group-level N4 Session includes the destination IP/MAC address(es) of these UEs or for Ethernet PDU Sessions includes an Ethernet PDU Session Information that refers to all MAC addresses used as source address within traffic received by the UPF on the corresponding N19 tunnel.

-	The SMF may proactively update N4 rules for group level N4-Session to enable correct routing of packets towards UE whose PSA UPF has been determined (PDU Session establishment) or reallocated.

-	The SMF may also

-	either configure a default PDR in the group-level N4 Session to capture the packets pertaining to 5G VN group communication with an unknown destination address and associated witha URR to trigger reporting of such events. This enables the SMF to reactively configure relevant PDR and FAR to forward the packets to another UPF via N19 or on N6, based on the reports from UPF; the packet that has triggered such event may be discarded before the SMF has updated the relevant PDR and FAR.

-	or configure

-	a PDR capturing any traffic from the local switch with an unknown destination address (i.e. a PDR with source interface set to 5G VN Internal and network instance set to the identity of the 5G VN group and wildcard Destination Address) together with no indication that the packet has been forwarded over N19 ; this PDR is associated with a FAR requiring to replicate this packet over all N19 interfaces related with the group (a FAR associated with multiple instances of the Outer Header Creation attribute).

When forwarding traffic over N19, the UPF adds in the N19 tunnel header an indication that the packet has already been forwarded over N19.  This is controlled by a FAR attribute.

-	a PDR capturing any traffic from the local switch with an unknown destination address (i.e. a PDR with source interface set to 5G VN Internal and network instance set to the identity of the 5G VN group and wildcard Destination Address) together with the indication that the packet has been forwarded over N19 ; this PDR may be associated with a FAR requiring to drop the traffic or with a FAR requiring to forward the traffic to N6

NOTE: 	such PDR can relate to an URR with e.g. a Periodic measurement threshold attribute in order to assess the traffic being dropped or sent over N6

In order to support Ethernet broadcast traffic, the following applies:

The SMF may configure

- Associated with a PDR identifying Ethernet broadcast traffic on the access side of N4 Sessions corresponding to PDU Sessions of 5G VN group members: a FAR requiring to replicate this packet over all N19 interfaces  and access interfaces related with the group except the incoming N4 access session

This uses a FAR associated with multiple instances of the Outer Header Creation attribute. When forwarding traffic over N19, the UPF adds in the N19 tunnel header an indication that the packet has already been forwarded over N19.  This is controlled by a FAR attribute.

-	a PDR capturing any traffic from the local switch with a broadcast Ethernet destination address (i.e. a PDR with source interface set to 5G VN Internal and network instance set to the identity of the 5G VN group and a broadcast Destination Address) together with the indication that the packet has been forwarded over N19; this PDR is associated with a FAR requiring to replicate this packet over all access interfaces (together with the removal of the indication that the packet has been forwarded over N19) related with the group;

The corresponding FAR is associated with multiple instances of the Outer Header Creation attribute.

.

.

Editor's note:	Handling of multicast traffic with local switch and N19-based forwarding is FFS.

Editor's note:	Whether there is any impact the ARP/ND proxying features is FFS.

NEXT CHANGE

##### 5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR |  |
| Rule ID |  | Unique identifier to identify this rule |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied |  |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN" , "5G VN internal", "5G VN N19" | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3) | packet filter set, application ID, Ethernet PDU Session |
| information | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet | Information and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6, TS 23.501. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application ID |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. NOTE X. | DN or the local DN), from "5G VN internal" (i.e. local switch), or from "5G VN N19" (i.e. N19 interface). |
|  | Indication that the packet has been forwarded over N19 | May contain the value “already forwarded over N19”. Applies only to Source interface = "5G VN N19". Usage is defined in clause 5.8.2.13. |  |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14 | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch" and N19 forwarding is used for different 5G VN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE X: Ethernet PDU Session Information may apply in case the source interface is within "N6-LAN", "5G VN N19" or “5G VN internal". It refers to Ethernet addresses within traffic having been received for the same Network instance and for the same N4 session. |  |  |  |

NEXT CHANGE

##### 5.8.2.11.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.2.11.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. |  |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included. |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). |  |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal" or "5G VN N19". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN or the local DN), to 5G LAN internal (i.e. local switch), or to 5G LAN N19 (i.e. N19 interface). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP+QFI, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI). Any extension header stored for this packet shall be added. Multiple occurrence of the “Outer header creation” attribute may be defined (NOTE X). |
| “Indication that the packet has been forwarded over N19” creation | May contain the value “already forwarded over N19”. Applies only to Destination interface = "5G VN N19". |  |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. |  |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | Contains one of the following policies identified by a TSP ID: - an N6-LAN steering policy to steer the subscriber's traffic to the appropriate N6 service functions deployed by the operator, or - a local N6 steering policy to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7. or a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G VN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF, and included in the FAR according to the description in clauses 5.6.7 and 6.1.3.14 of 23.503 [45] for local N6 steering and 6.1.3.14 of 23.503 [45] for N6-LAN steering. The TSP ID action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 in TS 29.244 [65]. NOTE 7: The use of "5G VN Internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. To avoid that the packet matches again the same PDR, it is assumed that the packet has been modified in some way (e.g. tunnel header has been removed, the packets being received from the source interface 5G VN Internal of the same 5G VN group). NOTE X: This may be used e.g. to support 5G VN group communication (see clause 5.8.2.13) |  |  |

END OF CHANGES