3GPP TSG-SA WG2 Meeting #133	S2-1905535

13–17 May 2019, Reno, China	(Revision of S2-190xxxx)

Source:	Huawei, HiSilicon

Title:	Update the input data and output analytics for UE mobility analytics and UE communication analytics

Document for:	Approval

Agenda Item:	6.11

Work Item / Release:	eNA / Rel-16

Abstract of the contribution: This contribution proposes to:

- Add the Type Allocation code (TAC) as the input data of the UE Mobility and Communication analytics.

- Add the UE group ID into the UE Mobility analytics and UE Communication analytics.

- Discussion

Observation 1):

Type Allocation Code (TAC) is the initial eight-digit portion of the 15-digit IMEI or 16-digit IMEISV codes, which identifies a particular model (and often revision) of vendor for wireless network, and is defined in clause 6.2.1, TS 23.003:

In general, devices with the same TAC may have the similar expected behavior, for example, electricity meters made by company A may have the same TAC, and they never move and communicate to the network once a month. Thus, devices with the same TAC may have similar mobility and communication behavior. Thus, TAC can be an input data and output analytics for UE mobility and communication parameters analytics.

From security aspect, devices with the same TAC may also have the similar abnormal behavior. The attacker always hijacks the devices who have the same vulnerability using malicious application, and utilizes them to trigger some attack, e.g. DDoS attack. The devices with the same TAC may have the same vulnerability, e.g. software vulnerability, hardware vulnerability, etc. because they are batch production, once a vulnerability exists, all the devices are suffering the same vulnerability. For example, Mirai compromises thousands of camera devices using the same vulnerability to trigger a DDoS attack to DNS Server. Those devices are specific versions of specific companies. Thus, TAC can be an input data and output analytics for abnormal behavioral related network data analytics too.

Proposal 1):

It is proposed to add the TAC as the input data and output analytics for the mobility analytics, communication analytics and abnormal behavioral related network data analytics.

Observation 2):

As defined in clause 6.7.1, the Target of Analytics Reporting of the Analytics ID "UE Mobility" or "UE Communication" can be any UE or an Internal Group Identifier.

However, the UE group information in the output analytics of the "UE Mobility" or "UE Communication" are missing in somewhere.

Proposal 2):

It is proposed to add the UE group ID into the "UE Mobility" or "UE Communication" related network data analytics.

- Proposal

It is proposed to update the clause 6.7.2, TS 23.288.

**** First Change ****

6.10	UE mobility analytics

6.10.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from 5GC, OAM and/or AFs, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE.

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

6.10.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.10.2-1;

Table 6.10.2-1: UE information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.10.2-2;

Table 6.10.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

6.10.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics, UE mobility predictions:

-	UE mobility statistics is defined in the Table 6.10.3-1.

Table 6.10.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a group of UEs or a specific UE, e.g. internal group ID, TAC, SUPI List or SUPI. |
| UE location (1..max) |  |
| >UE location | TA or cells which the UE stays |
| >Time slot | Time slot that the UE moves to this location |
| >Average Duration | Average time period that the UE stays in this location |
| >Frequency | Number of times that the UE moves to this location measured during the Observation period |

-	UE mobility predictions is defined in the Table 6.10.3-2.

Table 6.10.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a group of UEs or a specific UE, e.g. internal group ID, TAC, SUPI List or SUPI. |
| Predicted UE location (1..max) |  |
| >UE location | TA or cells where the UE may move into |
| >Time slot | Time slot within which the UE may move to this location during the Observation period |
| >Duration | Time period that the UE may stay in this location during the Observation period |
| >Confidence | Confidence of this prediction |

6.10.4	Procedures

6.10.4.1	General

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF or to an AF. When the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

6.10.4.2	NF or trusted AF requesting analytics

Figure 6.10.4.2-1: UE mobility analytics provided to an NF or trusted AF

1.	The NF or trusted AF sends a request to the NWDAF for analytics on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF or AF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The NF or AF provides the UE id or Internal Group ID which is the target for analytics.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

Editor's note:	Details for NWDAF collecting data from other NFs, AFs and OAM are FFS.

Editor's note:	How the NWDAF retrieves the serving AMFs is FFS.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested UE mobility analytics to the NF or trusted AF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.10.3.

5-6. If at step 1, the NF or trusted AF has subscribed to receive notifications for UE mobility analytics, the NWDAF may generate new analytics and provide them to the NF or trusted AF.

6.10.4.3	Untrusted AF requesting analytics

Figure 6.10.4.3 -1: UE mobility analytics provided to an untrusted AF

1.	The untrusted AF sends a request to the NEF for analytics on a specific UE, using either the Nnef_AnalyticsExposure_Fetch or Nnef_AnalyticsExposure_Subscribe service operation. The AF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The AF provides the GPSI or External Group ID as the target for analytics. The NEF maps the GPSI into SUPI, or the External Group ID into Internal Group ID, according to information received from UDM.

2.	If the AF request is authorized, the NEF sends a request to the NWDAF for UE mobility analytics for this UE or group of UEs using procedure as specified in subclause 6.10.4.2.

3.	When receiving the results of the UE mobility analytics from NWDAF, the NEF provides the analytics to the AF.

4-5. The NWDAF may generate new UE mobility analytics for this UE or group of UEs and send them to the NEF. The NEF then forwards these analytics to the AF via Nnef_AnalyticsExposure_Notify service operation. The details for UE mobility analytics provided via NEF are defined in clause 6.10.3.

**** Next Change ****

6.11	UE Communication Analytics

6.11.1	General

In order to support some optimized operations, e.g. customized mobility management or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF and SMF that corresponds to this application ID.

6.11.2	Input Data

The NWDAF supporting data analytic on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.11.2-1.

Table 6.11.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be GPSI or external UE ID |
| Correlation information | AF | Identifying the application generating the traffic |
| Application ID | AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 |
| UE communication (1..max) | AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication expected by the application |
| >DL data rate |  | DL data rate of this communication expected by the application |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

The correlation information may be used to correlate the communication information collected from an AF with the communication information collected from SMF(s). Other correlations can be made within the NWDAF using UE communication time pattern.

Editor's Note:	 the exact nature of correlation information (e.g. IP address 5-tuple) is FFS.

6.11.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF includes the UE communication statistics or predictions as defined in the Table 6.11.3-1.

Table 6.11.3-1: UE Communication Analytics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a group of UEs or a specific UE, e.g. internal group ID, TAC, SUPI List or SUPI. |
| UE communications (1..max) |  |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (may be used together with 1). Example: every hour |
| > Scheduled communication time | Time zone and Day of the week when the UE may be available for communication. Example: Time: 13:00-20:00, Day: Monday |
| > Communication duration time | Duration interval time of periodic communication or the scheduled communication (may be used together with 1). Example: 5 minutes |
| > Traffic characterization | Estimated UL/DL traffic volume |
| > Traffic volume | Volume UL/DL |
| > Confidence | Confidence of the prediction if this is a communication pattern prediction |

### 6.11.4	Procedures

The NWDAF can provide UE communication related analytics, in the form of statistics or predictions or both, to a 5GC NF.

NF

NWDAF

1-Nnwdaf_AnalyticsInfo_Request

/ Nnwdaf_AnalyticsInfo_Subscribe

AF

SMF

2a-Naf_EventExposure_Subscribe

2b--Naf_EventExposure_Notify

2c-Nsmf_EventExposure_Subscribe

2d-Nsm_EventExposure_Notify

3- NWDAF derives new analytics

4-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

7-Nnwdaf_AnalyticsInfo_Response

/ Nnwdaf_AnalyticsInfo_Notify

6- NWDAF derives new analytics

5b--Naf_EventExposure_Notify

5d-Nsm_EventExposure_Notify

Figure 6.11.4-1: Procedure for UE communication analytics

1.	5GC NF to NWDAF: Nnwdaf_ AnalyticsSubscription_Subscribe (Analytics ID=UE communication analytics, Analytics Filter=SUPI).

5GC NF sends a request to the NWDAF for analytics on a specific UE(s), using either Nnwdaf_AnalyticsInfo or Nnwdaf_ AnalyticsSubscription_Subscribe service. The analytics type indicated by "Analytics ID" is set to "UE communication". The analytics target indicated by Analytics Filter is set to SUPI or a list of SUPI.

2a-b.	NWDAF to AF (Optional): Naf_ EventExposure_Subscribe (Event ID, external UE ID).

In order to provide the requested analytics, the NWDAF may subscribe per application communication information from AFs for the UE. The Event ID indicates communication report for the UE which is requested by the 5GC NF in the step 1. The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI.

This step is skipped if the NWDAF already has the requested analytics available or has subscribed to the AF.

2c-d- NWDAF to SMF: Nsmf_ EventExposure_Subscribe (Event ID, SUPI).

In order to provide the requested analytics, the NWDAF subscribes to UE session information from SMFs.

3.	The NWDAF derives requested analytics, in the form of UE communication statistics or predictions or both.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription_Notify.

The NWDAF provides requested UE communication analytics to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_ AnalyticsSubscription _Notify, depending on the service used in step 1.

5-6. If the NF subscribed UE communication analytics at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

**** Next Change ****

6.8	Abnormal behaviour related network data analytics

6.8.1	General

This clause defines how NWDAF identifies a group of UEs or a specific UE being misused or hijacked based on the expected UE behavioural parameters.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription request from a 5GC NF (e.g. AMF, SMF, or PCF), OAM or an AF.

The consumer of this analytics shall indicate in the request:

-	AnalyticsId set to "Abnormal behaviour”

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier.

-	Optionally an Application ID, DNN or S-NSSAI

-	An Observation period indicates the time when the statistics or predictions are requested.

-	Analytics Filter Information: optional expected behaviour, list of risk IDs to monitor with associated thresholds

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

When the NWDAF detects those UEs that deviate from the expected UE behavioural information, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.8.3.

6.8.2	Input Data

The expected UE behavioural parameters that provided to the NWDAF are defined in Table 6.7.3-1.

The UE behavioural information collected from 5GC NFs is as specified in clause 6.7.2.

On request of the service consumer, the NWDAF shall collect and analyse the behaviour analytics from the 5GC NFs (SMF, AMF) or AF.

6.8.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.8.3-1. The NWDAF shall notify the consumer of the exception(s) with appropriate exception ID if the exceptions level trespasses above or below the threshold.

Table 6.8.3-1: Abnormal behaviour Analytics

| Information | Description |
| --- | --- |
| UE ID list | Could be SUPI, Internal-Group-Identifier, external UE ID, TAC which indicates UE being subject to the risk(s) identified by the Exception ID(s) |
| Exceptions (1..max) |  |
| >Exception ID | The risk detected by NWDAF |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| >Additional measurement | Specific information for each risk |

If PCF subscribes notifications on “Abnormal behaviour”, the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.8.3-2 gives examples of AM/SM policies and corresponding actions for solving each risk.

Table 6.8.3-2: Examples of policies and actions for risk solving

| Exception ID and description | AM/SM policy | Actions of NFs |
| --- | --- | --- |
| Unexpected UE location | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong stationary UE | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust UE registration area. |
| Too frequent Service Access/Abnormal traffic volume | NWDAF notifies AF (Service Provider) |  |

6.8.4	Procedure

Figure 6.8.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a. A consumer NF/OAM subscribes to NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID set to “Abnormal behaviour”, Analytics Filter(s) = Internal-Group-Identifier or SUPI).

A consumer NF/OAM may subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b. AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Analytics Filter(s) = External-group identifier or External UE ID).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsSubscription_Subscribe (Analytics ID, Analytics Filter(s) = External-group-identifier or External UE ID).

An AF may also subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE, where the subscription message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier.

2. 	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related AMF if it has not subscribed such data.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

Editor´s note:		Whether the AMF needs to check whether the UE's behaviour matches its expected UE behavioural information is FFS.

3. 	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

Editor´s note:		Whether the SMF needs to check whether the UE's behaviour matches its expected UE behavioural information is FFS.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator’s policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription)/OAM (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, Internal-Group-Identifier or SUPI, Exception level)).

If the NWDAF determines to send a notification to the subscribed 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify services. Based on the notification, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.8.3-1.

5b.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, External UE ID)).

If the NWDAF determines to send a notification to the subscribed AF, the NWDAF needs to include external UE ID of the identified UE into the notification message.

NOTE:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

*************** End of changes ***************